Adjusted Trading Arrangements under SEM Regional Integration Workstream

Options by SSE Renewables

Under the broad objective of seeking greater alignment of SEM with regional European markets, particularly BETTA, and given the imperative to ensure that the upcoming East-West Interconnector is compliant with the EU Congestion Management Guidelines (CMG), the Intra-Day Trading modification proposed by the Regulatory Authorities seeks an adjustment to the current trading arrangements in SEM.

Considering the progress of this modification proposal through the two working group meetings deliberating on it, it is obvious that the definite objectives of the proposal have not been very clear. The proposal, and the use of the term Intra-Day Trading, for which various conflicting views have been expressed, certainly appears to have conflated two different problem domains into one. Is the proposal about providing more trading opportunities than currently exist in SEM, hence moving to greater alignment with BETTA? Or is this simply about reallocating unused interconnector capacity, to fulfil CMG? If the latter is the case, then perhaps the proposed Adjusted Trading Arrangements should be termed Unused Interconnector Capacity Reallocation.

If Unused Interconnector Capacity Reallocation, we argue that this will require a new SEM mechanism, in a similar way as the capital requirements of generation capacity are addressed separate from the delivered energy using the capacity payment mechanism (CPM).

Thus we see that there are two different issues to be addressed under the modification proposal:

- Unused Interconnector Capacity Reallocation – Addressing the CMG; and

- Providing More Trading Opportunities – Moving towards alignment with BETTA.

These two issues are not mutually exclusive. However it is imperative that they are treated as separate processes and advanced discretely, even if they are implemented simultaneously.

# Unused Interconnector Capacity Reallocation – Addressing the CMG

To address the issue arising under this subject, we propose a new auction mechanism to be run by the TSOs post-release of the ex-ante market schedule, for the sole purpose of reallocating and using the balance of available interconnector capacity not committed within the ex-ante market schedule. Following release of the ex-ante market schedule day ahead, the TSOs will publish the balance of interconnector capacity not committed for the upcoming SEM Trading Day and call for offers/bids. Interested Participants can submit offers/bids to flow electricity across the interconnector making use of such capacity.

Nature of the auction mechanism

The mechanism will entail implicit auctions for efficiency reasons. The capacity charge element of bids will be predetermined as the weighted average value of previously allocated capacity in each Trading Period. This capacity charge will be returned to long-term interconnector capacity holders, implying a UIOSI feature.

Timing of the auctions

The auction will run a rolling basis across the relevant Trading Day, with gate closure for each Trading Period occurring at least 3 hours, and schedules produced at least 2 hours prior to the start of that Trading Period to facilitate the requisite nominations into BETTA.

Settlement basis

This mechanism will involve a pay-as-bid basis for settlements.

Implications for SEM

Flows across the interconnector resulting from this mechanism will have no effect on MSQs or SMP. Where plants (presumably price-effecting) have been constrained as a result of new flows on the interconnector, such plants would receive constraint payments.

There would be implications for the imperfections fund in SEM. However, if designed properly, it is possible for the impact to be a positive one, i.e. lower the overall requirements for imperfections.

# Providing More Trading Opportunities – Moving towards alignment with BETTA

Given the fundamental difference in the designs of SEM and BETTA, we believe that ‘true’ alignment between the two markets would require well-thought out structural changes, in either or both markets. In the short- to medium-term however, we are of the opinion that some levels of alignment can be achieved in a progressive sense, within the constraints of the two market designs. In addressing this issue, we propose a 2-step approach which takes this into cognisance, and which also recognises that the very tight time-scales imposed on this working group to arrive at a/solution(s).

### Step 1 – Split the SEM Trading Day into 2 Trading Sessions; Introduce 1 Within-Day Gate Closure (Timeline: within the current time-scales of the Intra-Day Trading modification)

Under this proposal, the current Trading Day will be divided into two Trading Sessions (Session 1: 0600 – 1700; Session 2: 1700 – 0600). Trading Session 1 (TS1) would still retain the Gate Closure at 1000 D-1 (or at a later time, if feasible), while Trading Session 2 (TS2) would have a new within-day Gate Closure introduced, possibly at 1000 D (or again at a later time, if feasible).

The two Sessions will run as current SEM processes do, but provide the benefit of having SMPs over the peak periods account for changes in market conditions.

This we believe in implementable in the time-scales proposed by the modification proposal. In addition we do not believe that this introduces any changes in the current workings times of any market participant, and hence is less impactful on resources. Furthermore, it provides the market an opportunity to gain experiential and operational knowledge of the value (or otherwise) of greater alignment with BETTA.

### Step 2 – Split the SEM Trading Day into 6 Trading Sessions; Move the current Gate Closure closer to the start of the Trading Day; Introduce 5 Within-Day Gate Closure (Timeline: Contingent on other work being progressed under the SEM Regional Integration workstream)

Under this proposal, SEM would move to even closer alignment with BETTA timelines, with 6 Trading Sessions of 4-hour length each aligned with EFA blocks.

This should be a slightly longer term objective of providing more trading opportunities within SEM, given the tight time-scales provided in the modification proposal; the greater impact this change would have on SEM design and on market participants’ (including MO, TSOs and MDPs) resources; and the unknown factors relating to other work being considered under the broader SEM Regional Integration workstream. Furthermore it would be necessary to determine and demonstrate that true benefits would accrue from such a structure.