SEMO Proposal on Intraday Trading

GC = Gate Closure

GO = Gate Opening

EAn = Ex-Ante Market Schedule

WDn = Within Day Market Schedule

- SEMO proposal on Intra-Day Trading

This proposal builds on modification Mod_18_10. It includes two extra Ex-Ante runs as did the modification, listed in the diagram above as EA2 and EA3. The proposal also introduces a  within day run of the UUC, designated WD1.In the diagram above, the respective Gate Openings, while listed at the same time as the publication of the previous schedules, are assumed to be as early as possible after the publication and may actually be later to account for modification of Interconnector User Nominations into MIUN.

As per the proposed modification, the EA1 schedule will set IUN/MIUN for Interconnector Users with firm capacity holdings secured in the separate auction. These allocations can be protected in later runs by Participant bidding. Should Participants wish to unwind this position, they can do this by bidding themselves back out of merit in later auctions.

The suggestion is for the timing of EA3 to be at 17:00, to ensure that schedules for the Interconnector can be closed out 12 hours before the start of the Trading Day at 6AM. This will ensure that the interconnector schedule can be accommodated with respect to generators with a start-up notification time of 12 hours, in cold state. The results of this schedule, in terms of interconnector MIUNs, will then be fixed for the first 12 hours of the Trading Day.

Following this, after the start of the Trading Day, a Within Day run of the MSP software (WD1) will be completed. This run will have a thirty hour optimisation horizon, starting from 18:00 running to midnight the following day (D+1). This run follows on from the previous runs, with MIUN allocations from the EA2 and EA3 runs held protected unless the Participant elects to bid out of their position.

While introducing some limitations around the Within Day schedule, this still adheres to the basic principal of the SEM being an unconstrained pool market. The commitment decisions held as firm in the Within Day schedule are based on Ex-Ante runs which are totally unconstrained. This version does not attempt to address constraint issues or security of supply issues around the firm allocation of energy on the interconnector. It is expected that if it is required to alter the flow of an interconnector for security of supply issues, this should be managed through some form outside of market arrangements to be developed in separate workstreams.

For further consideration –

- Should Generator start up notification times be included in this proposal? We can consider if Generators who are subject to start-up notification times that are longer than the time between the publication of WD1 and their commitment from the previous EA runs are carried over as must-run. These generator positions in terms of commitment only will be respected.

- Should Generators start-up notifications be fully respected in WD1? This can be considered to ensure that the schedule does not set interconnector flow while depending on starting plant whose notification time is longer than the time between the publication of WD1 and the Trading Period in which they are to be committed.

- If we consider the start up notifications both for starts from the EA3 and looking forward in the next twelve hour period, the solver must determine from committed MIUN allocations and the Min Stable Generation of any unit whose commitment is firm, what remaining level of the System Load needs to be scheduled from the remaining generators who are available based on notification times and unallocated interconnector capacity. From the results of this run, the interconnector schedule for the next 12 hours of the Trading Day will be fixed. Taking account of already committed plant, no generator will be started without due notification and generators already subject to notifications will be started.

- What commercial offer data is to be used for the last 18 hours of WD1? Will this require the MSP solver to use two sets of data for each generator?

- Consideration needs to be given to the publication of price from the WD1 run. SMP calculation is done across the Trading Day and running the MSP software in this manner will result in the system calculating Uplift with respect to generator’s positions across the twenty four hour period starting from 18:00. This SMP will not be valid. A suggestion would be to use the Shadow Price and not calculate Uplift in this run. As a separate workstream is looking at firm day-ahead pricing, this may mean that the Within Day price is of no consequence?