| MODIFICATION PROPOSAL FORM |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Proposal Submitted by: | Date Proposal received by Secretariat: |  | Type of Proposal |  |  | Number: |
| SEMO | 11 November 2010 |  | Standard |  |  | Mod_42_10 |
| Contact Details for Modification Proposal Originator |  |  |  |  |  |  |
| Name: Katia Compagnoni |  | Telephone number: (01) 23 70257 |  | e-mail address: katia.compagnoni@sem-o.com |  |  |
| Modification Proposal Title: Changes to the Single Ramp Up Rate and the Single Ramp Down Rate Calculation |  |  |  |  |  |  |
| Trading and Settlement Code and/or Agreed Procedure change? |  |  |  |  | Trading and Settlement Code |  |
| Section(s) affected by Modification Proposal: |  |  |  |  | 5.120; H.2; I.12; N33;N35; Glossary |  |
| Version Number of the Code/Agreed Procedure used in Modification drafting: |  |  |  |  | V 7.0 (Note: This modification takes into account the text changes in the T&SC version which includes MOD_35_10 which is RA Approved but not yet implemented). |  |
| Modification Proposal Description (Clearly show proposed code change using tracked changes & include any necessary explanatory information) |  |  |  |  |  |  |
| - 5.120 The Market Operator shall procure that within the Technical Offer Data or Generator Unit Technical Characteristics for each Pumped Storage Unit, any submitted value for Minimum Generation shall not be used within the MSP Software. - APPENDIX H: Participant and Unit Registration and Deregistration Participation Notice - H.2 In completing a Participation Notice as set out in paragraph 2.33, a Party (or an Applicant as applicable) shall include the additional Registration Data pursuant to paragraph 2.33.17 as set out in Table H.1 below. Certain Registration Data items shall be classified as Validation Registration Data as outlined in Table H.1. Table H.2 – Data, required from Party (or Applicant as applicable) registering the Unit Name; Term; Relevant Units; Validation Registration Data. Maximum Generation; All Generator Units except Interconnector Units, Interconnector Error Units, Interconnector Residual Capacity Units, Demand Side Units and Netting Generator Units. The Maximum Generation shall be submitted equal to the Registered Capacity of the Generator Unit.; Yes. Lower Registered Capacity; All Generator Units except Interconnector Units, Interconnector Error Units, Interconnector Residual Capacity Units, Demand Side Units and Netting Generator Units.; Yes. Pumped Storage Flag; All Generator Units except Interconnector Units, Interconnector Error Units, Interconnector Residual Capacity Units, Demand Side Units and Netting Generator Units.; Yes. Technical Offer Data - 1.12 Each Participant shall submit Technical Offer Data to the Market Operator in respect of each of its Generator Units in accordance with the following provisions, subject to the exceptions outlined in paragraphs I.13 to I.18 inclusive: - Technical Offer Data shall be submitted before Gate Closure for the Trading Day to which the data relates, as set out in paragraph 4.4 and no more that 28 days before Gate Closure; - 2. Data shall be submitted to reflect the real capabilities of the relevant Generator Unit net of Unit Load as set out in paragraph 4.26; - 3. Data shall be submitted in respect of a Generator Unit such that it is consistent with data submitted for that Unit under the applicable Grid Code, scaled, where appropriate, by the appropriate Distribution Loss Adjustment Factor as set out in paragraph 4.27; - 4.Certain Technical Offer Data items shall be classified as Validation Technical Offer Data as set out in this paragraph I.12 and in paragraphs I.17 and I.18; - 5. A Forecast Availability Profile shall be submitted in accordance with paragraph 4.28; - 6. A Forecast Minimum Output Profile shall be submitted in accordance with paragraph 4.29; - 7. A Forecast Minimum Stable Generation Profile shall be submitted in accordance with paragraph 4.30; - 8. A Short-Term Maximisation Capability, a Short-Term Maximisation Time, a Minimum Generation shall be submitted and shall be classified as Validation Technical Offer Data; and - Operation of the MSP SOFTware Derivation of Single Ramp Up Rate - Each Price Maker Generator Unit that is not Under Test shall be represented in the MSP Software as having a Single Ramp Up Rate for the Optimisation Time Horizon that limits the rate at which its average MW Output can be scheduled to increase from one Trading Period to the next, to a value determined by the Market Operator as follows: - 1. For each Price Maker Generator Unit that is not Under Test and is not a Demand Side Unit, Pumped Storage Unit or Interconnector Unit, the Single Ramp Up Rate, expressed in MW per Trading Period shall be calculated as follows: if RampUpTime+DwellTime≠0SingleRampUpRate=OutputRangeRampUpTime+DwellTime×60×TPDelseSingleRampUpRate will be set to a non-limiting value Where - Output Range, expressed in MW, is the maximum value of Availability (derived in accordance with paragraphs N.37 to N.39 below) that occurs in any Trading Period in the Optimisation Time Horizon less the value of Minimum Generation which is the Accepted value for a given Trading Day associated with the Optimisation Time Horizon; - Ramp Up Time, expressed in minutes, is the minimum time it would take that Generator Unit to increase its instantaneous Output from its Minimum Generation to its Availability calculated using Ramp Up Rate 1 to Ramp Up Rate 5 (to the extent that such values have been provided) and Ramp Up Break Point 1 to Ramp Up Break Point 4 (to the extent that such values have been provided). In determining Ramp Up Time, Ramp Up Rate i applies between a MW Output of Ramp Up Break Point i-1 and a MW Output of Ramp Up Break Point i, where if there is no defined Ramp Up Break Point i-1 then Ramp Up Rate i applies for all MW Output levels below Ramp Up Break Point i while if there is no defined Ramp Up Break Point i then Ramp Up Rate i applies for all MW Output levels above Ramp Up Break Point i-1, with the exception that if that the largest Ramp Up Break Point j value is less than the Maximum Availability of the unit over the day then Ramp Up Rate j applies between Ramp Up Break Point j-1 and the Maximum Availability of the Unit. If there are no Ramp Up Break Point values provided then Ramp Up Rate 1 applies for all levels of MW Output. For the avoidance of doubt, if Ramp Up Break Point j is the last valid Ramp Up Break Point provided (for j increasing from j=1), then no Ramp Up Rate n>j+1 or Ramp Up Break Point n>j is to be considered in the calculation of Ramp Up Time. - Dwell Time, expressed in minutes, is the sum of all Dwell Times corresponding to Dwell Time Triggers, between and including the Unit’s Minimum Generation and its Availability for that Optimisation Time Horizon; and - TPD is the Trading Period Duration. - For Generator Units that are Pumped Storage Units, the value of Single Ramp Up Rate, expressed in MW per Trading Period, shall be calculated as follows if RampUpTime+DwellTime≠0 thenSingleRampUpRate=AvailabilityRampUpTime+DwellTime×60×TPDelseSingleRampUpRate will be set to a non-limiting value Where - a. Availability of the Generator Unit is derived in accordance with paragraphs N.37 to N.39 below; - b. Ramp Up Time, expressed in minutes, is the minimum time it would take that Generator Unit to increase its instantaneous Output from 0 MW to its Availability calculated using Ramp Up Rate 1 to Ramp Up Rate 5 (to the extent that such values have been provided) and Ramp Up Break Point 1 to Ramp Up Break Point 4 (to the extent that such values have been provided) and in accordance with the methodology in paragraph APPENDIX N: .1.b. - c Dwell Time, expressed in minutes, is the sum of all Dwell Times corresponding to Dwell Time Triggers, between and including an Output of 0 MW and the Generator Unit’s Availability; and - d. TPD is the Trading Period Duration. - For Demand Side Units the Single Ramp Up Rate, expressed in MW per Trading Period, shall be the Accepted value of Maximum Ramp Up Rate multiplied by 60 x TPD, except that if a value for Maximum Ramp Up Rate that is greater than zero is not Accepted then the value for the Single Ramp Up Rate that is used within the MSP Software will be set by the Market Operator to a value which will impose no restrictions on the Market Schedule Quantity of the Demand Side Unit. - For Interconnector Units the Single Ramp Up Rate shall be set by the Market Operator to a value which will impose no restrictions on the Market Schedule Quantity of the Interconnector Unit. - Where the values of Availability for the relevant MSP Software Run Type are derived in accordance with paragraphs N.37 to N.39. - For any Generator Unit, if the calculations of Single Ramp Up Rate set out above result in a value of zero then the Single Ramp Up Rate shall be set to a value which will impose no restrictions on the Market Schedule Quantity of the Generator Unit. Derivation of Single Ramp Down Rate - Each Price Maker Generator Unit that is not Under Test shall be represented in the MSP Software as having a Single Ramp Down Rate for the Optimisation Time Horizon that limits the rate at which Generator Unit average MW Output can decrease from one Trading Period to the next with a value determined by the Market Operator as follows: - For each Price Maker Generator Unit that is not Under Test and is not a Demand Side Unit, Pumped Storage Unit, or Interconnector Unit, the Single Ramp Down Rate value for each Trading Period h in the Optimisation Time Horizon, expressed in MW per Trading Period, equals Where - a. Output Range, expressed in MW, is the maximum value of Availability (derived in accordance with paragraphs N.37 to N.39 below) that occurs in any Trading Period over the Optimisation Time Horizon less the value of Minimum Generation which is the Accepted value for a given Trading Day associated with the Optimisation Time Horizon. - b. Ramp Down Time, expressed in minutes, is the minimum time it would take that Generator Unit to decrease its instantaneous Output from its Availability to its Minimum Generation calculated using Ramp Down Rate 1 to Ramp Down Rate 5 (to the extent that such values have been provided) and Ramp Down Breakpoint 1 to Ramp Down Break Point 4 (to the extent that such values have been provided). In determining Ramp Down Time, Ramp Down Rate i applies between a MW Output of Ramp Down Break Point i-1 and a MW Output of Ramp Down Break Point i, where if there is no defined Ramp Down Break Point i-1 then Ramp Down Rate i applies for all MW Output levels below Ramp Down Break Point i while if there is no defined Ramp Down Break Point i then Ramp Down Rate i applies for all MW Output levels above Ramp Down Break Point i-1, with the exception that if that the largest Ramp Down Break Point j value is less than the Maximum Availability of the unit over the day then Ramp Up Rate j applies between Ramp Up Break Point j and the Maximum Availability of the Unit. If there are no Ramp Down Break Point values provided then Ramp Down Rate 1 applies for all levels of MW Output. For the avoidance of doubt, if Ramp Down Break Point j is the last valid Ramp Down Break Point provided (for j increasing from j=1), then no Ramp Down Rate n>j+1 or Ramp Down Break Point n>j is to be considered in the calculation of Ramp Down Time. - c. Dwell Time, expressed in minutes, is the sum of all Dwell Times corresponding to Dwell Time Triggers, between and including the Unit’s Minimum Generation and its Availability; and - d. TPD is the Trading Period Duration. - For Generator Units that are Pumped Storage Units, the Single Ramp Down Rate, expressed in MW per Trading Period, shall be calculated as follows if RampDownTime+DwellTime≠0 thenSingleRampDownRate=AvailabilityRampDownTime+DwellTime×60×TPDelseSingleRampDownRate will be set to a non-limiting value Where - a. Availability of the Generator Unit is derived in accordance with paragraphs N.37 to N.39 below; - b. Ramp Down Time, expressed in minutes, is the minimum time it would take that Generator Unit to decrease its instantaneous Output from its Availability to 0 MW calculated using Ramp Down Rate 1 to Ramp Down Rate 5 (to the extent that such values have been provided) and Ramp Down Break Point 1 to Ramp Down Break Point 4 (to the extent that such values have been provided) and in accordance with the methodology in paragraph N.35.1.b. - c. Dwell Time, expressed in minutes, is the sum of all Dwell Times corresponding to Dwell Time Triggers, between and including an Output of 0 MW and the Generator Unit’s Availability; and - d. TPD is the Trading Period Duration. - For Demand Side Units, the Single Ramp Down Rate, expressed in MW per Trading Period, shall be the Accepted value of Maximum Ramp Down Rate multiplied by 60 x TPD, except that if a value for Maximum Ramp Down Rate that is greater than zero is not Accepted then the value for the Single Ramp Down Rate that is used within the MSP Software will be set by the Market Operator to a value which will impose no restrictions on the Market Schedule Quantity of the Demand Side Unit. - For Interconnector Units the Single Ramp Down Rate shall be set to a value which will impose no restrictions on the Market Schedule Quantity of the Interconnector Unit. Glossary Definitions Lower Registered Output; means the minimum level of Output at which a Generator Unit may operate submitted for the Generator Unit in accordance with Appendix H: "Participant and Unit Registration and Deregistration". Lower Registered Output is zero except for Pumped Storage Units, for which the Lower Registered Output shall be equal to the pumping capability.. Minimum Generation; means the minimum MW Output which a Generator Unit can generate continuously.. Minimum Stable Generation; means the time weighted average Outturn Minimum Stable Generation for each Trading Period within the Trading Day for a Generator Unit, calculated as described in Section 4.49.. |
| Modification Proposal Justification (Clearly state the reason for the Modification & how it furthers the Code Objectives) |  |  |  |  |  |  |
| At present, the method used to calculate the Single Ramp Up and Down Rates in SEM results in values that fluctuate considerably from one Trading Day to the next and are not always a valid representation of the actual capabilities of the unit. Clearly this is not the intention of the T&SC, as it aims to set down rules that accurately model Generator technical characteristics. The negative impact of the current formulation of the Single Ramp Rate has been discussed and demonstrated in two reports recently published by SEMO : ”Pricing Events in the SEM: September 20th, 2010” and “Increased Use of MIP as the Market Solver in September 2010”. Both reports analyse the impacts of the Single Ramp Rate on the scheduling of units and Market Prices. The proposed modification aims to more accurately model reality and thus achieve a less volatile application of ramp limitations in the Market Schedule. With the proposed change, Ramp Rates will not be impacted by an artificially low Minimum Stable Generation when coming back from (or going into) an outage. In making the desired change in the T&SC, it was also realised that the terms "Minimum Stable Generation" and "Minimum Generation" are sometimes used inappropriately in the T&SC. Therefore this modification also seeks to rectify this anomaly This Modification furthers the code objective N.2: “to facilitate the efficient, economic and coordinated operation, administration and development of the Single Electricity Market in a financially secure manner”; |  |  |  |  |  |  |
| Implication of not implementing the Modification (Clearly state the possible outcomes should the Modification not be made , or how the Code Objectives would not be met) |  |  |  |  |  |  |
| If this Modification is not implemented the Single Ramp Up or Down Rates used in the Market will continue to fluctuate considerably from one Trading Day to the other and will not always be a valid representation of the actual capabilities of the unit. The intention of the Code is to model in the Market, all the technical characteristics that are used in Dispatch and the current formulation of the Code has been shown not to achieve this. |  |  |  |  |  |  |
| Please return this form to Secretariat by e-mail to modifications@sem-o.com |  |  |  |  |  |  |

- Notes on completing Modification Proposal Form:

- If a person submits a Modification Proposal on behalf of another person, that person who proposes the material of the change should be identified on the Modification Proposal Form as the Modification Proposal Originator.

- Any person raising a Modification Proposal shall ensure that their proposal is clear and substantiated with the appropriate detail including the way in which it furthers the Code Objectives to enable it to be fully considered by the Modifications Committee.

- Each Modification Proposal will include a draft text of the proposed Modification to the Code.

- For the purposes of this Modification Proposal Form, the following terms shall have the following meanings:

Code:	means the Trading and Settlement Code for the Single Electricity Market

Modification Proposal:	means the proposal to modify the Code as set out in the attached form

Derivative Work:	means any text or work which incorporates or contains all or part of the Modification Proposal or any adaptation, abridgement, expansion or other modification of the Modification Proposal

The terms “Market Operator”, “Modifications Committee” and “Regulatory Authorities” shall have the meanings assigned to those terms in the Code.

In consideration for the right to submit, and have the Modification Proposal assessed in accordance with the terms of Section 2 of the Code (and Agreed Procedure 12), which I have read and understand, I agree as follows:

- 1.	I hereby grant a worldwide, perpetual, royalty-free, non-exclusive licence:

  - to the Market Operator and the Regulatory Authorities to publish and/or distribute the Modification Proposal for free and unrestricted access;

  - to the Regulatory Authorities, the Modifications Committee and each member of the Modifications Committee to amend, adapt, combine, abridge, expand or otherwise modify the Modification Proposal at their sole discretion for the purpose of developing the Modification Proposal in accordance with the Code;

  - to the Market Operator and the Regulatory Authorities to incorporate the Modification Proposal into the Code;

1.4	to all Parties to the Code and the Regulatory Authorities to use, reproduce and distribute the Modification Proposal, whether as part of the Code or otherwise, for any purpose arising out of or in connection with the Code.

2.	The licences set out in clause 1 shall equally apply to any Derivative Works.

3.	I hereby waive in favour of the Parties to the Code and the Regulatory Authorities any and all moral rights I may have arising out of or in connection with the Modification Proposal or any Derivative Works.

4.	I hereby warrant that, except where expressly indicated otherwise, I am the owner of the copyright and any other intellectual property and proprietary rights in the Modification Proposal and, where not the owner, I have the requisite permissions to grant the rights set out in this form.

5.	I hereby acknowledge that the Modification Proposal may be rejected by the Modifications Committee and/or the Regulatory Authorities and that there is no guarantee that my Modification Proposal will be incorporated into the Code.