The Single Electricity Market (SEM)

Agreed Procedure 2: Interconnector User Capacity Right Calculation and Dispatch Notification

Version 4.01

20 November 2007<date tba>

SEM Agreed Procedure

| Title | Agreed Procedure 2: Interconnector User Capacity Right Calculation and Dispatch Notification |
| --- | --- |
| Version | 4.01 |
| Date | 20th November 2007<date tba> |

Table of Contents

1.	Introduction	4

1.1	Background and Purpose	4

1.2	Scope Of Agreed Procedure	4

1.3	Definitions	4

1.4	Compliance With Agreed Procedure	4

2.	Descriptive Overview	5

2.1	Registration of Interconnector Entities	5

2.2	Available Transfer Capacity	5

2.3	Interconnector Unit Nominations	6

2.4	SO Interconnector Trades	6

2.5	Post Gate Closure Reductions in ATC	7

3.	Procedure Definition	8

3.1	Interconnector Registration Data Maintenance	8

3.2	Initial Available Transfer Capacity Notification	8

3.3	Updates to Available Transfer Capacity	8

3.4	Modified Interconnector Unit Nominations	9

4.	Swimlane Diagram	10

4.1	Interconnector Registration Data Maintenance	10

4.2	Swimlane - Initial Available Transfer Capacity Notification	11

4.3	Swimlane – Updates to Available Transfer Capacity	13

4.4	Modified Interconnector Unit Nominations	16

5.	Procedural Steps	20

5.1	Interconnector Registration Data Maintenance	20

5.2	Initial Available transfer Capacity Submission	20

5.3	Updates to Available Transfer Capacity	21

5.4	Modified Interconnector Unit Nominations	22

APPENDIX 1: Definitions and Abbreviations	25

Definitions	25

Abbreviations	28

APPENDIX 2: Calculation of Modified Interconnector Unit Nominations	29

Introduction	29

Definitions	29

Rules for the calculation of the Modified Interconnector Unit Nominations	29

DOCUMENT HISTORY

| Version | Date | Author | Comment |
| --- | --- | --- | --- |
| 1.0 | 18/10/2006 | SEM Implementation Team | Incorporation of more comments |
| 2.3a | 10/04/2007 | Regulatory Authorities | Updated with RA397 and Code 1.3 |
| 2.7 | 25/04/2007 | Regulatory Authorities | Internal review |
| 3.2 | 22/06/2007 | Regulatory Authorities | Approved for Go-Active by Regulatory Authorities and TSO/SEM Programme |
| 4.0 | 20/11/2007 | Modification Panel Secretariat | Incorporating RA-Approved Mod_45_07 |

RELATED DOCUMENTS

| Document Title | Version | Date | By |
| --- | --- | --- | --- |
| Trading and Settlement Code | V4.30 | 20/11/2007 21 July 2008 | Regulatory Authorities |
| BETTA Operational Procedures | Version 3.0 | November 05 | SONI |
| Moyle Interconnector Procedures | Version 1.0 | 7th March 2005 | SONI |
| Agreed Procedure 1 "Participant and Unit Registration and Deregistration" |  |  |  |
| Agreed Procedure 4 “Transaction Submission and Validation” |  |  |  |
| Agreed Procedure 6 “Publications” |  |  |  |
| Agreed Procedure 11 "Market System Operation, Testing, Upgrading and Support" |  |  |  |

- Introduction

  - Background and Purpose

This Agreed Procedure describes the specific procedures for the treatment of Interconnectors which connect the Transmission Systems of Ireland or Northern Ireland with systems outside Ireland and Northern Ireland and which interact with the SEM (the “Single Electricity Market”).  This Agreed Procedure sets out the processes for the operation of Interconnectors as required by the Trading and Settlement Code (the “Code”) and with which Parties to the Code must comply.

  - Scope Of Agreed Procedure

This Agreed Procedure is a definition of procedural steps to be followed by the Market Operator, Interconnector Owner, Interconnector Administrator(s), System Operator(s) and Participants. It forms an annexe to, and is governed by, the Code. This document is a statement of process and procedure. Parties’ rights and obligations are set out in the Code.

  - Definitions

Save as expressly defined, words and expressions defined in the Code shall have the same meanings when used in this Agreed Procedure.  Abbreviations and definitions that are specific to this Agreed Procedure are set out within Appendix 1.

References to particular sections relate internally to this Agreed Procedure unless specifically noted.

  - Compliance With Agreed Procedure

Compliance with this Agreed Procedure is required under the terms as set out the Code.

- Descriptive Overview

Interconnectors and their related Units have special treatment under the SEM that differs from that of other Generator Units.  The details are set out in the Code and this Section 2 of this Agreed Procedure provides a non-legally binding overview of those provisions, for information purposes only, as context for the remaining provisions of this Agreed Procedure.

The Code provides that an Interconnector Owner (the entity that owns or controls an Interconnector) will register the Interconnector using Type 1 Channel, and in doing so shall appoint an Interconnector Administrator to perform day-to-day functions under the Code. The Interconnector Administrator will be designated as the responsible Party for all Type 2 Channel and Type 3 Channel communications in respect to the Interconnector.

Any procedures established for the interface to any relevant market outside SEM are outside the scope of this Agreed Procedure.

For each Interconnector, an Interconnector Error Unit and an Interconnector Residual Capacity Unit are registered.  The Interconnector Error Unit is used for managing and settling Interconnector imbalances.  The Interconnector Residual Capacity Unit is intended for the utilization utilisation of residual capacity by the relevant System Operator, subject to commercial agreement.

On registration of the Interconnector, the Interconnector Owner determines the Interconnector Administrator and the Interconnector Error Unit is registered to the Interconnector Administrator.

The procedures to be followed by the Interconnector Administrator in determining the Active Interconnector Unit Capacity Holdings and in recovering the Interconnector imbalances fall outside the scope of this Agreed Procedure.

In this Agreed Procedure, whenever a Party is required to submit data via a Type 1 Channel then the person from that Party must be duly authorised to send that data as set out in Agreed Procedure 11 "Market System Operation, Testing, Upgrading and Support".

For any values in MW or MWh for an Interconnector, positive values relate to imports to the Pool and negative values relate to exports from the Pool.

  - Registration of Interconnector Entities

An Interconnector is registered in accordance with Agreed Procedure 1 "Participant and Unit Registration and Deregistration".  As part of this process, the Interconnector Owner determines the identity of the Interconnector Administrator and the parties responsible for the registration of the Interconnector Residual Capacity Unit and the Interconnector Error Unit.

The Interconnector Owner maintains the Interconnector Registration Data in accordance with Agreed Procedure 1 "Participant and Unit Registration and Deregistration".

The Interconnector Administrator may also maintain the Interconnector Technical Data which is a subset of the Interconnector Registration Data, in accordance with this Agreed Procedure.

  - Available Transfer Capacity

The Available Transfer Capacity of an Interconnector is determined by the relevant Interconnector Administrator and the System Operator and is notified to the Market Operator by 9:30 on TD-2 by the System Operator.

Revisions to Available Transfer Capacity after the initial notifications shall be notified to the Market Operator by the relevant System OperatorInterconnector Administrator.

  - Interconnector Unit Nominations

    - Pre Gate Closure

Interconnector Users shall submit Commercial Offer Data for their Interconnector Units prior to Gate Closure. The Commercial Offer Data consists of up to ten Price Quantity Pairs per Interconnector Unit per Trading Period in the Trading Day, Maximum Interconnector Unit Import Capacity and Maximum Interconnector Unit Export Capacity.

In accordance with the Code, where no valid Commercial Offer Data is submitted in respect of an Interconnector Unit in a Trading Period, the Maximum Interconnector Unit Import Capacity and the Maximum Interconnector Unit Export Capacity is set to zero.

The Interconnector Administrator shall submit the Active Interconnector Unit Capacity Holding Data for each Interconnector Unit prior to Gate Closure consistent with the ATC in each direction.

    - Calculation of Modified Interconnector Unit Nominations

The Market Operator shall by 11:00 on the day prior to the start of the Trading Day determine Interconnector Unit Nominations for each Interconnector Unit from the Ex-Ante Indicative MSP Software Run based on the Active Interconnector Unit Capacity Holding, Commercial Offer Data and Interconnector Technical Data.  In calculating the Interconnector Unit Nominations, the Ramp Rate for each Interconnector Unit will be set to a value of 99999.9 MW/min, and the relevant Accepted Aggregate Interconnector Ramp Rate will be applied as a limit across all corresponding Interconnector Units.

The Market Operator shall submit to the Interconnector Administrator the values of Interconnector Unit Nominations for each Interconnector Unit.

The Interconnector Administrator shall calculate the Modified Interconnector Unit Nominations (MIUNs) by applying the appropriate Aggregate Interconnector Ramp Rate and with respect to the rules in Appendix 2 - Calculation of Modified Interconnector Unit Nominations. The Interconnector AdministratorMarket Operator shall in calculating the Modified Interconnector Unit Nominations (MIUNs) take into account the latest Available Transfer Capacity for the relevant Interconnector for the Trading Day. The Interconnector Administrator shall submit the MIUNs for the Optimisation Time Horizon to the Market Operator by 11:45 on the day prior to the start of the Trading Day.

The Market Operator shall by 12:00 on the day prior to the start of the Trading Day submit to each Interconnector User in respect of its Interconnector Units the corresponding MIUNs Modified Interconnector Unit Nominations.

The Market Operator shall by 12:00 on the day prior to the start of the Trading Day submit Aggregate Modified Interconnector Unit Nomination (AMIUNs) to the relevant System Operator, which represent aggregate import and export MW values per Trading Period over each Interconnector registered in their Jurisdiction.

  - SO Interconnector Trades

The System Operator using the Interconnector Residual Capacity Unit shall be entitled under the terms of the Code, subject to commercial agreement, to make SO Interconnector Trades across the relevant Interconnector in either direction, using any available Interconnector capacity which is not allocated to Interconnector Users under the aggregate of the prevailing Modified Interconnector Unit Nominations.

SO Interconnector Trades must be conducted after Gate Closure and after the calculation of the Modified Interconnector Unit Nominations MIUNs by the Market Operator and the submission to the relevant System Operator of the Aggregate Modified Interconnector Unit Nominations.

  - Post Gate Closure Reductions in ATC

In the event of a reduction in the magnitude of the Available Transfer Capacity (this includes reductions in the absolute magnitude of the Maximum Import Available Transfer Capacity and/or the Maximum Export Available Transfer Capacity) after Gate Closure the Interconnector Administrator shall notify the Market Operator of the revised Available Transfer Capacity. In addition, as required the Modified Interconnector Unit Nominations shall be recalculated by the Interconnector Administrator Market Operator and re-issued to each Interconnector User for each of their Interconnector Units.

The Modified Interconnector Unit Nominations for each Interconnector Unit shall be revised to the minimum extent necessary, taking into account any SO Interconnector Trades which are in the opposite direction to the aggregate of the MIUNs, but not taking into account any SO Interconnector Trades which are in the same direction to the aggregate of the MIUNs.

The Modified Interconnector Unit Nominations shall be recalculated by the Interconnector Administrator Market Operator in accordance with the rules in Calculation of Modified Interconnector Unit Nominations. The Interconnector Administrator Market Operator shall submit the revised MIUNs to the relevant Interconnector Users and the aggregate of the MIUNs to the relevant System Operator.

  - Point of application of values

All values which are expressed in MW, MW/min or MWh and which are not Loss-Adjusted in relation to an Interconnector, Interconnector Units, Interconnector Residual Capacity Units or Interconnector Error Units shall be applicable at the point of Connection.

- Procedure Definition

  - Interconnector Registration Data Maintenance

The Interconnector Owner or Interconnector Administrator as appropriate shall inform the Market Operator of changes to the Interconnector Technical Data prior to Gate Closure.  Any submission of Interconnector Technical Data which uses the Type 1 Communication Channel shall be made at least five Working Days before Gate Closure for the first Trading Day for which the relevant data shall be effective.  In such circumstances, the Market Operator shall confirm receipt of the data by return fax by 11:00 on the next Working Day.  In the absence of receipt of such confirmation, the relevant Party shall re-submit the relevant data.

Within 1 Working Day of receipt or by TD-1 12:00 hours, whichever is the earlier, the Market Operator shall submit the revised Interconnector Technical Data to the relevant System Operator.

  - Initial Available Transfer Capacity Notification

The System Operator Interconnector Administrator determines the Available Transfer Capacity (consisting of the Maximum Import Available Transfer Capacity and the Maximum Export Available Transfer Capacity) for each Interconnector for each Trading Period in the Optimisation Time Horizon.  ATC will be determined and submitted to the Market Operator by 09.30 on the day prior to Gate Closure (TD-2) using according to the Interconnector Available Transfer Capacity Data Transaction set out in Appendix K of the Code.

The Market Operator shall publishes ATCs via the MPI and the Market Operator Website by 10.00 on the day prior to Gate Closure (TD-2).

  - Updates to Available Transfer Capacity

Whenever there is a change to the ATC on an Interconnector after the initial notification in 3.2, the relevant Interconnector Administrator shall notify the System Operator. Where that any change in ATC occurs prior to Gate Closure, the System Operator relevant Interconnector Administrator shall submit the revised ATC to the Market Operator as soon as possible, using  in accordance with the Interconnector Available Transfer Capacity Data Transaction set out in Appendix K of the Code. The relevant System Operator Interconnector Administrator for each corresponding Interconnector shall submit the final ATCs to the Market Operator by TD+1 12:00 hours, using in accordance with the Interconnector Available Transfer Capacity Data Transaction set out in Appendix K of the Code.

Whenever there is a change in ATC prior to Gate Closure then the Interconnector Administrator shall take account of such change and if possible recalculate and resubmit the Active Interconnector Unit Capacity Holdings prior to Gate Closure.

Whenever there is a reduction in ATC, from the ATC used to produce the final Active Interconnector Unit Capacity Holdings, prior to the submission calculation of the MIUNs from the Interconnector Administrator to by the Market Operator, then where possible, the Interconnector Administrator Market Operator shall take into account the reduction in ATC in calculating the MIUNs (in paragraph 3.4 of this Agreed Procedure).

Whenever there is a change in ATC after the initial calculation of MIUNSs or their submission by the Interconnector Administrator to the Market Operator, then the Interconnector Administrator Market Operator shall recalculate revised MIUNs, taking into account the corresponding change in ATC. The revised MIUNs shall each be in the same direction and must not exceed in absolute magnitude the value of the relevant corresponding IUNs submitted by the Market Operator to the Interconnector Administrator under paragraph 3.4.6.

The Interconnector Administrator Market Operator shall submit:

- as soon as possible after identifying the change in ATC, the revised MIUNs to the relevant Interconnector Users using MITS.

- as soon as possible after identifying receiving notification of the reduction in ATC, the Aggregate MIUNs to the relevant System Operator using the Modified Interconnector Unit Nominations Data Transaction as set out in Appendix K of the Code.

- by TD+1 12:00 hours, the latest revised MIUNs to the Market Operator using the Modified Interconnector Unit Nominations Data Transaction set out in Appendix K of the Code.

  - Modified Interconnector Unit Nominations

Prior to Gate Closure, the Interconnector Administrator shall submit Active Interconnector Unit Capacity Holding Data for the relevant Interconnector to the Market Operator using the Active Interconnector Unit Capacity Holding Data Transaction set out in Appendix K of the Code.

The Market Operator shall notify ies Interconnector Users of their Active Interconnector Unit Capacity Holdings by TD-1 10:30 hours via the MPI.

The Market Operator shall runs complete the Ex-Ante Indicative MSP Software Run to determine the Ex-Ante Indicative Market Schedule (IMS) by TD-1 11:00 hours. For each Interconnector Unit, the MSP Software will use the corresponding Active Interconnector Unit Capacity Holdings, Commercial Offer Data for the Interconnector Units, a Ramp Rate for each Interconnector Unit set to a value of 99999.9 MW/min, and the Accepted Aggregate Interconnector Ramp Rate for the relevant Interconnector, applied as a limit across all Interconnector Units.

The Interconnector Unit Nominations are equal to the indicative value of MSQs (for the corresponding Interconnector Units) for those Interconnector Units that are produced calculated by Ex-Ante Indicative MSP Software Run and included in the Ex-Ante Indicative Market Schedule.

By TD-1 11:00 hours, the Market Operator submits the Interconnector Unit Nominations to the Interconnector Administrator using the Interconnector Unit Nominations Data Transaction set out in Appendix K of the Code.

The Interconnector Administrator Market Operator shall calculates and submits to the Market Operator, the Modified Interconnector Unit Nominations  MIUNs (based on the Interconnector Unit Nominations) by TD-1 11:45 using the Modified Interconnector Unit Nominations Data Transaction set out in Appendix K of the Code and submit such relevant calculated values to Interconnector Users .

The Market Operator submits the Modified Interconnector Unit Nominations to Interconnector Users  by TD-1 12:00 using the Modified Interconnector Unit Nominations Transaction as set out in Appendix K of the Code.

The Market Operator shall submits the Aggregate Modified Interconnector Unit Nominations for each Interconnector to the relevant System Operator by TD-1 12:00 using the Modified Interconnector Unit Nominations Data Transaction as set out in Appendix K of the Code.

- Swimlane Diagram

These swimlanes are provided as an illustration of the Procedural Steps. The Procedural Steps take precedence, in the event of conflict between the swimlanes and the Procedural Steps.

  - Interconnector Registration Data Maintenance

  - Swimlane - Initial Available Transfer Capacity Notification

  - Swimlane – Updates to Available Transfer Capacity

  - Modified Interconnector Unit Nominations

- Procedural Steps

  - Interconnector Registration Data Maintenance

| # | Procedural Step | Timing | Method | By/ From | To | Transaction |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Submit to Market Operator modified Interconnector Technical Data on change of data. | Prior to Gate Closure | Type 1 Channel / Type 3 Channel | Interconnector Administrator / Interconnector Owner | Market Operator | Interconnector Technical Data |
| 2 | If the modification has been received via Type 1 Channel then send confirmation by fax to sender | By next WD 11:00hours | Fax | Market Operator | Interconnector Administrator / Interconnector Owner |  |
| 3 | Submit modified Interconnector Registration to System Operator. | Earlier of within 1 WD of receipt or TD-1 12:00hrs | Type 3 Channel | Market Operator | System Operator | Interconnector Technical Data |

  - Initial Available transfer Capacity Submission

| # | Procedural Step | Timing | Method | By/ From | To | Transaction |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Determination of the Available Transfer Capacity (ATC) by the relevant System Operator Interconnector Administrator for each Interconnector and submission to the Market Operator. | 09:30, TD-2 | Type 3 Channel | System OperatorInterconnector Administrator | Market Operator | Interconnector Available Transfer Capacity Data Transaction as set out in Appendix K of the Code. |
| 2 | Publication of the ATC in accordance with Agreed Procedure 6 “Publications” | 10:00, TD-2 | Market Operator Website | Market Operator | Parties | Agreed Procedure 6 “Publications” |

  - Updates to Available Transfer Capacity

| # | Procedural Step | Timing | Method | By/ From | To | Transaction |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Identify a reduction in Available Transfer Capacity on an Interconnector Go to Step 2 and Step 56 | As occurs | - | Interconnector Administrator | - |  |
| 2 | Submit revised ATC to System Operator | As soon as possible after reduction in ATC | As agreed between IA and SO | Interconnector Administrator | System Operator |  |
| 23 | If ATC change prior to Gate Closure go to step 34 Otherwise go to step 45 | As occurs |  | System Operator Interconnector Administrator | Parties Market Operator |  |
| 34 | Submit revised ATC to Market Operator Stop | As soon as possible after reduction in ATC | Type 3 Channel | System Operator Interconnector Administrator | Market Operator | Interconnector Available Transfer Capacity Data Transaction (Appendix K of the Code) |
| 45 | Submit final ATC to Market Operator Stop | By TD+1 12:00 hours | Type 3 Channel | System Operator Interconnector Administrator | Market Operator | Interconnector Available Transfer Capacity Data Transaction (Appendix K of the Code) |
| 56 | If there is sufficient time to use the revised ATC in calculation and submission of AIUCH prior to Gate Closure go to Step 67 Else, if there is sufficient time to use the revised ATC in calculation and submission of MIUNs prior to 121:0045 TD-1 go to step 78 Else go to step 89 | - | - | Interconnector Administrator | - |  |
| 67 | Go to Step 2 in section 5.4 Modified Interconnector Unit Nominations |  |  |  |  |  |
| 78 | Go to Step 5 in section 5.4 Modified Interconnector Unit Nominations |  |  |  |  |  |
| 89 | Calculate revised MIUNs based on the revised ATC and submit the MIUNs to the relevant Interconnector Users | As soon as possible | MITS | Interconnector Administrator Market Operator | Interconnector User |  |
| 910 | Calculate revised Aggregate MIUNs based on the revised ATC and submit the MIUNs to the relevant System Operator | As soon as possible | MITS CMS | Interconnector Administrator Market Operator | System Operator |  |
| 11 | Submit revised MIUNs for the Trading Day | By TD+1 12:00 hours | Type 3 Channel | Interconnector Administrator | Market Operator | Modified Interconnector Unit Nominations Data Transaction (Appendix K of the Code) |

  - Modified Interconnector Unit Nominations

|  | Procedural Step | Timing | Method | By/ From | To | Transaction |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Submission of Active Interconnector Unit Capacity Holdings to the Market Operator | By 10:00, TD-1 | Type 3 Channel | Interconnector Administrator | Market Operator | Active Interconnector Unit Capacity Holding Data Transaction (Appendix K of the Code) |
| 2 | If there is a reduction in ATC on the Interconnector prior to Gate Closure and after submission of the Active Interconnector Unit Capacity Holdings to the Market Operator, where possible recalculate and submit the revised Active Interconnector Unit Capacity Holdings | By 10:00, TD-1 | Type 3 Channel | Interconnector Administrator | Market Operator | Active Interconnector Unit Capacity Holding Data Transaction (Appendix K of the Code) |
| 3 | Submission of Active Interconnector Unit Capacity Holdings to Interconnector Users | By 10:30, TD-1 | MPI | Market Operator | Interconnector User |  |
| 4 | Market Operator determines the Interconnector Unit Nominations from the Ex-Ante Indicative MSP Software Run and submits the data to the Interconnector Administrator | By 11:00, TD-1 | MPI | Market Operator | Interconnector Administrator | Interconnector Unit Nominations Data Transaction (Appendix K of the Code) |
| 5 | Interconnector Administrator Market Operator calculates MIUNs (based on the IUNs provided by the Market Operator and any revised ATCs that have occurred after Gate Closure) and submits to the Market Operator, the Modified Interconnector Unit Nominations | By 11.45 12:00, TD-1 | MITS Type 3 Channel | Interconnector Administrator Market Operator | Market Operator | Modified Interconnector Unit Nominations Transaction (Appendix K of the Code) |
| 6 | Submission of Modified Interconnector Unit Nominations MIUNs to Interconnector Users | By 12:00, TD-1 | MPI | Market Operator | Interconnector User | Modified Interconnector Unit Nominations Transaction (Appendix K of the Code) |
| 7 | Submission of Aggregate Modified Interconnector Unit Nominations to relevant System Operator | By 12:00, TD-1 | Type 3 Channel | Market Operator | System Operator | Modified Interconnector Unit Nominations Transaction (Appendix K of the Code) |

- Definitions and Abbreviations

Definitions

| Accepted | As defined in the Code |
| --- | --- |
| Active Interconnector Unit Capacity Holding | As defined in the Code |
| Active Interconnector Unit Capacity Holding Data | As defined in the Code |
| Active Interconnector Unit Export Capacity Holding | As defined in the Code |
| Active Interconnector Unit Import Capacity Holding | As defined in the Code |
| Aggregate Interconnector Ramp Rate | As defined in the Code |
| Aggregate Modified Interconnector Unit Nomination | As defined in the Code |
| Agreed Procedure | As defined in the Code |
| Available Transfer Capacity | As defined in the Code |
| Capacity Payment | As defined in the Code |
| Change Point | As defined in Calculation of Modified Interconnector Unit Nominations |
| Code | As defined in the Code |
| Commercial Offer Data | As defined in the Code |
| Ex-Ante Indicative Market Schedule | As defined in the Code |
| Ex-Ante Indicative MSP Software Run | As defined in the Code |
| Gate Closure | As defined in the Code |
| Interconnector | As defined in the Code |
| Interconnector Administrator | As defined in the Code |
| Interconnector Error Unit | As defined in the Code |
| Interconnector Owner | As defined in the Code |
| Interconnector Registration Data | As defined in the Code |
| Interconnector Residual Capacity Unit | As defined in the Code |
| Interconnector Technical Data | As defined in the Code |
| Interconnector Unit | As defined in the Code |
| Interconnector Unit Nominations | As defined in the Code |
| Interconnector User | As defined in the Code |
| Interconnector Technical Data | As defined in the Code |
| Intersection | As defined in Calculation of Modified Interconnector Unit Nominations |
| Logical Interconnector | As defined in Calculation of Modified Interconnector Unit Nominations |
| Market Operator | As defined in the Code |
| Market Operator Website | as defined in Agreed Procedure 1 "Participant and Unit Registration and Deregistration" |
| Market Participant Interface | as defined in Agreed Procedure 1 "Participant and Unit Registration and Deregistration" |
| Market Schedule Quantity | As defined in the Code |
| Maximum Export Available Transfer Capacity | As defined in the Code |
| Maximum Export Available Transfer Capacity | As defined in the Code |
| Maximum Interconnector Unit Export Capacity | As defined in the Code |
| Maximum Interconnector Unit Import Capacity | As defined in the Code |
| Modified Interconnector Unit Nominations | As defined in the Code |
| Minimum Interconnector Export Level | As defined in the Code |
| Minimum Interconnector Import Level | As defined in the Code |
| No Load Cost | As defined in the Code |
| Optimisation Time Horizon | As defined in the Code |
| Participant | As defined in the Code |
| Party | As defined in the Code |
| Pool | As defined in the Code |
| Price Quantity Pair | As defined in the Code |
| Quantity | As defined in the Code |
| Ramp Rate | As defined in the Code |
| Regulatory Authorities | As defined in the Code |
| Run-Through | As defined in Calculation of Modified Interconnector Unit Nominations |
| Settlement Period | As defined in the Code |
| Single Electricity Market | As defined in the Code |
| Start Up Costs | As defined in the Code |
| SO Interconnector Export Quantity | As defined in the Code |
| SO Interconnector Import Quantity | As defined in the Code |
| SO Interconnector Trade | As defined in the Code |
| System Operator | As defined in the Code |
| Trading Day | As defined in the Code |
| Trading Period | As defined in the Code |
| Trip | As defined in Calculation of Modified Interconnector Unit Nominations |
| Unit | As defined in the Code |

Abbreviations

| AMIUN | Aggregate Modified Interconnector Unit Nomination |
| --- | --- |
| ATC | Available Transfer Capacity |
| CMS | Central Market Systems |
| IUN | Interconnector Unit Nomination |
| MIUN | Modified Interconnector Unit Nomination |
| MPI | Market Participant Interface |
| MSQ | Market Schedule Quantity |
| SEM | Single Electricity Market |
| TD | Trading Day |

- Calculation of Modified Interconnector Unit Nominations

Introduction

This Appendix describes in general terms the rules used by the Interconnector Administrator Market Operator to calculate Modified Interconnector Unit Nominations from the Interconnector Unit Nominations.

Definitions

In this Appendix:

| “Change Point” | is a point that occurs whenever any Interconnector Unit Nomination changes or whenever the Aggregate Interconnector Ramp Rate changes; |
| --- | --- |
| “Intersection” | means the situation where the Interconnector is dispatched upwards but before the target can be achieved the Interconnector must be dispatched downwards in order to achieve the next Change Point’s target; |
| “Logical Interconnector” | means the interconnector in which electricity flows in one direction, i.e. electricity flowing from Scotland to Northern Ireland will form one logical interconnector and electricity flowing from Northern Ireland to Scotland will form another logical interconnector; |
| “Run-Through” | Means the situation where the Interconnector is dispatched upwards but cannot achieve its target by the next Change Point, and cannot achieve the Change Point’s expected position at the Change Point boundary; |
| “Trip” | Means a technical failure on an Interconnector which causes a reduction in the magnitude of the Available Transfer Capacity in either direction. |

Rules for the calculation of the Modified Interconnector Unit Nominations

The value of each Modified Interconnector Unit Nomination must be in the same direction and must not exceed in absolute magnitude the relevant Interconnector Unit Nomination for any Interconnector Unit in any Trading Period.

The ramp rate for the Interconnector applies to the sum of all the Interconnector Unit Nominations and not to the individual Interconnector Unit Nominations.

The Interconnector Ramp Rate is divided up pro-rata to Active Interconnector Unit Capacity Holdings of equal priority.  Active Interconnector Unit Capacity Holdings of equal priority must start and finish their ramp at the same time.

Where an Interconnector Unit Nomination for a Trading Period is less than the Interconnector Unit Nomination for the previous Trading Period, the Interconnector Unit Nomination must be ramped in order to reach the next Interconnector Unit Nomination by the Settlement Period boundary.  This ramping may take place over any number of Trading Periods, including Trading Periods within the previous day if necessary.

Where an Interconnector Unit Nomination for a Trading Period is greater than the Interconnector Unit Nomination for the previous Trading Period the Interconnector Unit Nomination must not start ramping until the Trading Period boundary.  This ramping may take place over any number of Trading Periods.

Where multiple Interconnector Unit Nominations for a particular Trading Period change in a manner that results in no change to the sum of all the Interconnector Unit Nominations for that Trading Period, the ramps shall be instantaneous.

The Interconnector ramps between 0 and Minimum Interconnector Import Level instantaneously.

The Interconnector ramps between 0 and Minimum Interconnector Export Level instantaneously.

If the total Interconnector Unit Nominations for a Trading Period are > Minimum Interconnector Export Level and < Minimum Interconnector Import Level, then each of the Interconnector Unit Nominations should be considered to be zero for the purpose of calculating the Modified Interconnector Unit Nominations.

Any instant ramp resulting from the Interconnector passing through Minimum Interconnector Export Level, 0MW or Minimum Interconnector Import Level should be allocated to the individual Interconnector Unit Nominations that are ramping at the time of the instant ramp.

Any Interconnector Unit Nominations moving in opposition to the net Interconnector flow are deemed to move instantaneously on the Change Point boundary.  The instant ramp is allocated to Interconnector Unit Nominations moving with the Interconnector on a pro-rata basis.

If a Trip occurs on a Logical Interconnector then all Interconnector Unit Nominations on that Logical Interconnector ramp instantly to the Logical Interconnector’s ATC.

The sum of the Interconnector Unit Nominations for a Logical Interconnector cannot exceed, in absolute terms, the ATC for that Logical Interconnector.