SEMO Data Publication Guide

Issue 3.5

COPYRIGHT NOTICE

All rights reserved. This entire publication is subject to the laws of copyright. This publication may not be reproduced or transmitted in any form or by any means, electronic or manual, including photocopying without the prior written permission of EirGrid plc and SONI Limited.

DOCUMENT DISCLAIMER

Every care and precaution is taken to ensure the accuracy of the information provided herein but such information is provided without warranties express, implied or otherwise howsoever arising and EirGrid plc and SONI Limited to the fullest extent permitted by law shall not be liable for any inaccuracies, errors, omissions or misleading information contained herein.

# Table of Contents

Table of Contents	2

Table of Tables	6

1	Disclaimer and Content Information	10

2	Introduction and Background	11

2.1	Scope of this Document	11

2.2	Structure of the SEMO Data Publication Guide	11

2.2.1	Data Publication Guide Sections Overview	11

2.2.2	Availability of Information in the Data Publication guide	12

3	Data Publication Types	13

3.1.1	An Overview of Data Publication Types	13

4	Data Publication Report Formats, Delivery Types, and Access Mechanisms	14

4.1	Data Publication Formats	14

4.2	Data Publication Delivery Types	14

4.3	Data Publication Access Mechanisms	15

4.3.1	Type 2 Access Mechanism	15

4.3.2	Type 3 Access Mechanism	17

5	Capacity Market Publications	18

5.1	Capacity Market Development	18

5.2	Capacity Market Data	18

5.2.1	Registration	18

5.2.2	Capacity Qualification	19

5.2.3	Capacity Auctions	19

5.2.4	Capacity Settlement Parameters	19

6	Balancing Market Publications	20

6.1	Balancing Market Development	20

6.2	Balancing Market Data	20

6.2.1	Registration	20

6.2.2	Forecast Data	21

6.2.3	Inputs, Commercial and Technical Offer Data	21

6.2.4	Settlement Data	23

6.2.5	Other	24

6.2.6	Operational Reports	24

6.3	Market Methodologies and Processes	25

6.3.1	Methodologies	25

6.3.1	Operational processes	25

6.4	Market Operator Performance	25

6.4.1	Market Operator Performance	25

7	Appendix A: Capacity Market Publication Details	26

7.1	Capacity Market Development	26

7.1.1	CM-001: Capacity Market Code (Including Agreed procedures)	26

7.1.2	CM-002: Effective Modifications Not Yet incorporated Into The Capacity Market Code	26

7.1.3	CM-003: Modification Finalisation Date for a Capacity Auction	26

7.1.4	CM-004: Modification Proposal and Associated Status	26

7.1.5	CM-005: Regulatory Authority Notices	27

7.1.6	CM-006: Quarterly Report on status of Modification Proposals	27

7.1.7	CM-007: Decision on Modification Proposal	27

7.1.8	CM-008: Modification proposal Form	27

7.1.9	CM-009: Capacity Auction Monitor, Terms of Reference	27

7.1.10	CM-010: Capacity Auction Audit report (non-Confidential)	28

7.1.11	CM-011: Dispute Process Timetable	28

7.1.12	CM-012: Notice of Dispute Form	28

7.1.13	CM-013: Panel Chair, Panel Members and Associated CVs	28

7.2	Capacity Market Data	29

7.2.1	CM-014: List of Capacity Market Code Parties and Participants	29

7.2.2	CM-015: Notice of Making or Lifting a Suspension Order	29

7.2.3	CM-016: Termination Order	29

7.2.4	CM-017: Accession Fees and Participation Fees	29

7.2.5	CM-018: Qualification Charge, Fixed System Operator Charge and Variable System Operator Charge	30

7.2.6	CM-019: Market Qualification report	30

7.2.7	CM-020: Reporting Schedule for Awarded New Capacity	31

7.2.8	CM-021: Implementation Progress Reports Form	31

7.2.9	CM-022: Notice of Termination of Awarded Capacity	31

7.2.10	CM-023: Capacity Auction Timetable	31

7.2.11	CM-024: Capacity Auction Information Pack	32

7.2.12	CM-025: Cancellation of capacity Auction	32

7.2.13	CM-027: Capacity Auction results	32

7.2.14	CM-028: Amended Capacity Auction timetable Following General Systems Failure	33

7.2.15	CM-029: Updated Annual Capacity Payment Exchange Rate and Monthly Capacity Payment Exchange Rates	33

7.2.16	CM-030: Capacity Auction results – Constraints Data	34

8	Appendix B: Balancing Market Details	35

8.1	Balancing Market Development	35

8.1.1	BM-118: Trading and Settlement Code (including Agreed Procedures)	35

8.1.2	BM-119: Schedule and dates of Modification Panel meetings	35

8.1.3	BM-120: Modification Proposal	35

8.1.4	BM-121: Public Consultation on Modification Proposal	35

8.1.5	BM-122: Responses to Public Consultation on Modification Proposal	35

8.1.6	BM-123: Further information on Modification Proposal	36

8.1.7	BM-124: Final Recommendation Report	36

8.1.8	BM-125: Regulatory Authority decision on Final Modification Recommendation	36

8.1.9	BM-126: Members and Chairperson of the Modification Committee	36

8.1.10	BM-127: Terms of Reference for Market Operator Audit	37

8.1.11	BM-128: Audit Report	37

8.2	Balancing Market Data	37

8.2.1	BM-001: List of Registered Units Report	37

8.2.2	BM-002: List of Registered Parties Report	38

8.2.3	BM-003: Registered Capacity Report	38

8.2.4	BM-004: Making or Lifting of a Suspension Order	39

8.2.5	BM-005: Termination Order	39

8.2.6	BM-006: Accession Fees and participation Fees	39

8.2.7	BM-007: Supplier Suspension Delay Period	39

8.2.8	BM-008: Generator Suspension Delay period	40

8.2.9	BM-009: Annual Load Forecast Report	40

8.2.10	BM-010: Daily Load Forecast Summary Report	40

8.2.11	BM-011: Monthly Load Forecast and Assumptions Report	41

8.2.12	BM-012: Four Day Rolling Wind Unit Forecast Report	41

8.2.13	BM-013: Four Day Aggregated Rolling Wind Unit Forecast Report	42

8.2.14	BM-014: Forecast Imbalance Report	42

8.2.15	BM-015: Net Imbalance Volume Forecast Report	43

8.2.16	BM-016: Aggregated Wind Forecast Report	44

8.2.17	BM-018: Daily Interconnector NTC Report	44

8.2.18	BM-019: Annual Combined Loss Adjustment Factor Report	45

8.2.19	BM-020: Aggregated Final Physical Notifications Report	45

8.2.20	BM-021: Anonymised Inc / Dec Curves Report	46

8.2.21	BM-022: System Shortfall Imbalance Index & flattening Factor	46

8.2.22	BM-023: Final Physical Notifications Report	47

8.2.23	BM-024: Generator Unit Under Test Notice	47

8.2.24	BM-025: Imbalance Price Report (Imbalance Pricing Period)	48

8.2.25	BM-026: Imbalance Price Report (Imbalance Settlement Period)	48

8.2.26	Imbalance Price Supporting Information Report	49

8.2.27	BM-028: Price Materiality Threshold	51

8.2.28	BM-029: Settlement Recalculation Threshold	51

8.2.29	BM-030: Imbalance Weighting Factor	51

8.2.30	BM-031: De Minimis Acceptance Threshold	51

8.2.31	BM-032: Daily Technical Offer Data Report	51

8.2.32	BM-033: Forecast Availability Report	55

8.2.33	BM-034: Commercial Offer Data Report	55

8.2.34	BM-036: Daily Demand Control Data Transaction Report	57

8.2.35	BM-037: Daily Dispatch Instructions (D+1) Report	57

8.2.36	BM-038: Daily Dispatch Instructions (D+4) Report	58

8.2.37	BM-039: Tracked Difference Payment Shortfall Amount	59

8.2.38	BM-040: Carbon Price	59

8.2.39	BM-041: Natural Gas Fuel Price	59

8.2.40	BM-042: Oil Fuel Price	59

8.2.41	BM-043: Peaking Unit THeoretical Efficiency	60

8.2.42	BM-044: Natural Gas Carbon Intensity Factor	60

8.2.43	BM-045: Oil Carbon Intensity Factor	60

8.2.44	BM-046: Aggregated Settlement Period	60

8.2.45	BM-047: Annual Stop-Loss Limit Factor	60

8.2.46	BM-048: Billing Period Stop-Loss Limit Factor	61

8.2.47	BM-049: Total Capacity Awarded	61

8.2.48	BM-050: Response Period Duration	61

8.2.49	BM-051: Annual Capacity charge Exchange Rate	61

8.2.50	BM-052: Capacity Duration Exchange Rate	61

8.2.51	BM-053: Capacity Charge Metered Quantity Factor	62

8.2.52	BM-054: Supplier Capacity Charge price	62

8.2.53	BM-055: Difference Payment Socialisation Mulitplier	62

8.2.54	BM-056: Market Price Cap	62

8.2.55	BM-057: Market Price Floor	63

8.2.56	BM-058: Residual meter Volume Intervan Proportion	63

8.2.57	BM-059: Residual Error Volume Price	63

8.2.58	BM-060: Fixed Market Operator Charge (supplier Unit)	63

8.2.59	BM-061: Fixed Market Operator Charge (Generator Unit)	63

8.2.60	BM-062: Variable Market Operator Price	64

8.2.61	BM-063: Engineering Tolerance	64

8.2.62	BM-064: MW Tolerance	64

8.2.63	BM-065: System Per Unit Regulation Factor	64

8.2.64	BM-066: DOG/PUG/TOLOG/TOLUG for Over/Under Generation	65

8.2.65	BM-067: Transmission Loss Adjustment Factor	65

8.2.66	BM-068: Distribution Loss Adjustment Factor	65

8.2.67	BM-069: Combined Loss Adjustment Factor	66

8.2.68	BM-070: Imperfections Price	66

8.2.69	BM-071: Imperfections charge Factor	66

8.2.70	BM-072: Testing Tariff	66

8.2.71	BM-073: Schedule of Testing Tariffs	67

8.2.72	BM-074: Fixed Credit Requirement, in respect of Generator and/or Supplier Units	67

8.2.73	BM-075: Days in Historical Assessment Period	67

8.2.74	BM-076: Analysis Percentile Parameter	67

8.2.75	BM-077: Credit Cover Adjustment Trigger	67

8.2.76	BM-078: Currency Cost Price	68

8.2.77	BM-079: Currency Cost Change Factor	68

8.2.78	BM-080: Annual Maintenance Schedule – Transmission Line Outage	68

8.2.79	BM-081: Two Year Maintenaince Schedule – Generator outages Schedule	68

8.2.80	BM-082: Monthly Maintenance Schedule – Generator Unit Outages	69

8.2.81	BM-083: Monthly Maintenance Schedule – Transmission System Line Outages	69

8.2.82	BM-084: Daily Trading Exchange Rate Report	69

8.2.83	BM-162: LTS Operational Schedule Report	69

8.2.84	BM-163: RTIC Operational Schedule Report	70

8.3	Settlement Data	71

8.3.1	BM-085: Settlement Calendar	71

8.3.2	BM-086: Daily Meter Data Report	72

8.3.3	BM-087: Initial Interconnector Flows and Residual Capacity Report	73

8.3.4	BM-088: Daily Transmission Outage Schedule Report	73

8.3.5	BM-089: Average System Frequency Report	74

8.3.6	BM-090: SO Interconnector Trades Report	75

8.3.7	BM-091: Unit Data Report	75

8.3.8	BM-094: Metered Volumes by Jurisdiction	76

8.3.9	BM-095: Balancing and Imbalance Market Cost View Report	77

8.3.10	BM-096: Dispatch Quantity	77

8.3.11	BM-098: Aggregated Contracted Quantities for Generation Report	78

8.3.12	BM-099: Aggregated Contracted Quantities for Demand Report	78

8.3.13	BM-100: Aggregated Contracted Quantities for Wind Report	79

8.3.14	BM-101: Average Outturn Availability	79

8.3.15	BM-159: Outturn Availability	80

8.3.16	BM-160: Hourly Dispatch Instructions Report	80

8.3.17	BM-161: Hourly SO Interconnector Trades Report	81

8.3.18	BM-102: Unit Under Test	81

8.3.19	BM-103: Energy Market Financial Publication – Indicative	82

8.3.20	BM-104: Energy Market Financial Publication – Initial	83

8.3.21	BM-165: Energy Market Financial Publication – m+4	83

8.3.22	BM-166: Energy Market Financial Publication – m+13	83

8.3.23	BM-167: Energy Market Financial Publication – Adhoc	84

8.3.24	BM-105: Capacity Payments by Market	84

8.3.25	BM-106: Capacity Payments by Unit	85

8.3.26	BM-107: Daily Generator Unit Technical Characteristic Data Transaction Report	85

8.3.27	BM-108: Credit Assessment Price	86

8.3.28	BM-109: Billing Period Undefined Potential Exposure Quantity	86

8.3.29	BM-110: Trading Payments and Charges	86

8.3.30	BM-111: Capacity Quantity Scaling factor	87

8.3.31	BM-112: Required Capacity Quantity	87

8.3.32	BM-113: Reserve Adjustment Capacity	87

8.3.33	BM-145: Energy Market Information Publication – Indicative	87

8.3.34	BM-146: Energy Market Information Publication – Initial	88

8.3.35	BM-168: Energy Market Information Publication – M+4	88

8.3.36	BM-169: Energy Market Information Publication – M+13	89

8.3.37	BM-170: Energy Market Information Publication – Adhoc	89

8.3.38	BM-147: Capacity Market Information Publication – Indicative	89

8.3.39	BM-148: Capacity Market Information Publication – Initial	90

8.3.40	BM-171: Capacity Market Information Publication – M+4	90

8.3.41	BM-172: Capacity Market Information Publication – M+13	90

8.3.42	BM-173: Capacity Market Information Publication – Adhoc	91

8.3.43	BM-149: Capacity Market Financial Publication – Indicative	91

8.3.44	BM-150: Capacity Market Financial Publication – Initial	92

8.3.45	BM-174: Capacity Market Financial Publication – M+4	92

8.3.46	BM-175: Capacity Market Financial Publication – M+13	92

8.3.47	BM-176: Capacity Market Financial Publication – Adhoc	93

8.3.48	BM-151: Metered Generation Information Publication - Indicative	93

8.3.49	BM-152: Metered Generation Information Publication – Initial	94

8.3.50	BM-177: Metered Generation Information Publication – M+4	94

8.3.51	BM-178: Metered Generation Information Publication – M+13	94

8.3.52	BM-179: Metered Generation Information Publication – Adhoc	95

8.3.53	BM-153: Make Whole Payments	95

8.3.54	BM-154: Cross Border Monthly Capacity Data	96

8.3.55	BM-155: Cross Border Weekly Energy Data	97

8.3.56	BM-115: Proposed Market Operator Isolated Market System Testing Schedule	97

8.3.57	BM-116: Calculations and Methodology used by the Market Operator during administered Settlement	98

8.3.58	BM-117: Market Operator Performance Report	98

8.3.59	BM-130: Quarterly Renewable Dispatch Down (Constraint & Curtailment)	98

8.3.60	BM-131: Annual Renewable Dispatch Down (Constraint & Curtailment)	98

8.3.61	BM-132: Quarterly Constraint Cost Outturn	98

8.3.62	BM-137: Monthly Constraint Update	99

8.3.63	BM-138: Weekly Constraint Update	99

8.3.64	BM-139: Ad Hoc Constraint Update	99

8.4	Balancing Market Methodologies and Processes	99

8.4.1	BM-129: Ad Hoc Constraint Update	99

8.4.2	BM-136: Balancing market Principles Statement	100

8.4.3	BM-146: Determination of Scheduling and Dispatch Policy Parameters	100

8.4.4	BM-147: System Operator and Non-Marginal Flagging	100

8.4.5	BM-140: Demand Forecasting	100

8.4.6	BM-141: Scheduling	100

8.4.7	BM-142: Dispatch	101

8.4.8	BM-143: Cross-Zonal Actons	101

8.4.9	BM-144: Wind Farm Controllability Categorisation	101

8.5	Balancing Market Operator Performance	101

8.5.1	BM-133: Annual All-Island Transmission System Performance Report	101

8.5.2	BM-134: Schduling and Dispatch Policy Parameter Performance Report	101

8.5.3	BM-135: Annual Schduling and Dispatch Process Audit Report	102

9	Appendix C: SEMO Website APi Specification	103

# Table of Tables

Table 1: Structure of the SEMO Data Publication Guide	12

Table 2: An Overview of Data Publication Types	13

Table 3: Data Publication Formats	14

Table 4: Data Publication Access Mechanisms	15

Table 5: Capacity Market: Market Development Publications	18

Table 6: Capacity Market: Registration Publications	19

Table 7: Capacity Market: Capacity Qualification Publications	19

Table 8: Capacity Market: Capacity Auction Publications	19

Table 9: Capacity Market: Capacity Settlement Parameter Publications	19

Table 10: Balancing Market: Market Development Publications	20

Table 11: Balancing Market: Registration Publications	21

Table 12: Balancing Market: Forecast Publications	21

Table 13: Balancing Market: Inputs, Commercial, and Technical Offer Data Publications	23

Table 14: Balancing Market: Settlement Data Publications	24

Table 15: Balancing Market: Other Publications	24

Table 16: Balancing Market: Operational Reports	24

Table 17: Balancing Market: Methodologies	25

Table 18: Balancing Market: Operational Processes	25

Table 19: Balancing Market: Market Methodologies and Processes	25

Table 20: Market Qualification Report	31

Table 21: Capacity Auction Results Main Section	33

Table 22: Capacity Auction Results Footer Section	34

Table 23: List of Registered Units Report	38

Table 24: List of Registered Parties Report	38

Table 25: Registered Capacity Report	39

Table 26: Annual Load Forecast Report	40

Table 27: Daily Load Forecast Summary Report	41

Table 28: Monthly Load Forecast and Assumptions Report	41

Table 29: Four Day Rolling Wind Unit Forecast Report	42

Table 30: Four Day Aggregated Rolling Wind Unit Forecast Report	42

Table 31: Forecast Imbalance Report	43

Table 32: Net Imbalance Volume Forecast Report	43

Table 33: Aggregated Wind Forecast Report	44

Table 34: Daily Interconnector NTC Report	45

Table 35: Combined Loss Adjustment Factor Report	45

Table 36: Aggregated Physical Notifications Report	46

Table 37: Anonymised Inc/Dec Curves Report	46

Table 38: Daily Shortfall Imbalance Index & Flattening Factor Report	47

Table 39: Final Physical Notifications Report	47

Table 40: Imbalance Price Report (Imbalance Pricing Period)	48

Table 41: Imbalance Price Report (Imbalance Settlement Period)	49

Table 42: Imbalance Price Supporting Information Report	50

Table 43: Definition of ConstraintID Values Used in BM-027	51

Table 44: Daily Technical Offer Data Report - Standard Units	55

Table 45: Forecast Availability Report	55

Table 46: Commercial Offer Data Report – General Public	57

Table 47: Daily Demand Control Data Transaction Report	57

Table 48: Daily Dispatch Instructions (D+1) Report	58

Table 49: Daily Dispatch Instructions (D+4) Report	59

Table 50: Under/Over Generation Report Header Field Description	65

Table 51: Under/Over Generation Report Field Description	65

Table 52: Daily Trading Day Exchange Rate Report	69

Table 53: LTS Operational Schedule Report	70

Table 54: BM-163: RTIC Operational Schedule Report	71

Table 55: Settlement Calendar Header Field Description	71

Table 56: Settlement Calendar Field Description	72

Table 57: Daily Meter Data Report	73

Table 58: Initial Interconnector Flows and Residual Capacity Report	73

Table 59: Daily Transmission Outage Schedule Report	74

Table 60: Average System Frequency Report	74

Table 61: SO Interconnector Trades Report	75

Table 62: Unit Data Report	76

Table 63: Metered Volumes by Jurisdiction Report Header Field Description	76

Table 64: Metered Volumes by Jurisdiction Report	76

Table 65: Balance and Imbalance Market Cost View Field Description	77

Table 66: Dispatch Quantity	77

Table 67: Aggregated Contracted Quantities for Generation Report	78

Table 68: Aggregated Contracted Quantities for Demand Report	79

Table 69: Aggregated Contracted Quantities for Wind Report	79

Table 70: Average Outturn Availability Report	80

Table 71: Outturn Availability Report	80

Table 72: Hourly Dispatch Instructions Report	81

Table 73: Hourly SO Interconnector Trades Report	81

Table 74: Unit Under Test Report	82

Table 75: General Public Settlement Header Field Description	82

Table 76: General Public Settlement Detail Field Description	83

Table 77: Capacity Payments by Market Report Header Field Description	84

Table 78: Capacity Payments by Market Report Detail Field Description	85

Table 79: Capacity Payments by Unit Report - Header Field Description	85

Table 80: Capacity Payments by Unit Report - Detail Field Description	85

Table 81: Daily Generator Unit Technical Characteristics Data Transaction Report	86

Table 82: Energy Market Information Publication Header Field Description	88

Table 83: Energy Market Information Publication Detail Field Description	88

Table 84: Capacity Market Information Publication Header Field Description	90

Table 85: Capacity Market Information Publication Detail Field Description	90

Table 86: Capacity Market Financial Publication Header Field Description	91

Table 87: Capacity Market Financial Publication Detail Field Description	92

Table 88: Metered Generation Information Publication -Header Field Description	93

Table 89: Metered Generation Information Publication -Detail Field Description	94

Table 90: Make Whole Payments Report Header Field Description	95

Table 91: Make Whole Payments Report Detail Field Description	96

Table 92: Cross Border Monthly Capacity Report Header Field Description	96

Table 93: Cross Border Monthly Capacity Report Summary Field Description	96

Table 94: Cross Border Monthly Capacity Report Field Description	96

Table 95: Cross Border Energy Data Report Header Field Description	97

Table 96: Cross Border Energy Data Report Summary Field Description	97

Table 97: Cross Border Energy Data Report Field Description	97

Document History

| Version | Date | Author | Comment |
| --- | --- | --- | --- |
| 1.0 | 05 May 2017 | I-SEM Programme | Initial Release of I-SEM Data Publication Guide. Cross-Reference: Level 2 Milestone # 231 |
| 1.1 | 14 October 2017 | I-SEM Programme | Incremental release of I-SEM Data Publication Guide. |
| 1.2 | 01 December 2017 | I-SEM Programme | Incremental release of I-SEM Data Publication Guide Cross-Reference: Level 2 Milestone # 7k |
| 1.3 | 09 February 2018 | I-SEM Programme | Incremental release of I-SEM Data Publication Guide Cross-Reference: Level 2 Milestone # 234a |
| 2.0 | 28 March 2018 | I-SEM Programme | Incremental release of I-SEM Data Publication Guide Cross-Reference: Level 2 Milestone # 234b |
| 3.0 | 14 December 2018 | SEMO | Updates to align branding for SEMO only Minor edits for website navigation, website API Updates to align with I-SEM Technical Specification, Volume C, Balancing Market, Releases 9.0 and 9.1 |
| 3.1 | 28 February 2020 | SEMO | Minor updates to section 8.3.1 (Settlement Calendar) to include additional MARKET type FMOC. |
| 3.2 | 16 October 2020 | SEMO | Minor updates to BM-145 the Energy Market Information report changing the VALUE: ORDER from NUMBER(2) to STRING(25) |
| 3.3 | 14/01/2022 | SEMO | - BM_145/ – Energy Market Financial Publication Report - Addition of the following reports to cover Energy Market Financial Publication Report for M+4, M+13 & Ad-hoc. - BM_146 – Energy Market Financial Publication Report - Addition of the following reports to cover Energy Market Financial Publication Report for M+4, M+13 & Ad-hoc. - BM_147 – Capacity Market Information Publication Report Addition of the following reports to cover Capacity Market Information Publication Report for M+4, M+13 & Ad-hoc. - BM_148 – Capacity Market Information Publication Report Addition of the following reports to cover Capacity Market Information Publication Report for M+4, M+13 & Ad-hoc. - BM_149 – Capacity Market Financial Publication Report Addition of the following reports to cover Capacity Market Financial Publication Report for M+4, M+13 & Ad-hoc. - REPT_066 – Metered Volumes by Jurisdiction. - Addition of the following reports to Metered Volumes by Jurisdiction Report for M+4, M+13 & Ad-hoc. - REPT_066 - Metered Volumes by Jurisdiction Report - M+4 - REPT_066 - Metered Volumes by Jurisdiction Report - M+13 - REPT_066 - Metered Volumes by Jurisdiction Report - PUB_Daily Meter Data (Public). - Addition of the following reports to Metered PUB_Daily Meter Data (Public)for M+4, M+13 & Ad-hoc. - PUB_Daily Meter Data M+4 (Public) M+4 - PUB_Daily Meter Data M+13 (Public)M+13 - PUB_Daily Meter Data Adhoc (Public) |
| 3.4 | 25/07/2024 | SEMO | General Cleanup of document. Scheduling and Dispatch Change Programme related Balancing Market Publication Detail changes for the following reports, - BM-014: Forecast Imbalance Report - BM-034: Commercial Offer Data Report - BM-032: Daily Technical Offer Data Report |
| 3.5 | 21/04/2026 | SEMO | - Update to section 8.2.31 to update number format for ESPS STORAGE CYC EFY field within Daily Technical Offer Data Report - Addition of Synchronous Condenser Fuel type to BM-032: Daily Technical Offer Data Report and BM-091: Unit Data Report |

Distribution List

Name

General Public

# Disclaimer and Content Information

This document has been prepared to provide the general public with information about publicly available data from the Balancing market and Capacity market. The following disclaimers relate to the content of this document and any use by Market Participants or the general public of the information provided therein. - EirGrid and SONI accept no responsibility for decisions made or actions taken by Participants as a result of the information presented in this document or associated documents. Furthermore, EirGrid and SONI do not indemnify any commercial or organisational decisions made by Participants in relation to the information herein. - The information provided in this document is based on documentation and information provided by the technology systems vendors and service providers whose systems and services are used in the operations of the Balancing market and Capacity market. - Further changes to the data or access mechanisms presented in this document may change as a result of the ongoing work to update the public website and/or changes to the market design through the market modifications process. As such, EirGrid and SONI will be issuing updated versions of this document and any associated documents to reflect those changes.

# Introduction and Background

## Scope of this Document

The I-SEM Data Publication Guide provides details of the information published for and available to the general public by:

- EirGrid, as Transmission System Operator, as required to discharge its associated obligations in relation to the Capacity Market, as set out in the Capacity Market Code,

- SEMO, as Market Operator, as required to discharge its associated obligations in relation to the Balancing Market, as set out in the Trading and Settlement Code (TSC),

## Structure of the SEMO Data Publication Guide

### Data Publication Guide Sections Overview

The following table describes the sections within this Data Publication guide. It details the availability of the information within this issue and what additional information is to be provided in future issues.

| Section # | Section Name | Content included in this Issue |
| --- | --- | --- |
| 1 | Disclaimer and Content Information Important information in relation to interpretation of the content presented in the SEMO Data Publication Guide. | Issue 3 contains the complete data for this topic. |
| 2 | Introduction and Background Sets out the scope of the SEMO Data Publication Guide and describes the structure of the document. | Issue 3 contains the complete data for this topic. |
| 3 | Data Publication Types Provides a definition of the data publication report types and how they are organised on the SEMO website. | Issue 3 contains the complete data for this topic. |
| 4 | Data Publication Report Formats and Access Mechanisms Provides a definition of the data publication report types and the mechanisms by which information will be access via the SEMO website. | Issue 3 contains the complete data for this topic. |
| 5 | Capacity Market Publications Provides a list of the data publications associated with 1) market development and 2) a list of data publications relating to market data (relating to the Capacity Market qualifications and auctions). | Issue 3 contains the complete data for this topic. |
| 6 | Balancing Market Publications Provides a list of the data publications associated with the Balancing Market). | Issue 3 contains the complete data for this topic. |
| App A | Capacity Market Publication Details Provides the details for each data publication associated with the Capacity Market. This includes the report name, data types, report format, and the access mechanisms available for the report. Furthermore, where applicable, a sample report file or extract is provided. | Issue 3 contains the complete data for this topic. |
| App B | Balancing Market Publication Details Provides the details for each data publication associated with the Balancing Market. This includes the report name, data types, report format, and the access mechanisms available for the report. Furthermore, where applicable, a sample report file or extract is provided. | Issue 3 contains the complete data for this topic. |
| App C | SEMO Website API Specification Provides the technical specification for accesing and using the SEMO public website API. | Issue 3 contains the complete data for this topic. |

Table 1: Structure of the SEMO Data Publication Guide

### Availability of Information in the Data Publication guide

This Data Publication Guide is a living document and is subject to change as the SEMO Balancing Market and SEMO Capacity Market evolve. If there is information not yet available for publication, but known to the Market Operators and/or TSOs, it will be highlighted in this document using the format shown below.

Note: If  information planned for inclusion in the SEMO Data Publication Guide is not yet available for a given issue, it will be noted throughout the document, highlighted by this colored frame.

# Data Publication Types

This section describes the types of data publications available to the general public and how those data publication types are organised and classified within the SEMO website.

Many different data publications types will be available for the Balancing Market and Capacity Market. The different types of data publications described in this section of the document may apply to all markets.

### An Overview of Data Publication Types

| Data Publication Type | Definition |
| --- | --- |
| Market Data | Data publications related to the input data and parameters used by the various markets for operations and the data resulting from the markets’ operations |
| Market Development | Data publications related to the process by which the market rules (and associated obligations) are agreed, modified and reported on via the Modifications Committee, along with the processes by which Disputes are handled and Regulatory Authority decisions published. |
| Market Messages | Data publications (messages) which provide updates on the status of market operations. This includes notices on identifying pricing, settlement or invoicing delays, providing details of outages of the technology systems used in the operations of the market, and notification of resettlement activities. |
| Market Operator Performance | Data publications which provide information on the performance of market operations. This includes financial statistics, help desk call statistics, details of operational timing, and occurrence of occasional components such as disaster recovery. |
| Market Methodologies and Processes | Data publications that specify methodologies used in the calculation or formation of market data, and operational processes used by the market operators. |

Table 2: An Overview of Data Publication Types

# Data Publication Report Formats, Delivery Types, and Access Mechanisms

This section describes the different formats in which the data publications will be delivered and how the general public may access the data publications on the SEMO website.

## Data Publication Formats

A variety of formats will be used to deliver data to the general public. The table below offers the list of applicable data publication types.

| Format Code | Name | Description |
| --- | --- | --- |
| CSV | Comma Separated Values | A CSV file stores tabular data (numbers and text) in plain text. Each line of the file is a data record. Each record consists of one or more fields, separated by commas. |
| XML | Extensible Markup Language | Extensible Markup Language (XML) is a markup language that defines a set of rules for encoding documents in a format that is both human-readable and machine-readable. An XML file stores related data within a given hierarchy, described (marked up) by elements. The structure of the XML file is defined and validated by an XML schema (*.xsd). |
| PDF | Portable Document Format | The Portable Document Format (commonly referred to as PDF) is a file format used to present documents in a manner independent of application software, hardware, and operating systems. |
| DOC | Microsoft Word Document | A document formatted for viewing in Microsoft Word. If the file extension is *.docx, this notates the file is compatible with the Office Open XML international standard for Office documents. |
| XLS | Microsoft Excel Document | A document formatted for viewing in Microsoft Excel. |

Table 3: Data Publication Formats

## Data Publication Delivery Types

The delivery type for a data publication characterizes the nature of how the data are stored and presented by the SEMO website and how the general public may retrieve them. There are two delivery types: static and dynamic.

- Static data publication delivery type is for those publications that are created by the particular market system or market process and uploaded to the SEMO website for retrieval individually by the general public. These publications can be delivered in a variety of formats.

- Dynamic data publications are those publications that are created by the particular market system or market process and whose data are uploaded to the SEMO website for retrieval by the general public using a search/filter function. These publications can be delivered in a variety of formats.

## Data Publication Access Mechanisms

A variety of access mechanisms will be provided to the general public for acquiring the various data publications via the SEMO website. Currently, the following methods are available.

| Method | Details |
| --- | --- |
| Type 2 (browser-based) retrieval | For static data publication delivery, this mechanism provides the general public with a webpage, or series of web pages, that list the available data publication for download, with a hyperlink for downloading the given report. For dynamic data publication delivery, this mechanism provides the general public with a webpage, or series of web pages, that allow the user to query specific data publications with a given set of filter criteria. This webpage then lists the available data publication/s for download, with a hyperlink for downloading the given report/s. |
| Type 3 (API-based) retrieval | For static data publication delivery, this mechanism provides the general public with an API (application programmatic interface) for retrieving a list of data publications, and the data publications themselves. For dynamic data publication delivery, this mechanism provides the general public with an API (application programmatic interface) that allow the user to query specific data publications with a given set of filter criteria. This API then returns the data publication via an XML message. |

Table 4: Data Publication Access Mechanisms

### Type 2 Access Mechanism

A description of Type 2 access for market publications and data on the SEMO website is shown below.

Conventions Used in this Section

| Convention | Description |
| --- | --- |
| “SEMO Home Page” | The starting location/page when navigating to http://sem-o.com |
| The “>” separator | Indicates a progression from one page to another via a hyperlink. E.g. SEMO Home Page > Market Data > Static Reports describes the linking from the SEMO Home Page to the SEMO Static Reports page. |
| Any reference made in “quotes” | Refers to a specific section of a webpage. E.g. “Market Messages” is a section of the SEMO Home Page. |

For the SEMO Website:

| Publication Type | Location |
| --- | --- |
| Market Date: Static Reports | SEMO Home Page > Market Data > Static Reports |
| Market Data: Dynamic Reports | SEMO Home Page > Market Data > Dynamic Rpeorts |
| Market Development - Rules - Modifications | SEMO Home Page > Publications > Market Development > Market Rules > Modifications |
| Market Operator Performance | SEMO Home Page > Publications > General Publications |
| Market Methodologies and Processes | SEMO Home Page > Publications > General Publications |
| TSO Resonsibilities | SEMO Home Page > Publications > TSO Responsibilities |
| Market Messages | SEMO Homepage “Market Messages” SEMO Home Page > Market Messages |

An example of SEMO > Market Data > Static Reports is shown below.

Figure 1: Example of Website Navigation for SEMO > Market Data > Static Reports

### Type 3 Access Mechanism

Type 3 access for static reports from the SEMO website is via a public API. Details of this API are found in Appendix C: SEMO Public Website API Specification.

# Capacity Market Publications

The data publications that will be available via the SEMO website for the Capacity Market can be organised into two categories: market development and market data. The definition for those groupings, and a list of the known data publications for the Capacity Market, are shown below.

- Market Development – the process by which the market rules (and associated obligations) are agreed, modified and reported on via the Modifications Committee, along with the processes by which Disputes are handled and Regulatory Authority decisions published.

- Market Data – data and information published in relation to Capacity Market registration, capacity qualification, capacity auction scheduling, conduct and results, secondary trading of Capacity Rights and settlement input parameters.

## Capacity Market Development

| ID | Document | Category | Format |
| --- | --- | --- | --- |
| CM-001 | Capacity Market Code (including Agreed Procedures) | To be added later | Word / PDF |
| CM-002 | Effective Modifications not yet incorporated into the Capacity Market Code | To be added later | Word / PDF |
| CM-003 | Modification Finalisation Date for a Capacity Auction | To be added later | Word / PDF |
| CM-004 | Modification Proposal and associated status | To be added later | Word / PDF |
| CM-005 | Regulatory Authority Notices | To be added later | Word / PDF |
| CM-006 | Quarterly Report on status of Modification Proposals | To be added later | Word / PDF |
| CM-007 | Decision on Modification Proposal | To be added later | Word / PDF |
| CM-008 | Modification Proposal Form | To be added later | Word / PDF |
| CM-009 | Capacity Auction Monitor, terms of reference | To be added later | Word / PDF |
| CM-010 | Capacity Auction Audit Report (non-confidential) | To be added later | Word / PDF |
| CM-011 | Dispute Process Timetable | To be added later | Word / PDF |
| CM-012 | Notice of Dispute Form | To be added later | Word / PDF |
| CM-013 | Panel Chair, Panel Members and associated CVs | To be added later | Word / PDF |

Table 5: Capacity Market: Market Development Publications

## Capacity Market Data

### Registration

| ID | Document | Category | Format |
| --- | --- | --- | --- |
| CM-014 | List of Capacity Market Code Parties and Participants | Registration | Word / PDF |
| CM-015 | Notice of making or lifting a Suspension Order | Registration | Word / PDF |
| CM-016 | Termination Order | Registration | Word / PDF |
| CM-017 | Accession Fees and Participation Fees | Registration | Word / PDF |
| CM-018 | Qualification Charge, Fixed System Operator Charge and Variable System Operator Charge | Registration | Word / PDF |

Table 6: Capacity Market: Registration Publications

### Capacity Qualification

| ID | Document | Category | Format |
| --- | --- | --- | --- |
| CM-019 | Market Qualification Report | Qualification | CSV |
| CM-020 | Reporting schedule for Awarded New Capacity | Qualification | Word / PDF |
| CM-021 | Implementation Progress Reports form | Qualification | Word / PDF |
| CM-022 | Notice of termination of Awarded Capacity | Qualification | Word / PDF |

Table 7: Capacity Market: Capacity Qualification Publications

### Capacity Auctions

| ID | Document | Category | Format |
| --- | --- | --- | --- |
| CM-023 | Capacity Auction Timetable | Capacity Auction | Word / PDF |
| CM-024 | Capacity Auction Information Pack | Capacity Auction | Word / PDF |
| CM-025 | Cancellation of Capacity Auction | Capacity Auction | Word / PDF |
| CM-027 | Capacity Auction Results | Capacity Auction | CSV |
| CM-028 | Amended Capacity Auction Timetable following General System Failure | Capacity Auction | Word / PDF |
| CM-030 | Capacity Auction Results – Constraints Data | Capacity Auction | CSV |

Table 8: Capacity Market: Capacity Auction Publications

### Capacity Settlement Parameters

| ID | Document | Category | Format |
| --- | --- | --- | --- |
| CM-029 | Updated Annual Capacity Payment Exchange Rate and Monthly Capacity Payment Exchange Rates | Market Parameters | Word / PDF |

Table 9: Capacity Market: Capacity Settlement Parameter Publications

# Balancing Market Publications

The data publications that will be available via the SEMO website for the Balancing Market can be organised into two groupings: market development and market data. The definition for those groupings, and a list of the known data publications for the Balancing Market, are shown below.

- Market Development – the process by which the market rules (and associated obligations) are agreed, modified and reported on via the Modifications Committee, along with the processes by which Disputes are handled and Regulatory Authority decisions published.

- Market Data – data and information published in relation to Balancing Market including registration, forecast data, commercial and technical offer data, imbalance price, and settlement information.

- Market Methodologies and Processes – informational reports on the methodologies and processes used by the market operator and TSO when conducting functions such as scheduling and dispatch, imbalance pricing calculations, flagging and tagging, and cross-zonal capacity calculations.

- Market Operator Performance – informational reports on the performance of the market operator when conducting key functions such as imbalance pricing, flagging and tagging, and scheduling and dispatch.

## Balancing Market Development

| ID | Document | Category | Format |
| --- | --- | --- | --- |
| BM-118 | Trading and Settlement Code (including Agreed Procedures) | Balancing Market Rules & Modifications | Doc / PDF |
| BM-119 | Schedule and dates of Modification Panel meetings | Balancing Market Rules & Modifications | Doc / PDF |
| BM-120 | Modification Proposal | Balancing Market Rules & Modifications | Doc / PDF |
| BM-121 | Public consultation on Modification Proposal | Balancing Market Rules & Modifications | Doc / PDF |
| BM-122 | Responses to Public Consultation on Modification Proposal | Balancing Market Rules & Modifications | Doc / PDF |
| BM-123 | Further information on Modification Proposal | Balancing Market Rules & Modifications | Doc / PDF |
| BM-124 | Final Recommendation Report | Balancing Market Rules & Modifications | Doc / PDF |
| BM-125 | Regulatory Authority decision on Final Modification Recommendation | Balancing Market Rules & Modifications | Doc / PDF |
| BM-126 | Members and Chairperson of the Modification Committee | Balancing Market Rules & Modifications | Doc / PDF |
| BM-127 | Terms of Reference for Market Operator Audit | Balancing Market Rules & Modifications | Doc / PDF |
| BM-128 | Audit Report | Balancing Market Rules & Modifications | Doc / PDF |

Table 10: Balancing Market: Market Development Publications

## Balancing Market Data

### Registration

| ID | Document | Category | Format |
| --- | --- | --- | --- |
| BM-001 | List of Registered Units | Registration | XML |
| BM-002 | List of Registered Parties | Registration | XML |
| BM-003 | Registered Capacity | Registration | XML |
| BM-004 | Making or lifting of a Suspension Order | Registration | Doc / PDF |
| BM-005 | Termination Order | Registration | Doc / PDF |
| BM-006 | Accession Fees and Participation Fees | Registration | Doc / PDF |
| BM-093 | Date of the next meeting of the Modifications Committee | Registration | Doc / PDF |
| BM-007 | Supplier Suspension Delay Period | Registration | Doc / PDF |
| BM-008 | Generator Suspension Delay Period | Registration | Doc / PDF |

Table 11: Balancing Market: Registration Publications

### Forecast Data

| Report ID | Document | Category | Format |
| --- | --- | --- | --- |
| BM-009 | Annual Load Forecast | Forecast Data | XML |
| BM-010 | Daily Load Forecast Summary | Forecast Data | XML |
| BM-011 | Monthly Load Forecast and Assumptions | Forecast Data | XML |
| BM-012 | Four Day Rolling Wind Power Unit Forecast | Forecast Data | XML |
| BM-013 | Four Day Aggregated Rolling Wind Unit Forecast | Forecast Data | XML |
| BM-014 | Forecast Imbalance | Forecast Data | XML |
| BM-015 | Net Imbalance Volume Forecast | Forecast Data | XML |
| BM-016 | Aggregated Wind Forecast | Forecast Data | XML |
| BM-018 | Daily Interconnector NTC | Forecast Data | XML |

Table 12: Balancing Market: Forecast Publications

### Inputs, Commercial and Technical Offer Data

| ID | Document | Category | Format |
| --- | --- | --- | --- |
| BM-019 | Annual CLAF | Inputs, Commercial and Technical Offer Data | XML |
| BM-020 | Aggregated Final Physical Notifications | Inputs, Commercial and Technical Offer Data | XML |
| BM-021 | Anonymised Inc/Dec Curves | Inputs, Commercial and Technical Offer Data | XML, CSV |
| BM-022 | System Shortfall Imbalance Index and System Imbalance Flattening Factor | Inputs, Commercial and Technical Offer Data | XML |
| BM-023 | Final Physical Notifications | Inputs, Commercial and Technical Offer Data | XML |
| BM-024 | Generator Unit Under Test Notice | Inputs, Commercial and Technical Offer Data | Doc / PDF |
| BM-025 | Imbalance Price Report (Imbalance Pricing Period) | Inputs, Commercial and Technical Offer Data | XML |
| BM-026 | Imbalance Price Report (Imbalance Settlement Period) | Inputs, Commercial and Technical Offer Data | XML |
| BM-164 | Imbalance Price Suporting Information Report | Inputs, Commercial and Technical Offer Data | XML |
| BM-027 | Information Imbalance Price | Inputs, Commercial and Technical Offer Data | Doc / PDF |
| BM-028 | Price Materiality Threshold | Inputs, Commercial and Technical Offer Data | Doc / PDF |
| BM-029 | Settlement Recalculation Threshold | Inputs, Commercial and Technical Offer Data | Doc / PDF |
| BM-030 | Imbalance Weighting Factor | Inputs, Commercial and Technical Offer Data | Doc / PDF |
| BM-031 | De Minimis Acceptance Threshold | Inputs, Commercial and Technical Offer Data | Doc / PDF |
| BM-032 | Technical Offer Data Accepted | Inputs, Commercial and Technical Offer Data | XML |
| BM-033 | Forecast Availability | Inputs, Commercial and Technical Offer Data | XML |
| BM-034 | Commercial Offer Data Accepted | Inputs, Commercial and Technical Offer Data | XML |
| BM-036 | Demand Control | Inputs, Commercial and Technical Offer Data | XML |
| BM-037 | Daily Dispatch Instructions (D+1) Reports | Inputs, Commercial and Technical Offer Data | XML |
| BM-038 | Daily Dispatch Instructions (D+4) Reports | Inputs, Commercial and Technical Offer Data | XML |
| BM-039 | Tracked Difference Payment Shortfall Amount | Inputs, Commercial and Technical Offer Data | Doc / PDF |
| BM-040 | Carbon Price | Inputs, Commercial and Technical Offer Data | Doc / PDF |
| BM-041 | Natural Gas Fuel Price | Inputs, Commercial and Technical Offer Data | Doc / PDF |
| BM-042 | Oil Fuel Price | Inputs, Commercial and Technical Offer Data | Doc / PDF |
| BM-043 | Peaking Unit Theoretical Efficiency | Inputs, Commercial and Technical Offer Data | Doc / PDF |
| BM-044 | Natural Gas Carbon Intensity Factor | Inputs, Commercial and Technical Offer Data | Doc / PDF |
| BM-045 | Oil Carbon Intensity Factor | Inputs, Commercial and Technical Offer Data | Doc / PDF |
| BM-046 | Aggregated Settlement Period | Inputs, Commercial and Technical Offer Data | Doc / PDF |
| BM-047 | Annual Stop-Loss Limit Factor | Inputs, Commercial and Technical Offer Data | Doc / PDF |
| BM-048 | Billing Period Stop-Loss Limit Factor | Inputs, Commercial and Technical Offer Data | Doc / PDF |
| BM-049 | Total Capacity Awarded | Inputs, Commercial and Technical Offer Data | Doc / PDF |
| BM-050 | Response Period Duration | Inputs, Commercial and Technical Offer Data | Doc / PDF |
| BM-051 | Annual Capacity Charge Exchange Rate | Inputs, Commercial and Technical Offer Data | Doc / PDF |
| BM-052 | Capacity Duration Exchange Rate | Inputs, Commercial and Technical Offer Data | Doc / PDF |
| BM-053 | Capacity Charge Metered Quantity Factor | Inputs, Commercial and Technical Offer Data | Doc / PDF |
| BM-054 | Supplier Capacity Charge Price | Inputs, Commercial and Technical Offer Data | Doc / PDF |
| BM-055 | Difference Payment Socialisation Multiplier | Inputs, Commercial and Technical Offer Data | Doc / PDF |
| BM-056 | Market Price Cap | Inputs, Commercial and Technical Offer Data | Doc / PDF |
| BM-057 | Market Price Floor | Inputs, Commercial and Technical Offer Data | Doc / PDF |
| BM-058 | Residual Meter Volume Interval Proportion | Inputs, Commercial and Technical Offer Data | Doc / PDF |
| BM-059 | Residual Error Volume Price | Inputs, Commercial and Technical Offer Data | Doc / PDF |
| BM-060 | Fixed Market Operator Charge (Supplier Unit) | Inputs, Commercial and Technical Offer Data | Doc / PDF |
| BM-061 | Fixed Market Operator Charge (Generator Unit) | Inputs, Commercial and Technical Offer Data | Doc / PDF |
| BM-062 | Variable Market Operator Price | Inputs, Commercial and Technical Offer Data | Doc / PDF |
| BM-063 | Engineering Tolerance | Inputs, Commercial and Technical Offer Data | Doc / PDF |
| BM-064 | MW Tolerance | Inputs, Commercial and Technical Offer Data | Doc / PDF |
| BM-065 | System per Unit Regulation Factor | Inputs, Commercial and Technical Offer Data | Doc / PDF |
| BM-066 | DOG/PUG/TOLOG/TOLUG For Over//Under Generation | Inputs, Commercial and Technical Offer Data | Doc / PDF |
| BM-067 | Transmission Loss Adjustment Factors | Inputs, Commercial and Technical Offer Data | Doc / PDF |
| BM-068 | Distribution Loss Adjustment Factors | Inputs, Commercial and Technical Offer Data | Doc / PDF |
| BM-069 | Combined Loss Adjustment Factors | Inputs, Commercial and Technical Offer Data | Doc / PDF |
| BM-070 | Imperfections Price | Inputs, Commercial and Technical Offer Data | Doc / PDF |
| BM-071 | Imperfections Charge Factor | Inputs, Commercial and Technical Offer Data | Doc / PDF |
| BM-072 | Testing Tariff | Inputs, Commercial and Technical Offer Data | Doc / PDF |
| BM-073 | Schedule of Testing Tariffs | Inputs, Commercial and Technical Offer Data | Doc / PDF |
| BM-074 | Fixed Credit Requirement, in respect of Generator and/or Supplier Units | Inputs, Commercial and Technical Offer Data | Doc / PDF |
| BM-075 | Days in Historical Assessment Period | Inputs, Commercial and Technical Offer Data | Doc / PDF |
| BM-076 | Analysis Percentile Parameter | Inputs, Commercial and Technical Offer Data | Doc / PDF |
| BM-077 | Credit Cover Adjustment Trigger | Inputs, Commercial and Technical Offer Data | Doc / PDF |
| BM-078 | Currency Cost Price | Inputs, Commercial and Technical Offer Data | Doc / PDF |
| BM-079 | Currency Cost Charge Factor | Inputs, Commercial and Technical Offer Data | Doc / PDF |
| BM-080 | Annual Maintenance Schedule - Transmission Line Outages | Inputs, Commercial and Technical Offer Data | XLS |
| BM-081 | Two Year Maintenance Schedule - Generator Outages Schedule | Inputs, Commercial and Technical Offer Data | XLS |
| BM-082 | Monthly Maintenance Schedule – Generator Unit outages | Inputs, Commercial and Technical Offer Data | XLS |
| BM-083 | Monthly Maintenance Schedule – Transmission System line outages | Inputs, Commercial and Technical Offer Data | XLS |
| BM-084 | Trading Day Exchange Rate between euro (€) and pounds sterling (£) | Inputs, Commercial and Technical Offer Data | XML |

Table 13: Balancing Market: Inputs, Commercial, and Technical Offer Data Publications

### Settlement Data

| ID | Document | Category | Format |
| --- | --- | --- | --- |
| BM-085 | Settlement Calendar | Settlement | XML |
| BM-086 | Daily Meter Data (Metered Generation by Unit) | Settlement | XML |
| BM-087 | Initial Interconnector Flows and Residual Capacity | Settlement | XML |
| BM-088 | Daily Transmission Outage Schedule | Settlement | XML |
| BM-089 | System Frequency Report | Settlement | XML |
| BM-090 | SO Interconnector Trades | Settlement | XML |
| BM-091 | Unit Data | Settlement | XML |
| BM-095 | Balancing and Imbalance Market Cost View | Settlement | XML |
| BM-096 | Dispatch Quantity | Settlement | XML |
| BM-098 | Aggregated Contracted Quantities for Generation | Settlement | XML |
| BM-099 | Aggregated Contracted Quantities for Demand | Settlement | XML |
| BM-100 | Aggregated Contracted Quantities for Wind | Settlement | XML |
| BM-101 | Average Outturn Availability | Settlement | XML |
| BM159 | Outturn Availability | Settlement | XML |
| BM-160 | Hourly Dispatch Instructions Report | Settlement | XML |
| BM-161 | Hourly SO Interconnector Trades Report | Settlement | XML |
| BM-102 | Unit Under Test | Settlement | XML |
| BM-103 | Energy Market Financial Publication – Indicative | Settlement | XML |
| BM-104 | Energy Market Financial Publication –Initial | Settlement | XML |
| BM-105 | Capacity Payments by Market | Settlement | XML |
| BM-106 | Capacity Payments by Unit | Settlement | XML |
| BM-094 | Metered Volumes by Jurisdiction | Settlement | XML |
| BM-107 | Generator Unit Technical Characteristics Data Transaction | Settlement | XML |
| BM-108 | Credit Assessment Price | Settlement | Doc / PDF |
| BM-109 | Billing Period Undefined Potential Exposure Quantity | Settlement | Doc / PDF |
| BM-110 | Trading Payments and Charges | Settlement | Doc / PDF |
| BM-111 | Capacity Quantity Scaling Factor | Settlement | Doc / PDF |
| BM-112 | Required Capacity Quantity | Settlement | Doc / PDF |
| BM-113 | Reserve Adjustment Capacity | Settlement | Doc / PDF |
| BM-114 | Net Capacity Quantity | Settlement | XML |
| BM-145 | Energy Market Information Publication – Indicative | Settlement | XML |
| BM-146 | Energy Market Information Publication – Initial | Settlement | XML |
| BM-147 | Capacity Market Information Publication – Indicative | Settlement | XML |
| BM-148 | Capacity Market Information Publication – Initial | Settlement | XML |
| BM-149 | Capacity Market Financial Publication – Indicative | Settlement | XML |
| BM-150 | Capacity Market Financial Publication – Initial | Settlement | XML |
| BM-151 | Metered Generation Information Publication - Indicative | Settlement | XML |
| BM-152 | Metered Generation Information Publication – Initial | Settlement | XML |
| BM-153 | Make Whole Payments | Settlement | XML |
| BM-154 | Cross Border Monthly Capacity Data | Settlement | XML |
| BM-155 | Cross Border Weekly Energy Data | Settlement | XML |

Table 14: Balancing Market: Settlement Data Publications

### Other

| ID | Document | Category | Format |
| --- | --- | --- | --- |
| BM-130 | Quarterly Renewable Dispatch Down (Constraint & Curtailment) | TSO Responsibilities | Doc / PDF |
| BM-131 | Annual Renewable Dispatch Down (Constraint & Curtailment) | TSO Responsibilities | Doc / PDF |
| BM-132 | Quarterly Constraint Cost Outturn | TSO Responsibilities | Doc / PDF |

Table 15: Balancing Market: Other Publications

### Operational Reports

| ID | Document | Category | Format |
| --- | --- | --- | --- |
| BM-115 | Proposed Market Operator Isolated Market System Testing Schedule | TSO Responsibilities | Doc / PDF |
| BM-137 | Monthly Constraints Update | TSO Responsibilities | Doc / PDF |
| BM-138 | Weekly Constraints Update | TSO Responsibilities | Doc / PDF |
| BM-139 | Ad-hoc Constraints Update | TSO Responsibilities | Doc / PDF |
| BM-162 | LTS Operational Schedule Report, Member Public | TSO Responsibilities | XML |
| BM-163 | RTIC Operational Schedule Report, Member Public | TSO Responsibilities | XML |

Table 16: Balancing Market: Operational Reports

## Market Methodologies and Processes

### Methodologies

| ID | Document | Category | Format |
| --- | --- | --- | --- |
| BM-116 | Calculations and methodology used by the Market Operator during Administered Settlement | TSO Responsibilities | Doc / PDF |
| BM-129 | Interim Cross-Zonal Capacity Calculation | TSO Responsibilities | Doc / PDF |
| BM-136 | Balancing Market Principles Statement | TSO Responsibilities | Doc / PDF |
| BM-157 | Determination of Scheduling and Dispatch Policy Parameters | TSO Responsibilities | Doc / PDF |
| BM-158 | System Operator and Non-Marginal Flagging | TSO Responsibilities | Doc / PDF |

Table 17: Balancing Market: Methodologies

### Operational processes

| ID | Document | Category | Format |
| --- | --- | --- | --- |
| BM-140 | Demand Forecasting | TSO Responsibilities | Doc / PDF |
| BM-141 | Scheduling | TSO Responsibilities | Doc / PDF |
| BM-142 | Dispatch | TSO Responsibilities | Doc / PDF |
| BM-143 | Cross-Zonal Actions | TSO Responsibilities | Doc / PDF |
| BM-144 | Wind Farm Controllability Categorisation | TSO Responsibilities | Doc / PDF |

Table 18: Balancing Market: Operational Processes

## Market Operator Performance

### Market Operator Performance

| ID | Document | Category | Format |
| --- | --- | --- | --- |
| BM-117 | Market Operator Performance Report | MO Performance | Doc / PDF |
| BM-133 | Annual All-Island Transmission System Performance Report | MO Performance | Doc / PDF |
| BM-134 | Scheduling and Dispatch Policy Parameter Performance Report | MO Performance | Doc / PDF |
| BM-135 | Annual Scheduling and Dispatch Process Audit Report | MO Performance | Doc / PDF |

Table 19: Balancing Market: Market Methodologies and Processes

# Appendix A: Capacity Market Publication Details

This appendix includes detailed information on the reports and their data from the Capacity Market.

## Capacity Market Development

### CM-001: Capacity Market Code (Including Agreed procedures)

This document contains the Capacity Market Code including Agreed Procedures.

| I-SEM Report Reference: | CM-001 |
| --- | --- |
| Audience: | General Public |
| Frequency: | No less frequently than twice yearly in line with the Scheduled Release. |
| Report Format: | Word/PDF |

### CM-002: Effective Modifications Not Yet incorporated Into The Capacity Market Code

The report includes modifications that have been approved but not incorporated into the Capacity Market Code.

| I-SEM Report Reference: | CM-002 |
| --- | --- |
| Audience: | General Public |
| Frequency: | No less frequently than twice yearly in line with the Scheduled Release. |
| Report Format: | Word/PDF |

### CM-003: Modification Finalisation Date for a Capacity Auction

This report contains the date (“Modification Finalisation Date”) determined by the Regulatory Authorities by which Modifications Proposals should generally be made in order for them to have effect in respect of a Capacity Auction.

| I-SEM Report Reference: | CM-003 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Within two Working Day of receiving notice of determination by the Regulatory Authorities. |
| Report Format: | Word/PDF |

### CM-004: Modification Proposal and Associated Status

This report contains details pertaining to individual modification proposals and their associated status.

| I-SEM Report Reference: | CM-004 |
| --- | --- |
| Audience: | General Public |
| Frequency: | As required. |
| Report Format: | Word/PDF |

### CM-005: Regulatory Authority Notices

This report contains notices provided to the Systems operator by the Regulatory Authorities.

| I-SEM Report Reference: | CM-005 |
| --- | --- |
| Audience: | General Public |
| Frequency: | as soon as reasonably practicable, but not later than five Working Days of its receipt from the Regulatory Authorities. |
| Report Format: | Word/PDF |

### CM-006: Quarterly Report on status of Modification Proposals

This report contains the collated status of modification proposals.

| I-SEM Report Reference: | CM-006 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Quarterly, with a report not less than one month prior to the Modification Finalisation Date, if such a date has been set by the Regulatory Authorities |
| Report Format: | Word/PDF |

### CM-007: Decision on Modification Proposal

This report contains the Regulatory Authorities decision on an individual modification proposal.

| I-SEM Report Reference: | CM-007 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Within two working Days of the Regulatory Authorities providing a Modification Proposal decision to the System Operators |
| Report Format: | Word/PDF |

### CM-008: Modification proposal Form

This document contains the updated Modifications Proposal form.

| I-SEM Report Reference: | CM-008 |
| --- | --- |
| Audience: | General Public |
| Frequency: | As required. |
| Report Format: | Word/PDF |

### CM-009: Capacity Auction Monitor, Terms of Reference

This report details the terms of reference used by the Capacity Auction Monitor in monitoring the processes and procedures followed by the System Operators in carrying out the Qualification Process and conducting Capacity Auctions.

| I-SEM Report Reference: | CM-009 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Within one Working Day of its receipt from the Regulatory Authorities to the extent directed by the Regulatory Authorities |
| Report Format: | Word/PDF |

### CM-010: Capacity Auction Audit report (non-Confidential)

This report contains the non-confidential version of the Capacity Auction Audit Report.

| I-SEM Report Reference: | CM-010 |
| --- | --- |
| Audience: | General Public |
| Frequency: | This report contains the non-confidential version of the Capacity Auction Audit Report. |
| Report Format: | Word/PDF |

### CM-011: Dispute Process Timetable

This report contains details of the dispute process timetable.

| I-SEM Report Reference: | CM-011 |
| --- | --- |
| Audience: | General Public |
| Frequency: | As required |
| Report Format: | Word/PDF |

### CM-012: Notice of Dispute Form

This document contains the updated form of Notice of Dispute.

| I-SEM Report Reference: | CM-011 |
| --- | --- |
| Audience: | General Public |
| Frequency: | As required. |
| Report Format: | Word/PDF |

### CM-013: Panel Chair, Panel Members and Associated CVs

This report contains the names of members and chairperson of the panel, and the name and curriculum of the panel members.

| I-SEM Report Reference: | CM-013 |
| --- | --- |
| Audience: | General Public |
| Frequency: | As updated and at least within two working days. |
| Report Format: | Word/PDF |

## Capacity Market Data

### CM-014: List of Capacity Market Code Parties and Participants

This report contains the list of registered Capacity Market Code Parties and Participants.

| I-SEM Report Reference: | CM-014 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Updated at least within five Working Days of a successful Application for Registration or Deregistration. |
| Report Format: | Word/PDF |

### CM-015: Notice of Making or Lifting a Suspension Order

This report contains the notice of making or lifting a suspension for a participant.

| I-SEM Report Reference: | CM-015 |
| --- | --- |
| Audience: | General Public |
| Frequency: | It will be published as soon as reasonably practicable after being issued and at least within two Working Days of issue. |
| Report Format: | Word/PDF |

### CM-016: Termination Order

This report contains a notice of termination for a Party that specifies the time and date from which the Termination or Deregistration will take effect and the terms of the Termination or Deregistration.

| I-SEM Report Reference: | CM-016 |
| --- | --- |
| Audience: | General Public |
| Frequency: | It will be published as soon as reasonably practicable after being issued and at least within two Working Days of issue. |
| Report Format: | Word/PDF |

### CM-017: Accession Fees and Participation Fees

This report contains details of the approved rate and basis for Accession Fees and Participation Fees.

| I-SEM Report Reference: | CM-017 |
| --- | --- |
| Audience: | General Public |
| Frequency: | As updated and at least within two Working Days of update |
| Report Format: | Word/PDF |

### CM-018: Qualification Charge, Fixed System Operator Charge and Variable System Operator Charge

This report contains details of the approved rate and basis for Qualification Charge, Fixed System Operator Charge and Variable System Operator Charge.

| I-SEM Report Reference: | CM-018 |
| --- | --- |
| Audience: | General Public |
| Frequency: | As updated and at least within two Working Days of update |
| Report Format: | Word/PDF |

### CM-019: Market Qualification report

This report contains results from Capacity Market qualification processing.

| I-SEM Report Reference: | CM-019 |
| --- | --- |
| Periodicity: | Per Primary Auction |
| Report Name: | Market Qualification Report |
| File Names: | Provisional Qualification Report: MQR_P_[AUCTION_NAME]_[DOWNLOAD_DATE]_[DOWNLOAD_TIME] Final Auction Report: MQR_F_[AUCTION_NAME]_[DOWNLOAD_DATE]_[DOWNLOAD_TIME] Where: DOWNLOAD_DATE format is: dd-mm-yyyy DOWNLOAD_TIME format is: 24hh-mm-ss |
| Report Title: | Market Qualification Report |
| Audience: | General Public |
| Resolution: | Primary Auction |
| Time Span: | Capacity Period |
| Frequency: | Per Primary Auction |
| Report Format: | CSV (delimited with semicolon) |

Note: the following information is captured in the file name:

- Primary Auction Name

- Report Type (Provisional, Final)

| Field Name | Format | Description |
| --- | --- | --- |
| TECHNOLOGY CLASS ID | Text | Identifies the Technology class of the Generator Unit. |
| New Total Gross De-rated Capacity | NUMBER(8,3) | The Total Qualified Gross De-rated Capacity for this Capacity Type. |
| New Total Awarded Capacity | NUMBER(8,3) | The Total Awarded Capacity for this Capacity Type |
| New Total Net De-rated Capacity | NUMBER(8,3) | The Total Net De-rated Capacity for this Capacity Type |
| Existing Total Gross De-rated Capacity | NUMBER(8,3) | The Existing Qualified Gross De-rated Capacity for this Capacity Type. |
| Existing Total Awarded Capacity | NUMBER(8,3) | The Existing Awarded Capacity for this Capacity Type |
| Existing Total Net De-rated Capacity | NUMBER(8,3) | The Existing Net De-rated Capacity for this Capacity Type |
| Generator Unit Type | Text | The Generator Unit type classification. |
| Gross New De-rated Capacity | NUMBER(8,3) | The Gross New De-rated Capacity for this Capacity Type |
| Gross Existing De-rated Capacity | NUMBER(8,3) | The Gross Existing De-rated Capacity for this Capacity Type |

Table 20: Market Qualification Report

### CM-020: Reporting Schedule for Awarded New Capacity

This report contains the reporting schedule for Awarded New Capacity in respect of a new or refurbished Generator Unit or Interconnector.

| I-SEM Report Reference: | CM-020 |
| --- | --- |
| Audience: | General Public |
| Frequency: | As required |
| Report Format: | Word/PDF |

### CM-021: Implementation Progress Reports Form

This report contains the Implementation Reports Form which a Participant with Awarded New Capacity shall submit to the System Operators at each of the times specified in the applicable reporting schedule.

| I-SEM Report Reference: | CM-021 |
| --- | --- |
| Audience: | General Public |
| Frequency: | As required |
| Report Format: | Word/PDF |

### CM-022: Notice of Termination of Awarded Capacity

This report contains the Notice of Termination of Awarded Capacity issued to a Participant.

| I-SEM Report Reference: | CM-022 |
| --- | --- |
| Audience: | General Public |
| Frequency: | As required |
| Report Format: | Word/PDF |

### CM-023: Capacity Auction Timetable

This report contains the Capacity Auction Timetable, including updates and revocation.

| I-SEM Report Reference: | CM-023 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Within 2 Working Days of it being approved, determined, amended or revoked. |
| Report Format: | Word/PDF |

### CM-024: Capacity Auction Information Pack

This report contains information for Market Participants in advance of a Capacity Auction.

| I-SEM Report Reference: | CM-024 |
| --- | --- |
| Audience: | General Public |
| Frequency: | In advance of any and all Capacity Auctions. |
| Report Format: | Word/PDF |

### CM-025: Cancellation of capacity Auction

This report contains notice of cancellation of Capacity Auction.

| I-SEM Report Reference: | CM-025 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Five Working Days Prior to a Capacity Auction Submission Commencement date if the Regulatory Authorities have not provided a Demand Curve to the System Operators |
| Report Format: | Word/PDF |

### CM-027: Capacity Auction results

This report contains results from each Capacity Auction.

| I-SEM Report Reference: | CM-027 |  |
| --- | --- | --- |
| Data Source | CMP |  |
| Periodicity: | Per Capacity Auction |  |
| Report Name: | Market Auction Results |  |
| File Names: | Provisional Auction Results: MAR_P_[AUCTION_NAME]_[DOWNLOAD_DATE]_DOWNLOAD_TIME] Final Auction Results: MAR_F_[AUCTION_NAME]_[DOWNLOAD_DATE]_DOWNLOAD_TIME] Where: DOWNLOAD_DATE format is: dd-mm-yyyy DOWNLOAD_TIME format is: 24hh-mm-ss |  |
| Report Title: | Capacity Auction Results |  |
| Audience: | General Public |  |
| Resolution: | Capacity Market Auction |  |
| Time Span: | Auction time horizon |  |
| Frequency: | Per Capacity Auction |  |
| Report Format: | CSV (delimited with semicolon) |  |

Note: the following information is captured in the file name:

- Primary Auction ID

- Report type

| Field Name | Format | Description |
| --- | --- | --- |
| Delivery Period Start | DateTime YYYY-MM-DDTHH:MM:SSZ | The start of the delivery period. |
| Delivery Period End | DateTime YYYY-MM-DDTHH:MM:SSZ | The end of the delivery period. |
| Existing Capacity Price Cap | NUMBER(8,2) | Default Capacity Market Auction Offer Price Cap applicable to existing capacity. |
| Auction Price Cap | NUMBER(8,2) | Default Capacity Market Auction Offer Price Cap applicable to New Capacity |
| Annual Capacity Exchange Rate | NUMBER(5,4) | This will be used to convert prices from Euro to Sterling and vice-versa. |
| Auction Clearing Price in Euro | NUMBER(8,2) | Auction Clearing Price in Euro |
| Auction Clearing Price in Sterling | NUMBER(8,2) | Auction Clearing Price in Sterling. This could be higher than the clearing price due to the award of capacity to out of merit offers as a result of auction constraints |
| Average Price of Awarded Capacity in Euro | NUMBER(8,2) | Average Price of Awarded Capacity in Euro |
| Average Price of Awarded Capacity in Sterling | NUMBER(8,2) | Average Price of Awarded Capacity in Sterling |
| Average Duration of Awarded Capacity | NUMBER(8,2) | Average Duration of Awarded Capacity |
| Number of participating CMUs | NUMBER(4,0) | Number of CMUs that submitted an offer with quantity greater than zero, including Participants of CMU, where CMO offer was inserted on their behalf. |
| Number of successful CMUs | NUMBER(4,0) | Number of CMUs awarded greater than zero capacity |
| Total Social Welfare | NUMBER(8,2) | This is the sum of social welfares for each successful offer where social welfare is calculated as per the Primary Auction Algorithm |
| Total Capacity Offered | NUMBER(8,3) | The total quantity offered into the auction |
| Total Capacity Awarded | NUMBER(8,3) | The total quantity awarded in the auction |

Table 21: Capacity Auction Results Main Section

### CM-028: Amended Capacity Auction timetable Following General Systems Failure

This report contains the amended Capacity Auction Timetable following general systems failure.

| I-SEM Report Reference: | CM-028 |
| --- | --- |
| Audience: | General Public |
| Frequency: | As required |
| Report Format: | Word/PDF |

### CM-029: Updated Annual Capacity Payment Exchange Rate and Monthly Capacity Payment Exchange Rates

This report contains the updated Annual Capacity Payment Exchange Rate and Monthly Capacity Payment Exchange Rate.

| I-SEM Report Reference: | CM-029 |
| --- | --- |
| Audience: | General Public |
| Frequency: | As required |
| Report Format: | Word/PDF |

### CM-030: Capacity Auction results – Constraints Data

This report contains constraints data from each Capacity Auction.

| I-SEM Report Reference: | CM-030 |  |
| --- | --- | --- |
| Data Source | CMP |  |
| Periodicity: | Per Primary Auction |  |
| Report Name: | Capacity Auction Results |  |
| File Names: | Provisional Auction Results Constraints Data: MAR_P_[AUCTION_NAME]_[DOWNLOAD_DATE]_[DOWNLOAD_TIME]_Constraints Final Auction Results Constraints Data: MAR_F_[AUCTION_NAME]_[DOWNLOAD_DATE]_[DOWNLOAD_TIME]_Constraints Where: DOWNLOAD_DATE format is: dd-mm-yyyy DOWNLOAD_TIME format is: hh24-mi-ss |  |
| Report Title: | Capacity Auction Results |  |
| Audience: | General Public |  |
| Resolution: | Capacity Market Auction |  |
| Time Span: | Auction time horizon |  |
| Frequency: | Per Primary Auction |  |
| Report Format: | CSV (delimited with semicolon) |  |

Note: the following information is captured in the file name:

- Primary Auction ID

- Report type

| Field Name | Format | Description |
| --- | --- | --- |
| Constraint ID | Text | Unique Description of Constraint |
| Limit (MW) | NUMBER(8,3) | Constraint Limit in MW. |
| Awarded Capacity (MW) | NUMBER(8,3) | Awarded capacity of all CMU related to selected constraint in MW. |
| Satisfied | Text | Flag that shows whether constraint was satisfied for current auction results. Possible values: “Yes”, “No” |

Table 22: Capacity Auction Results Footer Section

# Appendix B: Balancing Market Details

## Balancing Market Development

### BM-118: Trading and Settlement Code (including Agreed Procedures)

This report contains the Trading and Settlement Code, including Agreed Procedures.

| I-SEM Report Reference: | BM-118 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Periodically as required |
| Report Format: | Word/PDF |

### BM-119: Schedule and dates of Modification Panel meetings

This report contains the planned schedule and dates of the Modification Panel meetings.

| I-SEM Report Reference: | BM-119 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Periodically as required |
| Report Format: | Word/PDF |

### BM-120: Modification Proposal

This report contains the details of a submitted Modification Proposal which has been accepted for consideration by the Modification Panel Committee.

| I-SEM Report Reference: | BM-120 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Periodically as required |
| Report Format: | Word/PDF |

### BM-121: Public Consultation on Modification Proposal

This report contains the request from the Modification Committee Secretariat for the public’s views on a particular Modification Proposal.

| I-SEM Report Reference: | BM-121 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Periodically as required |
| Report Format: | Word/PDF |

### BM-122: Responses to Public Consultation on Modification Proposal

This report contains the collated responses to the Consultation paper issued by the Modification Committee Secretariat for the public’s views on a particular Modification Proposal.

| I-SEM Report Reference: | BM-122 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Periodically as required |
| Report Format: | Word/PDF |

### BM-123: Further information on Modification Proposal

This report contains any further relevant information received by the Modifications Committee in relation to a particular Modification Proposal.

| I-SEM Report Reference: | BM-123 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Periodically as required |
| Report Format: | Word/PDF |

### BM-124: Final Recommendation Report

This report contains the Final Recommendation Report of the Modifications Committee on a particular Modification Proposal.

| I-SEM Report Reference: | BM-124 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Periodically as required |
| Report Format: | Word/PDF |

### BM-125: Regulatory Authority decision on Final Modification Recommendation

This report contains the Regulatory Authority decision on the Final Modification Recommendation of a Particular Modification Proposal.

| I-SEM Report Reference: | BM-125 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Periodically as required |
| Report Format: | Word/PDF |

### BM-126: Members and Chairperson of the Modification Committee

This report contains the names of the members and chairperson of the Modification Committee.

| I-SEM Report Reference: | BM-126 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Periodically as required |
| Report Format: | Word/PDF |

### BM-127: Terms of Reference for Market Operator Audit

This report contains the terms of reference by which the nominated Market Operator Auditor conducts an audit of the Code, its operation and implementation and the operations, trading arrangements, procedures and processes under the Code.

| I-SEM Report Reference: | BM-127 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Periodically as required |
| Report Format: | Word/PDF |

### BM-128: Audit Report

This report contains the Market Operator Auditor’s findings in the audit of the Code, its operation and implementation and the operations, trading arrangements, procedures and processes under the Code.

| I-SEM Report Reference: | BM-128 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Periodically as required |
| Report Format: | Word/PDF |

## Balancing Market Data

### BM-001: List of Registered Units Report

This report contains all Units registered to participate in the I-SEM.

| I-SEM Report Reference: | BM-001 |
| --- | --- |
| Data Source | Registration |
| Periodicity: | Daily |
| Report Name: | PUB_DailyRegisteredUnits |
| File Names: | PUB_DailyRegisteredUnits.xml |
| Report Title: | List of Registered Units Report |
| Audience: | General Public |
| Resolution: | n/a |
| Time Span: | Currently Active/Approved Units, as registered at the time of publication |
| Frequency: | Daily, only when a new Unit Registration is approved. |
| Report Format: | XML |

| Field Name | Format | Description |
| --- | --- | --- |
| PUBLISH TIME | TIME(YYYY-MM-DDTH24:MI) | Publish Date/time |
| PARTY ID | VARCHAR2(12) | Party ID (PY_xxxxxx) |
| PARTY NAME | VARCHAR2(60) | Company name |
| ASSOCIATED PARTICIPANT ID | VARCHAR2(12) | Participant ID (PT_xxxxxx) |
| ASSOCIATED PARTICIPANT NAME | VARCHAR2(60) | Participant name |
| TRADING SITE ID | VARCHAR2(12) | TS_xxxxxx |
| UNIT ID | VARCHAR2(32) | e.g. GU_xxxxxx |
| UNIT NAME | VARCHAR2(32) | Unit name |
| UNIT TYPE | VARCHAR2(4) | Unit Type as per Registration (e.g. GU, SU, TU, etc.) |
| INTERMEDIARY | CHAR(1) | Y/N flag |
| AOLR REGISTERED | CHAR(1) | Y/N flag |
| STATUS | VARCHAR2(16) | Registered (does not have de-registration date) De-Registered (has de-registration date) |
| REGISTRATION DATE | YYYY-MM-DD | Trading Day when registered |
| DEREGISTRATION DATE | YYYY-MM-DD | Trading Day when deregistered |

Table 23: List of Registered Units Report

### BM-002: List of Registered Parties Report

This report contains details of all Parties that are registered in the I-SEM.

| I-SEM Report Reference: | BM-002 |
| --- | --- |
| Data Source | Registration |
| Periodicity: | Daily |
| Report Name: | PUB_DailyRegisteredParties |
| File Names: | PUB_DailyRegisteredParties.xml |
| Report Title: | List of Registered Parties Report |
| Audience: | General Public |
| Resolution: | n/a |
| Time Span: | Currently Active/Approved Parties, as registered at the time of publication |
| Frequency: | Daily, only when a new Party Registration is approved. |
| Report Format: | XML |

| Field Name | Format | Description |
| --- | --- | --- |
| PUBLISH TIME | TIME(YYYY-MM-DDTH24:MI) | Publish Date/time |
| Short Name | VARCHAR2(12) | PY_xxxxxx |
| Company Name | VARCHAR2(60) | Company Name |
| Status | VARCHAR2(24) | Registered Party Terminated (Voluntarily) Terminated |
| Registration Date | YYYY-MM-DD | Trading Day when registered |
| Termination Date | YYYY-MM-DD | Trading Day when deregistered |

Table 24: List of Registered Parties Report

### BM-003: Registered Capacity Report

This report contains the Registered Capacity and Fuel Type information for all Generator and Demand Side Units currently registered in the I-SEM.

| I-SEM Report Reference: | BM-003 |
| --- | --- |
| Periodicity: | Monthly |
| Report Name: | PUB_MnlyRegisteredCapacity |
| File Names: | PUB_MnlyRegisteredCapacity.xml |
| Report Title: | Registered Capacity Report |
| Audience: | General Public |
| Resolution: | n/a |
| Time Span: | At the time of publication |
| Frequency: | Once every month |
| Report Format: | XML |

| Field Name | Format | Description |
| --- | --- | --- |
| PUBLISH TIME | TIME(YYYY-MM-DDTH24:MI) | Publish Date/time |
| Participant Id | VARCHAR2(12) | e.g. PT_xxxxxx |
| Participant Name | VARCHAR2(60) | Participant Name |
| RESOURCE NAME | VARCHAR2(32) | Resource identifier |
| Unit Name | VARCHAR2(32) | Unit Name |
| Registered Capacity | NUMBER(8,3) | Registered Capacity or MW |
| Dual Fuel Flag | CHAR(1) | Y/N |
| Fuel Type | VARCHAR2(22) | WIND, GAS, MULTI, HYDRO, PEAT, COAL, PUMP, BIO, GAS, OIL, DISTL, DEM(Demand) etc.. |
| Secondary Fuel Type | VARCHAR2(22) | Valid value from Fuel Type. |
| Jurisdiction | VARCHAR2(32) | ROI/NI |

Table 25: Registered Capacity Report

### BM-004: Making or Lifting of a Suspension Order

This report contains the notice of making or lifting a suspension for a participant.

| I-SEM Report Reference: | BM-004 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Periodically as required |
| Report Format: | Word/PDF |

### BM-005: Termination Order

This report contains a notice of termination for a Party that specifies the time and date from which the Termination or Deregistration will take effect and the terms of the Termination or Deregistration.

| I-SEM Report Reference: | BM-005 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Periodically as required |
| Report Format: | Word/PDF |

### BM-006: Accession Fees and participation Fees

This report contains details of the approved rate and basis for Accession Fees and Participation Fees.

| I-SEM Report Reference: | BM-006 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Periodically as required |
| Report Format: | Word/PDF |

### BM-007: Supplier Suspension Delay Period

This report contains the minimum period before a Suspension Order may take effect in respect of any Supplier Unit specified in the Suspension Order issued by the Market Operator.

| I-SEM Report Reference: | BM-007 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Periodically as required |
| Report Format: | Word/PDF |

### BM-008: Generator Suspension Delay period

This report contains the minimum period before a Suspension Order may take effect in respect of any Generator Unit specified in the Suspension Order issued by the Market Operator.

| I-SEM Report Reference: | BM-008 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Periodically as required |
| Report Format: | Word/PDF |

### BM-009: Annual Load Forecast Report

This report contains the load forecast per jurisdiction (ROI & NI) and an aggregated value for both Jurisdictions, for each Trading Period in the relevant year.

| I-SEM Report Reference: | BM-009 |
| --- | --- |
| Periodicity: | Annual |
| Report Name: | PUB_AnnualLoadFcst |
| File Names: | PUB_AnnualLoadFcst.xml |
| Report Title: | Annual Load Forecast Report |
| Audience: | General Public |
| Resolution: | Trading Period |
| Time Span: | Following year |
| Frequency: | Once every year, in August |
| Report Format: | XML |

| Field Name | Format | Description |
| --- | --- | --- |
| DELIVERY DATE | DATE (YYYY-MM-DD) | Calendar Day (referred to as “Day” in the Code). |
| START TIME | TIME(YYYY-MM-DDTH24:MI) | Start time of the 30 minute Trading Period |
| END TIME | TIME(YYYY-MM-DDTH24:MI) | End time of the 30 minute Trading Period |
| LOAD FORECAST ROI | NUMBER(10) | Load Forecast for ROI (MW) |
| LOAD FORECAST NI | NUMBER(10) | Load Forecast for NI (MW) |
| AGGREGATED FORECAST | NUMBER(10) | Aggregated Load Forecast for both Jurisdictions. |

Table 26: Annual Load Forecast Report

### BM-010: Daily Load Forecast Summary Report

This report contains the load forecast by Jurisdiction (ROI/NI) for each Trading Period in the following four Trading Days.

| I-SEM Report Reference: | BM-010 |
| --- | --- |
| Periodicity: | Daily |
| Report Name: | PUB_DailyLoadFcst |
| File Names: | PUB_DailyLoadFcst_YYYYMMDDHHMM.xml |
| Report Title: | Daily Load Forecast Summary Report |
| Audience: | General Public |
| Resolution: | Imbalance Settlement Period |
| Time Span: | Four calendar days |
| Frequency: | Following receipt of each Four Day Load Forecast |
| Report Format: | XML |

| Field Name | Format | Description |
| --- | --- | --- |
| TRADE DATE | DATE (YYYY-MM-DD) | Trading Day |
| START TIME | TIME(YYYY-MM-DDTH24:MI) | Start time of the 30 minute Trading Period |
| END TIME | TIME(YYYY-MM-DDTH24:MI) | End time of the 30 minute Trading Period |
| DELIVERY DATE | DATE (YYYY-MM-DD) | Calendar Day (referred to as “Day” in the Code). |
| LOAD FORECAST ROI | NUMBER(10) | Load Forecast for ROI (MW) |
| LOAD FORECAST NI | NUMBER(10) | Load Forecast for NI (MW) |
| AGGREGATED FORECAST | NUMBER(10) | Aggregated Load Forecast for both Jurisdictions. |

Table 27: Daily Load Forecast Summary Report

### BM-011: Monthly Load Forecast and Assumptions Report

This report contains the load forecast per jurisdiction (ROI & NI), an aggregated value for both Jurisdictions and assumptions, for each Trading Period in the following calendar month.

| I-SEM Report Reference: | BM-011 |
| --- | --- |
| Periodicity: | Monthly |
| Report Name: | PUB_MonthlyLoadFcst |
| File Names: | PUB_MonthlyLoadFcst.xml |
| Report Title: | Monthly Load Forecast and Aggregated Load Forecast and Assumptions Report |
| Audience: | General Public |
| Resolution: | Trading Period |
| Time Span: | Following month |
| Frequency: | Once every Month |
| Report Format: | XML |

| Field Name | Format | Description |
| --- | --- | --- |
| TRADE DATE | DATE (YYYY-MM-DD) | Trading Day |
| DELIVERY DATE | DATE (YYYY-MM-DD) | Calendar Day (referred to as “Day” in the Code). |
| START TIME | TIME(YYYY-MM-DDTH24:MI) | Start time of the 30 minute Trading Period |
| END TIME | TIME(YYYY-MM-DDTH24:MI) | End time of the 30 minute Trading Period |
| LOAD FORECAST ROI | NUMBER(10) | Load Forecast for ROI (MW) |
| LOAD FORECAST NI | NUMBER(10) | Load Forecast for NI (MW) |
| AGGREGATED FORECAST | NUMBER(10) | Aggregated Load Forecast for both Jurisdictions. |

Table 28: Monthly Load Forecast and Assumptions Report

### BM-012: Four Day Rolling Wind Unit Forecast Report

This report is produced four times per day and contains the expected output (MW) for each Wind Unit per 15 mins in the next four Trading Days, produced following the receipt of each accepted Wind Power Unit Forecast.

| I-SEM Report Reference: | BM-012 |
| --- | --- |
| Periodicity: | On_demand |
| Data Source: | TSO (WEF) |
| Report Name: | PUB_4DayRollWindUnitFcst |
| File Names: | PUB_4DayRollWindUnitFcst_YYYYMMDDHHMM.xml |
| Report Title: | Four Day Rolling Wind Unit Forecast Report |
| Audience: | General Public |
| Resolution: | 15 minutes |
| Time Span: | Next four Trading Days |
| Frequency: | Four times per day, following receipt of each Wind Unit Forecast |
| Report Format: | XML |

| Field Name | Format | Description |
| --- | --- | --- |
| TRADE DATE | DATE (YYYY-MM-DD) | Trading Day |
| START TIME | TIME(YYYY-MM-DDTH24:MI) | Start date/time of forecast data |
| END TIME | TIME(YYYY-MM-DDTH24:MI) | End date/time of forecast data |
| DELIVERY DATE | DATE (YYYY-MM-DD) | Calendar Day (referred to as “Day” in the Code). |
| Participant Name | VARCHAR2(12) | Participant Name |
| RESOURCE NAME | VARCHAR2(36) | Resource Name |
| JURISDICTION | VARCHAR2(5) | ROI (Republic of Ireland) or NI (Northern Ireland) |
| FORECAST MW | NUMBER(8,3) | Forecasted MW |

Table 29: Four Day Rolling Wind Unit Forecast Report

### BM-013: Four Day Aggregated Rolling Wind Unit Forecast Report

This report is run four times per day, containing the aggregated output (MW) across all Wind Units in each jurisdiction, for the next four Trading Days, produced following the receipt of each Wind Unit Forecast.

| I-SEM Report Reference: | BM-013 |
| --- | --- |
| Periodicity: | On_demand |
| Report Name: | PUB_4DayAggRollWindUnitFcst |
| File Names: | PUB_4DayAggRollWindUnitFcst_YYYYMMDDHHMM.xml |
| Report Title: | Four Day Aggregated Rolling Wind Unit Forecast Report |
| Audience: | General Public |
| Resolution: | 15 mins |
| Time Span: | Next four Trading Days |
| Frequency: | Four times per day |
| Report Format: | XML |

| Field Name | Format | Description |
| --- | --- | --- |
| TRADE DATE | DATE (YYYY-MM-DD) | Trading Day |
| START TIME | TIME(YYYY-MM-DDTH24:MI) | Start date/time of forecast data |
| END TIME | TIME(YYYY-MM-DDTH24:MI) | End date/time of forecast data |
| DELIVERY DATE | DATE (YYYY-MM-DD) | Calendar Day (referred to as “Day” in the Code). |
| LOAD FORECAST ROI | NUMBER(11,3) | Load Forecast for ROI (MW) |
| LOAD FORECAST NI | NUMBER(11,3) | Load Forecast for NI (MW) |
| AGGREGATED FORECAST | NUMBER(11,3) | Aggregated Load Forecast for both Jurisdictions. |

Table 30: Four Day Aggregated Rolling Wind Unit Forecast Report

### BM-014: Forecast Imbalance Report

This report details the sum of all the Physical Notifications which are submitted by Participants, the net Interconnector schedule, the TSO renewables forecast for non-Dispatchable renewable Units and the calculated imbalance, for each Imbalance Settlement Period until the end of the latest Trading Day for which the Day Ahead Market gate is closed.

| I-SEM Report Reference: | BM-014 |
| --- | --- |
| Data Source | Balancing Market Runs |
| Periodicity: | Hourly |
| Report Name: | PUB_HrlyForecastImbalance |
| File Names: | PUB_HrlyForecastImbalance_YYYYMMDDHHMM.xml |
| Report Title: | Forecast Imbalance Report |
| Audience: | General Public |
| Resolution: | Imbalance Settlement Period |
| Time Span: | Starting from the Hour Ahead of Imbalance Settlement Period until the end of the latest Trading Day for which the Day Ahead Market gate is closed with a value for each Imbalance Pricing Period (e.g. 5 mins).. |
| Frequency: | Every Hour |
| Report Format: | XML |

| Field Name | Format | Description |
| --- | --- | --- |
| TRADE DATE | DATE (YYYY-MM-DD) | Trading Day |
| START TIME | TIME(YYYY-MM-DDTH24:MI) | Settlement Imbalance Period Start Time |
| END TIME | TIME(YYYY-MM-DDTH24:MI) | Settlement Imbalance Period End Time |
| CALCULATED IMBALANCE | NUMBER(8,3) | Resulting calculated imbalance for the next Imbalance Settlement Period until the end of the latest Trading Day for which the Day Ahead Market has closed, calculated as: TSO DEMAND FORECAST – [TOTAL PN + NET INTERCONNECTOR SCHEDULE + TSO RENEWABLE FORECAST (PRIORITY DISPATCH)] Note: TSO RENEWABLE FORECAST (NPDR) is not included in the CALCULATED IMBALANCE as the PN volumes for Non-Priority Dispatch Renewable Units will already be included in the TOTAL PN values. |
| TOTAL PN | NUMBER(8,3) | Sum of the Spot PNs (MW) on the Settlement Imbalance Period Start Time. |
| NET INTERCONNECTOR SCHEDULE | NUMBER(8,3) | Net Interconnector Schedule |
| TSO DEMAND FORECAST | NUMBER(8,3) | TSO demand forecast (MW) |
| TSO RENEWABLE FORECAST (PRIORITY DISPATCH) | NUMBER(8,3) | TSO renewables forecast (for Priority Dispatch Renewable Units) |
| TSO RENEWABLE FORECAST (NPDR) | NUMBER(8,3) | TSO renewables forecast (for Non-Priority Dispatch Renewable Units) |

Table 31: Forecast Imbalance Report

### BM-015: Net Imbalance Volume Forecast Report

This report contains the single, total forecasted imbalance volume for the relevant Trading Day, updated every 30 minutes from 13:30 on Trading Day – 1 up to the first Intra-Day Gate Closure for the Trading Day.

| I-SEM Report Reference: | BM-015 |
| --- | --- |
| Periodicity: | On_demand |
| Report Name: | PUB_HrlyNetImbalVolumeForecast |
| File Names: | PUB_HrlyNetImbalVolumeForecast_YYYYMMDDHHMM.xml |
| Report Title: | Net Imbalance Volume Forecast |
| Audience: | General Public |
| Resolution: | Trading Day |
| Time Span: | Current Trading Day |
| Frequency: | Every 60 Minutes from 13:30 on Trading Day – 1 up to the first Intra-Day Gate Closure for the Trading Day. |
| Report Format: | XML |

| Field Name | Format | Description |
| --- | --- | --- |
| TRADE DATE | DATE (YYYY-MM-DD) | Trading Day |
| NET IMBALANCE FORECAST | NUMBER(11,3) | Forecast Imbalance in MW |

Table 32: Net Imbalance Volume Forecast Report

### BM-016: Aggregated Wind Forecast Report

This report is produced four times per day, following the receipt of each Wind Unit Forecast, containing aggregate 15 minute forecasts for all jurisdictions (i.e. the sum for all jurisdictions in REPT_028)..

| I-SEM Report Reference: | BM-016 |
| --- | --- |
| Data Source | TSO |
| Periodicity: | Daily (4 times per day) |
| Report Name: | PUB_15MinAggWindFcst |
| File Names: | PUB_15MinAggWindFcst_YYYYMMDDHHMM.xml |
| Report Title: | Aggregated Wind Forecast Report |
| Audience: | General Public |
| Resolution: | 15 mins |
| Time Span: | Next four Trading Days |
| Frequency: | Four times per day, following receipt of each Wind Unit Forecast |
| Report Format: | XML |

| Field Name | Format | Description |
| --- | --- | --- |
| TRADE DATE | DATE (YYYY-MM-DD) | Trading Day |
| DELIVERY DATE | DATE (YYYY-MM-DD) | Calendar Day (referred to as “Day” in the Code). |
| START TIME | TIME(YYYY-MM-DDTH24:MI) | Start date/time of forecast data |
| END TIME | TIME(YYYY-MM-DDTH24:MI) | Start date/time of forecast data |
| FORECAST | NUMBER(11,3) | Aggregated Wind Forecast |

Table 33: Aggregated Wind Forecast Report

### BM-018: Daily Interconnector NTC Report

This report is, by default, run once per day as received from the Coordinated Capacity Calculation process and reports the Net Transfer Capacity (NTC) for each interconnector, for each Trading Period per Trading Day. It is also produced when new NTC values are provided.

| I-SEM Report Reference: | BM-018 |
| --- | --- |
| Data Source | TSO |
| Periodicity: | Daily |
| Report Name: | PUB_DailyIntconNTC |
| File Names: | PUB_DailyIntconNTC_YYYYMMDDHHMM.xml |
| Report Title: | Daily Interconnector NTC Report |
| Audience: | General Public |
| Resolution: | Trading Period |
| Time Span: | Trading Day |
| Frequency: | Event driven for the TD following receipt of new NTC values, following receipt of results from the Co-ordinated Capacity Calculator for TD+1 and TD+2 |
| Report Format: | XML |

| Field Name | Format | Description |
| --- | --- | --- |
| TRADE DATE | DATE (YYYY-MM-DD) | Trading Day |
| RESOURCE NAME | VARCHAR2(32) | The name of the Interconnector |
| DELIVERY DATE | DATE (YYYY-MM-DDYYYY) | Calendar Day (referred to as “Day” in the Code). |
| START TIME | TIME(YYYY-MM-DDTH24:MI) | Start Time |
| END TIME | TIME(YYYY-MM-DDTH24:MI) | End Time |
| MAXIMUM IMPORT MW | NUMBER(11,3) | Maximum Import NTC for each Trading Period |
| MAXIMUM EXPORT MW | NUMBER(11,3) | Maximum Export NTC for each Trading Period |

Table 34: Daily Interconnector NTC Report

### BM-019: Annual Combined Loss Adjustment Factor Report

This report details Combined Loss Adjustment Factors (CLAFs) for Units for each Trading Period in the relevant year, accounting for Transmission and Distribution losses associated with the transfer of energy.

| I-SEM Report Reference: | BM-019 |
| --- | --- |
| Periodicity: | Annual |
| Report Name: | PUB_AnnualCLAF |
| File Names: | PUB_AnnualCLAF.xml |
| Report Title: | Annual Combined Loss Adjustment Factor Report |
| Audience: | General Public |
| Resolution: | Trading Period |
| Time Span: | Following year |
| Frequency: | Once every year, in August and ad hoc upon new Unit registration |
| Report Format: | XML |

| Field Name | Format | Description |
| --- | --- | --- |
| TRADE DATE | DATE (YYYY-MM-DD) | Trading Day |
| PARTICIPANT NAME | VARCHAR2(12) | The PT Identifier, which represents the name of Market Participant, as registered in the I-SEM Balancing Market Trading Interface. |
| RESOURCE NAME | VARCHAR2(32) | Resource identifier, e.g. GU_XXXXXX |
| DELIVERY DATE | DATE (YYYY-MM-DD) | Calendar Day (referred to as “Day” in the Code). |
| START TIME | TIME(YYYY-MM-DDTH24:MI) | Start time of the time period to which the Loss Factor relates |
| END TIME | TIME(YYYY-MM-DDTH24:MI) | End time of the time period to which the Loss Factor relates |
| LOSS FACTOR | NUMBER (4,3) | Loss Factor |

Table 35: Combined Loss Adjustment Factor Report

### BM-020: Aggregated Final Physical Notifications Report

This report details the aggregated Final Physical Notifications submitted by Generator and Demand Side Units for the Imbalance Settlement Period.

| I-SEM Report Reference: | BM-020 |
| --- | --- |
| Data Source | TSO |
| Periodicity: | Daily |
| Report Name: | PUB_HlfHrlyAggregatedPN |
| File Names: | PUB_HlfHrlyAggregatedPN_YYYYMMDDHHMM.xml |
| Report Title: | Aggregated Final Physical Notifications Report |
| Audience: | General Public |
| Resolution: | Imbalance Settlement Period |
| Time Span: | Relevant Imbalance Settlement Period |
| Frequency: | After each Balancing Market Gate Closure |
| Report Format: | XML, CSV |

| Field Name | Format | Description |
| --- | --- | --- |
| TRADE DATE | DATE (YYYY-MM-DD) | Trading Day |
| DELIVERY DATE | DATE (YYYY-MM-DD) | Calendar Day (referred to as “Day” in the Code). |
| START TIME | TIME(YYYY-MM-DDTH24:MI) | Start Time of the aggregated profile segment |
| START MW AGGREGATED PN | NUMBER(83) | Start MW of Aggregated Physical Notification |
| END TIME | TIME(YYYY-MM-DDTH24:MI) | End Time of the aggregated profile segment |
| END MW AGGREGATED PN | NUMBER(8,3) | End MW of Aggregated Physical Notification |

Table 36: Aggregated Physical Notifications Report

### BM-021: Anonymised Inc / Dec Curves Report

This report contains the anonymised composite valid submitted Inc/Dec values for the relevant Imbalance Settlement Period. The data describes a curve which comprises of Incs and Decs for all Participants, sorted by price in ascending order for each aggregate associated quantity.

| I-SEM Report Reference: | BM-021 |
| --- | --- |
| Data Source | TSO |
| Periodicity: | Daily |
| Report Name: | PUB_HrlyAnonymisedIncDecCurves |
| File Names: | PUB_HrlyAnonymisedIncDecCurves_YYYYMMDDHHMM.xml |
| Report Title: | Anonymised Inc / Dec Curves Report |
| Audience: | General Public |
| Resolution: | Trading Period |
| Time Span: | Imbalance Settlement Period |
| Frequency: | Hourly |
| Report Format: | XML |

| Field Name | Format | Description |
| --- | --- | --- |
| TRADE DATE | DATE (YYYY-MM-DD) | Trading Day |
| START TIME | TIME(YYYY-MM-DDTH24:MI) | Imbalance Settlement Period Start Time |
| END TIME | TIME(YYYY-MM-DDTH24:MI) | Imbalance Settlement Period End time |
| INC QUANTITY | NUMBER(8,3) | Aggregate inc quantity for a given price. |
| INC PRICE | NUMBER(8,2) | Submitted inc price |
| DEC QUANTITY | NUMBER(8,3) | Aggregate dec quantity for a given price. |
| DEC PRICE | NUMBER(8,2) | Submitted dec price |

Table 37: Anonymised Inc/Dec Curves Report

### BM-022: System Shortfall Imbalance Index & flattening Factor

This report contains the calculated System Shortfall Imbalance Index (the ratio of the energy shortfall divided by the total forecast energy demand for this Trading Day) and the System Imbalance Flattening Factor (a factor used to determine adjusted startup costs for use in the Operational Schedule Runs).

| I-SEM Report Reference: | BM-022 |
| --- | --- |
| Data Source | TSO |
| Periodicity | Daily |
| Report Name: | PUB_HrlySsiiSiff |
| File Names: | PUB_HrlySsiiSiff_YYYYMMDDHHMM.xml |
| Report Title: | System Shortfall Imbalance Index and Flattening Factor |
| Audience: | General Public |
| Resolution: Time Span | Trading Day Trading Day |
| Frequency: Format: | Hourly up to the first IDM Gate Closure for the Trading Day XML |

| Field Name | Format | Description |
| --- | --- | --- |
| TRADE DATE | DATE (YYYY-MM-DD) | Trading Day |
| SSII | NUMBER(8,3) | System Shortfall Imbalance Index |
| SIFF | NUMBER(8,3) | System Imbalance Flattening Factor |

Table 38: Daily Shortfall Imbalance Index & Flattening Factor Report

### BM-023: Final Physical Notifications Report

This report details the Final Physical Notifications submitted by Generator and Demand Side Units in the previous Trading Day.

| I-SEM Report Reference: | BM-023 |
| --- | --- |
| Data Source | MP submissions |
| Periodicity: | Daily |
| Report Name: | PUB_DailyFinalPhysicalNotifications |
| File Names: | PUB_DailyFinalPhysicalNotifications.xml |
| Report Title: | Final Physical Notifications Report |
| Audience: | General Public |
| Resolution: | Per Physical Notification |
| Time Span: | Previous Trading Day |
| Frequency: | Daily |
| Report Format: | XML |

| Field Name | Format |  | Description |
| --- | --- | --- | --- |
| PARTICIPANT NAME | VARCHAR2(12) |  | The PT Identifier, which represents the name of Market Participant, as registered in the Balancing Market. |
| RESOURCE NAME | VARCHAR2(32) |  | The name of the resource (e.g. the name of the Generating Unit, Supplier Unit, Demand Side Unit, Interconnector for which data is being reported). |
| TRADE DATE |  | DATE (YYYY-MM-DD) | Trading Day |
| DELIVERY DATE |  | DATE (YYYY-MM-DD) | Calendar Day (referred to as “Day” in the Code). |
| START TIME | TIME(YYYY-MM-DDTH24:MI) |  | Start Time of the Profile |
| START MW | NUMBER(8,3) |  | Start MW Point |
| END TIME | TIME(YYYY-MM-DDTH24:MI) |  | End Time of the Profile |
| END MW | NUMBER(8,3) |  | End MW Point |
| UNDER TEST FLG | CHAR(1) |  | Under Test Flag |
| REASON | VARCHAR2(255) |  | Reason for Under Test application |

Table 39: Final Physical Notifications Report

### BM-024: Generator Unit Under Test Notice

This report contains a Generator Unit Under Test Notice to the Market Operator by a participant, which will specify in all cases the Unit Under Test Start Date and time and the Unit Under Test End Date and time, and the Generator Unit Under Test.

| I-SEM Report Reference: | BM-024 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Periodically as required |
| Report Format: | Word/PDF |

### BM-025: Imbalance Price Report (Imbalance Pricing Period)

This report contains the Imbalance Price and quantity for BOA (QBOA/PBOA) based on the schedule outputs from RTD runs and real time dispatch instructions for the Imbalance Pricing Period. The report is published at the end of each Imbalance Price calculation process for the corresponding Imbalance Pricing Period.

| I-SEM Report Reference: | BM-025 |
| --- | --- |
| Data Source | RT IMBP Application |
| Periodicity: | Daily |
| Report Name: | PUB_5MinImbalPrc |
| File Names: | PUB_5MinImbalPrc_YYYYMMDDHHMM.xml |
| Report Title: | Imbalance Price Report |
| Audience: | General Public |
| Resolution: | Imbalance Pricing Period |
| Time Span: | Imbalance Pricing Period |
| Frequency: | End of imbalance price calculation process for the corresponding Imbalance Pricing Period |
| Report Format: | XML |

| Field Name | Format | Description |
| --- | --- | --- |
| TRADE DATE | DATE (YYYY-MM-DD) | Trading Day |
| START TIME | TIME(YYYY-MM-DDTH24:MI) | Start time of the Imbalance Price Period |
| END TIME | TIME(YYYY-MM-DDTH24:MI) | End time of the Imbalance Price Period |
| NET IMBALANCE VOLUME | NUMBER(8,3) | Net Imbalance Volume |
| DEFAULT PRICE USAGE | CHAR(1) | Default price used ‘Y’ or ‘N’ |
| ASP PRICE USAGE | CHAR(1) | Administered Scarcity Price Used ‘Y’ or ‘N’ |
| TOTAL UNIT AVAILABILITY | NUMBER(8,3) | Total Availability of all Units |
| DEMAND CONTROL VOLUME | NUMBER(8,3) | Demand Control Volume |
| PMEA | NUMBER(8,2) | Price of the Marginal Energy Action in EUR |
| QPAR | NUMBER(8,3) | Quantity Price Average Reference |
| ADMINISTERED SCARCITY PRICE | NUMBER(8,2) | Administered Scarcity Price in EUR |
| IMBALANCE PRICE | NUMBER(8,2) | Imbalance Price in EUR for the Imbalance Pricing Period |
| MARKET BACKUP PRICE | NUMBER(8,2) | Market Backup Price applicable for the Imbalance Settlement Period. |
| SHORT TERM RESERVE QUANTITY | NUMBER(8,3) | Short Term Reserve Quantity |
| OPERATING RESERVE REQUIREMENT | NUMBER(8,3) | Operating Reserve Requirement |

Table 40: Imbalance Price Report (Imbalance Pricing Period)

### BM-026: Imbalance Price Report (Imbalance Settlement Period)

This report contains the time weighted average Imbalance Price and net Imbalance Volume for each Trading Period (Imbalance Settlement Period). The report is published once after the end of each imbalance settlement period on the Trading Day.

| I-SEM Report Reference: | BM-026 |
| --- | --- |
| Data Source | RT IMBP Application |
| Periodicity: | Daily |
| Report Name: | PUB_30MinAvgImbalPrc |
| File Names: | PUB_30MinAvgImbalPrc_YYYYMMDDHHMM.xml |
| Report Title: | Imbalance Price Report (Imbalance Settlement Period) |
| Audience: | General Public |
| Resolution: | Imbalance Settlement Period |
| Time Span: | Previous Imbalance Settlement Period |
| Frequency: | Imbalance Settlement Period |
| Report Format: | XML |

| Field Name | Format | Description |
| --- | --- | --- |
| TRADE DATE | DATE (YYYY-MM-DD) | Trading Day |
| START TIME | TIME(HH:MM:SS) | Imbalance settlement period Start time |
| END TIME | TIME(HH:MM:SS) | Imbalance settlement period End time |
| NET IMBALANCE VOLUME | NUMBER(8,3) | Quantity of the Net Imbalance volume (QNIV) for the Imbalance Settlement Period |
| IMBALANCE SETTLEMENT PRICE | NUMBER(8,2) | Calculated average time weighted imbalance price for the Imbalance Settlement Period (PIMB) |

Table 41: Imbalance Price Report (Imbalance Settlement Period)

### Imbalance Price Supporting Information Report

This report contains the real time Imbalance Price and Quantity for BOA (QBOA/PBOA) based on the schedule outputs from RTID runs and real time dispatch instructions for each five minute interval. This report is available to the General Public. This report is published after every Imbalance Pricing run.

| I-SEM Report Reference: | BM-27 |
| --- | --- |
| Data Source | Balancing Market Runs |
| Periodicity: | Daily |
| Report Name: | PUB_5MinImbalPrcSuppInfo |
| File Names: | PUB_5MinImbalPrcSuppInfo_YYYYMMDDHHMM.xml |
| Report Title: | Imbalance Price Supporting Information Report |
| Audience: | General Public |
| Resolution: | Imbalance Pricing Period |
| Time Span: | Previous Imbalance Pricing Period |
| Frequency: | End of Imbalance Price calculation process for the corresponding Imbalance Pricing Period |
| Report Format: | XML |

| Field Name | Format | Description |
| --- | --- | --- |
| TRADE DATE | DATE (YYYY-MM-DD) | Trading Day |
| START TIME | TIME(YYYY-MM-DDTH24:MI) | Start time of the Imbalance Pricing Period |
| END TIME | TIME(YYYY-MM-DDTH24:MI) | End time of the Imbalance Pricing Period |
| PARTICIPANT NAME | VARCHAR2(12) | Participant Name |
| RESOURCE NAME | VARCHAR2(32) | Resource Name |
| PBOA | NUMBER(8,2) | PBOA |
| QBOA | NUMBER(8,3) | QBOA |
| RANK | NUMBER(10) | Rank of QBOA/PBOA |
| NON ENERGY FLAGS | NUMBER(1) | Non-Energy Flags (multiple entries as required with associated identifiers) |
| CONSTRAINT_ID | VARCHAR(20) | The Constraint ID associated with that unit, only present if there is a duplicate rank |
| SO FLAG | NUMBER(1) | SO flag (multiple entries as required with associated identifiers) |
| EMERGENCY_FLAG | NUMBER(1) | Emergency Flag |
| IMBALANCE PRICE FLAG | NUMBER(1) | Imbalance Price Flag |
| NET IMBALANCE VOLUME | NUMBER(8,3) | Net Imbalance Volume |
| PRICE AVERAGE REFERENCE TAG | NUMBER(8,2) | Price Average Reference Tag |
| NET IMBALANCE VOLUME TAG | NUMBER(8,3) | Net Imbalance Volume Tag |
| NON MARGINAL FLAG | NUMBER(1) | Non Marginal Flag |
| IMBALANCE PRICE TAG | NUMBER(8,2) | Imbalance Price Tag |

Table 42: Imbalance Price Supporting Information Report

| Constraint Type | Constraint ID | Test Type | Notes |
| --- | --- | --- | --- |
| PRM | S_PRM_TOT | OR | System wide primary operating reserve constraint |
| PRM | S_PRM_ROI | OR | ROI jurisdiction primary operating reserve constraint |
| PRM | S_PRM_NI | OR | NI jurisdiction primary operating reserve constraint |
| SEC | S_SEC_TOT | OR | System wide secondary operating reserve constraint |
| SEC | S_SEC_ROI | OR | ROI jurisdiction secondary operating reserve constraint |
| SEC | S_SEC_NI | OR | NI jurisdiction secondary operating reserve constraint |
| TR1 | S_TR1_TOT | OR | System wide tertiary 1 operating reserve constraint |
| TR1 | S_TR1_ROI | OR | ROI jurisdiction tertiary 1 operating reserve constraint |
| TR1 | S_TR1_NI | OR | NI jurisdiction tertiary 1 operating reserve constraint |
| TR2 | S_TR2_TOT | OR | System wide tertiary 2 operating reserve constraint |
| TR2 | S_TR2_ROI | OR | ROI jurisdiction tertiary 2 operating reserve constraint |
| TR2 | S_TR2_NI | OR | NI jurisdiction tertiary 2 operating reserve constraint |
| FFR | S_FFR_TOT | OR | System wide fast frequency response constraint |
| FFR | S_FFR_ROI | OR | ROI jurisdiction fast frequency response constraint |
| FFR | S_FFR_NI | OR | NI jurisdiction fast frequency response constraint |
| NRR | S_NEG_TOT | OR | System wide negative ramping reserve constraint |
| NRR | S_NEG_ROI | OR | ROI jurisdiction negative ramping reserve constraint |
| NRR | S_NEG_NI | OR | NI jurisdiction negative ramping reserve constraint |
| REP | S_REP_TOT | OR | System wide replacement reserve constraint |
| REP | S_REP_ROI | OR | ROI jurisdiction replacement reserve constraint |
| REP | S_REP_NI | OR | NI jurisdiction replacement reserve constraint |
| RM1 | S_RM1_TOT | OR | System wide ramp margin 1 constraint |
| RM1 | S_RM1_ROI | OR | ROI jurisdiction ramp margin 1 constraint |
| RM1 | S_RM1_NI | OR | NI jurisdiction ramp margin 1 constraint |
| RM3 | S_RM3_TOT | OR | System wide ramp margin 3 constraint |
| RM3 | S_RM3_ROI | OR | ROI jurisdiction ramp margin 3 constraint |
| RM3 | S_RM3_NI | OR | NI jurisdiction ramp margin 3 constraint |
| RM8 | S_RM8_TOT | OR | System wide ramp margin 8 constraint |
| RM8 | S_RM8_ROI | OR | ROI jurisdiction ramp margin 8 constraint |
| RM8 | S_RM8_NI | OR | NI jurisdiction ramp margin 8 constraint |
| INERTIA | S_INERTIA_TOT | Inertia | System wide inertia constraint |
| SNSP | S_SNSP_TOT | SNSP | System wide SNSP (asynchronous generation limit) constraint |
| RoCoF | S_RoCoF | ROCOF | RoCoF constraint |
| FREQMGT | S_FREQ | FREQMGT | Footroom limits in jurisdiction for frequency management during I/C outage |
| STL | S_STL_NI | STL | Limit on tie line flows to NI during single tie line outage |
| IARAMP | S_IARAMP | IARAMP | Inter-Area ramp constraints |
| NBMIN | S_NBMIN_TCGNAME | NB | All NB min constraints |
| NBMAX | S_NBMAX_TCGNAME | NB | All NB max constraints |
| MWMIN | S_MWMIN_TCGNAME | MW | All MW min constraints |
| MWMAX | S_MWMAX_TCGNAME | MW | All MW max constraints |
| MWR | S_MWR_TCGNAME | MWR | All MWR constraints |

Table 43: Definition of ConstraintID Values Used in BM-027

### BM-028: Price Materiality Threshold

This report contains the threshold detailed by the Regulatory Authorities to apply in the event of a pricing dispute.

| I-SEM Report Reference: | BM-028 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Periodically as required |
| Report Format: | Word/PDF |

### BM-029: Settlement Recalculation Threshold

This report contains the threshold approved by the Regulatory Authorities to apply in the event of a Settlement Rerun as a result of a Dispute or Settlement Query.

| I-SEM Report Reference: | BM-029 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Periodically as required |
| Report Format: | Word/PDF |

### BM-030: Imbalance Weighting Factor

This report contains the Imbalance Weighting Factor in an Imbalance Settlement Period.

| I-SEM Report Reference: | BM-030 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Periodically as required |
| Report Format: | Word/PDF |

### BM-031: De Minimis Acceptance Threshold

This report contains the de minimis acceptance threshold, which is the parameter determined by the Regulatory Authorities as the level below which Accepted Bids and Accepted Offers are to be excluded from the calculation of Imbalance Prices.

| I-SEM Report Reference: | BM-031 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Periodically as required |
| Report Format: | Word/PDF |

### BM-032: Daily Technical Offer Data Report

This report details the relevant Technical Offer Data for both Generator Units and Demand Side Units, for the previous Trading Day.

| I-SEM Report Reference: | BM-032 |
| --- | --- |
| Data Source | Market Participant |
| Periodicity: | Daily |
| Report Name: | PUB_DailyTechnicalOfferData |
| File Names: | PUB_DailyTechnicalOfferData.xml |
| Report Title: | Daily Technical Offer Data Report |
| Audience: | General Public |
| Resolution: | Trading Day |
| Time Span: | Previous Trading Day (TD-1) |
| Frequency: | Daily |
| Report Format: | XML |

| Field Name | Format | Description |
| --- | --- | --- |
| TRADE DATE | DATE (YYYY-MM-DD) | Trading Day |
| START TIME | TIME(YYYY-MM-DDTH24:MI) | Settlement Imbalance Period Start Time |
| END TIME | TIME(YYYY-MM-DDTH24:MI) | Settlement Imbalance Period End Time |
| DELIVERY DATE | DATE (YYYY-MM-DD) | Calendar Day (referred to as “Day” in the Code). |
| RESOURCE NAME | VARCHAR2(32) | The name of the resource (e.g. the name of the Generating Unit, Supplier Unit, Demand Side Unit, Interconnector for which data is being reported). |
| RESOURCE TYPE | VARCHAR2(4) | Indicates the type of resource for which data is being submitted - permitted values include: GU, DSU |
| JURISDICTION | VARCHAR2(32) | ROI (Republic of Ireland) or NI (Northern Ireland) |
| FUEL TYPE | VARCHAR2(22) | Fuel type. Possible values are: Oil Gas Peat Wind Biomass CHP Hydro Coal Distillate Nuclear Battery Storage Multi Fuel Pump Storage Fly Wheel Solar Compressed Air Storage Other Synchronous Condenser |
| PRIORITY DISPATCH YN | CHAR(1) | Indication of unit’s priority in the physical market schedule if in a tie to serve marginal demand. (Y/N) |
| DISPATCHABLE YN | CHAR(1) | Dispatchable flag ‘Y’ or ‘N’ |
| CONTROLLABLE YN | CHAR(1) | Controllable flag ‘Y’ or ‘N’ |
| PUMP STORAGE YN | CHAR(1) | It is a Y/N/Null field. Null for supplier, demand and Interconnector Y for Pumped Storage Unit |
| ENERGY LIMIT YN | CHAR(1) | Only applicable to Generators |
| UNDER TEST YN | CHAR(1) | It is a Y/N/Null field.  Null for Unit is not under test  Y for Unit is under test |
| FIRM ACCESS QUANTITY | NUMBER(8,3) | Total deep connected capacity designation for the unit |
| NON FIRM ACC QUANTITY | NUMBER(8,3) | Non-firm capacity for a unit in MW, i.e. part of a Generator Unit's Availability that does not have Firm Access. |
| SHORT TERM MAXIMISATION CAP | NUMBER(8,3) | Capacity above MAXGEN that can be sustained for a finite period of time (MW) |
| MINIMUM GENERATION | NUMBER(8,3) | Minimum Output of Generator Unit. The lowest value to which a unit can be scheduled. |
| MAXIMUM GENERATION | NUMBER(8,3) | Maximum stable Generation level in MW. |
| MINIMUM ON TIME | NUMBER(11,3) | Minimum time that must elapse from the time a Generation Unit is instructed to start-up before it can be instructed to shut down. |
| MINIMUM OFF TIME | NUMBER(11,3) | Minimum time that a Generation Unit must remain producing no Active Power commencing at the time when it first stops producing no Active Power. |
| MAXIMUM ON TIME | NUMBER(11,3) | Maximum time that can be elapse from the time a Generation Unit is instructed to start-up and run continuously before it must be instructed to shut down. |
| HOT COOLING BOUNDARY | NUMBER(5,2) | The duration in hours off load that indicates the standby status change of the unit from Hot to Warm. |
| WARM COOLING BOUNDARY | NUMBER(5,2) | The duration in hours off load that indicates the standby status change of the unit from Warm to Cold. |
| SYNCHRONOUS START UP TIME HOT | NUMBER(5,2) | Notification/Start-up times in hour for hot state |
| SYNCHRONOUS START UP TIME WARM | NUMBER(5,2) | Notification/Start-up times in hour for warm state |
| SYNCHRONOUS START UP TIME COLD | NUMBER(5,2) | Notification/Start-up times in hour for cold state |
| BLOCK LOAD COLD | NUMBER(8,3) | Block Loads for Cold state expressed in MW. |
| BLOCK LOAD HOT | NUMBER(8,3) | Block Loads for Hot state expressed in MW. |
| BLOCK LOAD WARM | NUMBER(8,3) | Block Loads for Warm state expressed in MW. |
| LOADING RATE COLD 1 | NUMBER(8,3) | First point of Load up rates between load up break points expressed in MW/min for Cold state. |
| LOADING RATE COLD 2 | NUMBER(8,3) | Second point of Load up rates between load up break points expressed in MW/min for Cold state |
| LOADING RATE COLD 3 | NUMBER(8,3) | Third point of Load up rates between load up break points expressed in MW/min for Cold state. |
| LOAD UP BREAK POINT COLD 1 | NUMBER(8,3) | First break points of the cold state load up curve expressed in MW. |
| LOAD UP BREAK POINT COLD 2 | NUMBER(8,3) | Second break points of the cold state load up curve expressed in MW. |
| LOADING RATE HOT 1 | NUMBER(8,3) | First point of Load up rates between load up break points expressed in MW/min for Hot state. |
| LOADING RATE HOT 2 | NUMBER(8,3) | Second point of Load up rates between load up break points expressed in MW/min for Hot state. |
| LOADING RATE HOT 3 | NUMBER(8,3) | Third point of Load up rates between load up break points expressed in MW/min for Hot state. |
| LOAD UP BREAK POINT HOT 1 | NUMBER(8,3) | First break points of the hot state load up curve expressed in MW. |
| LOAD UP BREAK POINT HOT 2 | NUMBER(8,3) | Second break points of the hot state load up curve expressed in MW. |
| LOADING RATE WARM 1 | NUMBER(8,3) | First point of Load up rates between load up break points expressed in MW/min for Warm state. |
| LOADING RATE WARM 2 | NUMBER(8,3) | Second point of Load up rates between load up break points expressed in MW/min for Warm state. |
| LOADING RATE WARM 3 | NUMBER(8,3) | Third point of Load up rates between load up break points expressed in MW/min for Warm state. |
| LOAD UP BREAK POINT WARM 1 | NUMBER(8,3) | First break points of the warm state load up curve expressed in MW. |
| LOAD UP BREAK POINT WARM 2 | NUMBER(8,3) | Second break points of the warm state load up curve expressed in MW. |
| SOAK TIME COLD 1 | NUMBER(3) | First point of the Soak Time associated with Trigger point in cold state. Soak times used during start up. |
| SOAK TIME COLD 2 | NUMBER(3) | Second point of the Soak Time associated with Trigger point in cold state. Soak times used during start up. |
| SOAK TIME TRIGGER POINT COLD 1 | NUMBER(8,3) | First trigger points for soak times in Cold state expressed in MW |
| SOAK TIME TRIGGER POINT COLD 2 | NUMBER(8,3) | Second trigger points for soak times in Cold state expressed in MW |
| SOAK TIME HOT 1 | NUMBER(3) | First point of the Soak Time associated with Trigger point in hot state. Soak times used during start up. |
| SOAK TIME HOT 2 | NUMBER(3) | Second point of the Soak Time associated with Trigger point in hot state. Soak times used during start up. |
| SOAK TIME TRIGGER POINT HOT 1 | NUMBER(8,3) | First trigger points for soak times in Hot state expressed in MW |
| SOAK TIME TRIGGER POINT HOT 2 | NUMBER(8,3) | Second trigger points for soak times in Hot state expressed in MW |
| SOAK TIME WARM 1 | NUMBER(3) | First point of the Soak Time associated with Trigger point in warm state. Soak times used during start up. |
| SOAK TIME WARM 2 | NUMBER(3) | Second point of the Soak Time associated with Trigger point in warm state. Soak times used during start up. |
| SOAK TIME TRIGGER POINT WARM 1 | NUMBER(8,3) | First trigger points for soak times in Warm state expressed in MW |
| SOAK TIME TRIGGER POINT WARM 2 | NUMBER(8,3) | Second trigger points for soak times in Warm state expressed in MW |
| RAMP UP RATE 1 | NUMBER(8,3) | First point of Ramp Up Curve (MW/min). |
| RAMP UP RATE 2 | NUMBER(8,3) | Second point of Ramp Up Curve (MW/min). |
| RAMP UP RATE 3 | NUMBER(8,3) | Third point of Ramp Up Curve (MW/min). |
| RAMP UP RATE 4 | NUMBER(8,3) | Fourth point of Ramp Up Curve (MW/min). |
| RAMP UP RATE 5 | NUMBER(8,3) | Fifth point of Ramp Up Curve (MW/min). |
| RAMP UP BREAK POINT 1 | NUMBER(8,3) | First break point of the Ramp Up Curve (MW) |
| RAMP UP BREAK POINT 2 | NUMBER(8,3) | Second break point of the Ramp Up Curve (MW) |
| RAMP UP BREAK POINT 3 | NUMBER(8,3) | Third break point of the Ramp Up Curve (MW) |
| RAMP UP BREAK POINT 4 | NUMBER(8,3) | Fourth break point of the Ramp Up Curve (MW) |
| DWELL TIME 1 | NUMBER(3) | First point of the Dwell Times associated with Trigger point for Dwell Times in MW. |
| DWELL TIME 2 | NUMBER(3) | Second point of the Dwell Times associated with Trigger point for Dwell Times in MW. |
| DWELL TIME 3 | NUMBER(3) | Third point of the Dwell Times associated with Trigger point for Dwell Times in MW. |
| DWELL TIME TRIGGER POINT 1 | NUMBER(8,3) | First trigger points for Dwell times in MW. |
| DWELL TIME TRIGGER POINT 2 | NUMBER(8,3) | Second trigger points for Dwell times in MW. |
| DWELL TIME TRIGGER POINT 3 | NUMBER(8,3) | Third trigger points for Dwell times in MW. |
| DWELL TIME DOWN 1 | NUMBER(3) | First point of the Dwell Times Down associated with Trigger point for Dwell Times Down in MW. |
| DWELL TIME DOWN 2 | NUMBER(3) | Second point of the Dwell Times Down associated with Trigger point for Dwell Times Down in MW. |
| DWELL TIME DOWN 3 | NUMBER(3) | Second point of the Dwell Times Down associated with Trigger point for Dwell Times Down in MW. |
| DWELL TIME DOWN TRIGGER POINT 1 | NUMBER(8,3) | First trigger points for Dwell times Down in MW. |
| DWELL TIME DOWN TRIGGER POINT 2 | NUMBER(8,3) | Second trigger points for Dwell times Down in MW. |
| DWELL TIME DOWN TRIGGER POINT 3 | NUMBER(8,3) | Third trigger points for Dwell times Down in MW. |
| RAMP DOWN RATE 1 | NUMBER(8,3) | Fifth point of Ramp Down Curve (MW/min). |
| RAMP DOWN RATE 2 | NUMBER(8,3) | Fourth point of Ramp Down Curve (MW/min). |
| RAMP DOWN RATE 3 | NUMBER(8,3) | Third point of Ramp Down Curve (MW/min). |
| RAMP DOWN RATE 4 | NUMBER(8,3) | Second point of Ramp Down Curve (MW/min). |
| RAMP DOWN RATE 5 | NUMBER(8,3) | First point of Ramp Down Curve (MW/min). |
| RAMP DOWN BREAK POINT 1 | NUMBER(8,3) | First break point of the Ramp Down Curve (MW) |
| RAMP DOWN BREAK POINT 2 | NUMBER(8,3) | Second break point of the Ramp Down Curve (MW) |
| RAMP DOWN BREAK POINT 3 | NUMBER(8,3) | Third break point of the Ramp Down Curve (MW) |
| RAMP DOWN BREAK POINT 4 | NUMBER(8,3) | Fourth break point of the Ramp Down Curve (MW) |
| DELOADING RATE 1 | NUMBER(8,3) | Second point of the deload rate curve from end point of startup period to deloading break point to zero expressed in MW. |
| DELOADING RATE 2 | NUMBER(8,3) | First point of the deload rate curve from end point of startup period to deloading break point to zero expressed in MW. |
| DELOAD BREAK POINT | NUMBER(8,3) | Deloading break expressed in MW. |
| MAXIMUM STORAGE CAPACITY | NUMBER(8,3) | Maximum possible capacity that can be produced by a Battery Storage Unit, or by the reservoir for a Pumped Storage Unit (MWh) |
| MINIMUM STORAGE CAPACITY | NUMBER(8,3) | Minimum possible capacity that can be produced by a Battery Storage Unit, or by the reservoir for a Pumped Storage Unit (MWh) |
| ESPS STORAGE CYC EFY | NUMBER(7,4) | Storage Cycle Efficiency for Battery Storage Units, expressed as a percentage value. 0 < ESPS Storage CYC EFY ≤ 100.0000 |
| PUMPING LOAD CAP | NUMBER(8,3) | The maximum amount of Active Power consumed by a Pumped Storage Unit when in Pumping Mode, or a Battery Storage Unit in Storage Mode (MW). |
| ENERGY LIMIT MWH | NUMBER(8,3) | For Energy Limited Unit |
| FIXED UNIT LOAD | NUMBER(8,3) | Fixed Unit Load (FUL) ≥ 0 |
| UNIT LOAD SCALAR | NUMBER(5,4) | Unit Load Scalar (ULS). 0 < ULS ≤ 1.9999 |
| DUAL FUEL FLAG | CHAR(1) | Dual Fuel capable flag. The valid values are ‘Y’ and ‘N’ |
| SECONDARY FUEL TYPE | VARCHAR2(22) | Secondary Fuel Type. |
| MAXIMUM RAMP UP RATE | NUMBER(8,3) | Rate of load increase. Rate of decreasing demand (MW/min). DSU-only field. |
| MAXIMUM RAMP DOWN RATE | NUMBER(8,3) | Rate of load reduction; Rate of increasing demand (MW/min). DSU-only field. |
| MINIMUM DOWN TIME | NUMBER(11,3) | Minimum amount of time the demand-side unit can be scheduled to be off (in Hours) DSU-only field. |
| MAXIMUM DOWN TIME | NUMBER(11,3) | Maximum amount of time the demand-side unit can be scheduled to be off (in Hours) DSU-only field. |

Table 44: Daily Technical Offer Data Report - Standard Units

### BM-033: Forecast Availability Report

This report details the forecast availability per Generator Unit and Demand Side Unit for each Trading Period in the previous Trading Day (including wind units).

| I-SEM Report Reference: | BM-033 |
| --- | --- |
| Data Source | Market Participant |
| Periodicity: | Daily |
| Report Name: | PUB_DailyForecastAvailability |
| File Names: | PUB_DailyForecastAvailability.xml |
| Report Title: | Forecast Availability Report |
| Audience: | General Public |
| Resolution: | Trading Period |
| Time Span: | At a user-configurable time prior to the start of the Trading Day, for the entire Trading Day |
| Frequency: | Daily |
| Report Format: | XML |

| Field Name | Format | Description |
| --- | --- | --- |
| TRADE DATE | DATE (YYYY-MM-DD) | Trading Day |
| PARTICIPANT NAME | VARCHAR2(12) | The PT Identifier, which represents the name of Market Participant, as registered in the Balancing Market. |
| RESOURCE NAME | VARCHAR2(32) | The name of the resource (e.g. the name of the Generating Unit, Supplier Unit, Demand Side Unit, Interconnector for which data is being reported). |
| RESOURCE TYPE | VARCHAR2(4) | Indicates the type of resource for which data is being submitted – permitted values include: GU and DSU |
| JURISDICTION | VARCHAR2(5) | ROI (Republic of Ireland) or NI (Northern Ireland) |
| DELIVERY DATE | DATE (YYYY-MM-DD) | Calendar Day (referred to as “Day” in the Code). |
| START TIME | TIME(YYYY-MM-DDTH24:MI) | Start Time |
| END TIME | TIME(YYYY-MM-DDTH24:MI) | End Time |
| FORECAST AVAILABILITY | NUMBER(8,3) | Forecast Availability Profile (MW) |
| FORECAST MINIMUM STABLE GEN | NUMBER(8,3) | Forecast Minimum Stable Generation Profile (MW) |
| FORECAST MINIMUM OUTPUT | NUMBER(8,3) | Forecast Minimum Output Profile (MW) |
| FUEL USE FLAG | CHAR(1) | Indicates the fuel use type selected. The valid values are ‘P’ and ‘S’ (denoting Primary and Secondary respectively). |

Table 45: Forecast Availability Report

### BM-034: Commercial Offer Data Report

This report is produced after the Trading Day, detailing the Simple Commercial Offer Data and Complex Commercial Offer Data, for each Imbalance Settlement Period.

| I-SEM Report Reference: | BM-034 |
| --- | --- |
| Data Source | MP submissions |
| Periodicity: | Daily |
| Report Name: | PUB_DailyCommercialOfferData |
| File Names: | PUB_DailyCommercialOfferData.xml |
| Report Title: | Commercial Offer Data Report – General Public |
| Audience: | General Public |
| Resolution: | Trade Period (23:00 D-1 to 23:00 D) |
| Time Span: | Previous Trading Day |
| Frequency: | Daily |
| Report Format: | XML |

| Field Name | Format | Description |
| --- | --- | --- |
| TRADE DATE | DATE (YYYY-MM-DD) | Trading Day |
| PARTICIPANT_NAME | VARCHAR2(12) | Name of the participant. |
| RESOURCE NAME | VARCHAR2(32) | The name of the resource (e.g. the name of the Generating Unit, Supplier Unit, Demand Side Unit, Interconnector for which data is being reported). |
| RESOURCE TYPE | VARCHAR2(4) | Indicates the type of resource for which data is being submitted – permitted values include: GU, DSU, |
| JURISDICTION | VARCHAR2(5) | ROI (Republic of Ireland) or NI (Northern Ireland) |
| BID TYPE | VARCHAR2(6) | Complex /Simple |
| DELIVERY DATE | DATE (YYYY-MM-DD) | Calendar Day (referred to as “Day” in the Code). |
| START TIME | TIME(YYYY-MM-DDTH24:MI) | Start Time |
| END TIME | TIME(YYYY-MM-DDTH24:MI) | End Time |
| INCPRICE1 | NUMBER(8,2) | INC PQ Pair Price 1 |
| INCQUANTITY1 | NUMBER(8,3) | INC PQ Pair Quantity 1 |
| INCPRICE2 | NUMBER(8,2) | INC PQ Pair Price 2 |
| INCQUANTITY2 | NUMBER(8,3) | INC PQ Pair Quantity 2 |
| INCPRICE3 | NUMBER(8,2) | INC PQ Pair Price 3 |
| INCQUANTITY3 | NUMBER(8,3) | INC PQ Pair Quantity 3 |
| INCPRICE4 | NUMBER(8,2) | INC PQ Pair Price 4 |
| INCQUANTITY4 | NUMBER(8,3) | INC PQ Pair Quantity 4 |
| INCPRICE5 | NUMBER(8,2) | INC PQ Pair Price 5 |
| INCQUANTITY5 | NUMBER(8,3) | INC PQ Pair Quantity 5 |
| INCPRICE6 | NUMBER(8,2) | INC PQ Pair Price 6 |
| INCQUANTITY6 | NUMBER(8,3) | INC PQ Pair Quantity 6 |
| INCPRICE7 | NUMBER(8,2) | INC PQ Pair Price 7 |
| INCQUANTITY7 | NUMBER(8,3) | INC PQ Pair Quantity 7 |
| INCPRICE8 | NUMBER(8,2) | INC PQ Pair Price 8 |
| INCQUANTITY8 | NUMBER(8,3) | INC PQ Pair Quantity 8 |
| INCPRICE9 | NUMBER(8,2) | INC PQ Pair Price 9 |
| INCQUANTITY9 | NUMBER(8,3) | INC PQ Pair Quantity 9 |
| INCPRICE10 | NUMBER(8,2) | INC PQ Pair Price 10 |
| INCQUANTITY10 | NUMBER(8,3) | INC PQ Pair Quantity 10 |
| DECPRICE1 | NUMBER(8,2) | DEC PQ Pair Price 1 |
| DECQUANTITY1 | NUMBER(8,3) | DEC PQ Pair Quantity 1 |
| DECPRICE2 | NUMBER(8,2) | DEC PQ Pair Price 2 |
| DECQUANTITY2 | NUMBER(8,3) | DEC PQ Pair Quantity 2 |
| DECPRICE3 | NUMBER(8,2) | DEC PQ Pair Price 3 |
| DECQUANTITY3 | NUMBER(8,3) | DEC PQ Pair Quantity 3 |
| DECPRICE4 | NUMBER(8,2) | DEC PQ Pair Price 4 |
| DECQUANTITY4 | NUMBER(8,3) | DEC PQ Pair Quantity 4 |
| DECPRICE5 | NUMBER(8,2) | DEC PQ Pair Price 5 |
| DECQUANTITY5 | NUMBER(8,3) | DEC PQ Pair Quantity 5 |
| DECPRICE6 | NUMBER(8,2) | DEC PQ Pair Price 6 |
| DECQUANTITY6 | NUMBER(8,3) | DEC PQ Pair Quantity 6 |
| DECPRICE7 | NUMBER(8,2) | DEC PQ Pair Price 7 |
| DECQUANTITY7 | NUMBER(8,3) | DEC PQ Pair Quantity 7 |
| DECPRICE8 | NUMBER(8,2) | DEC PQ Pair Price 8 |
| DECQUANTITY8 | NUMBER(8,3) | DEC PQ Pair Quantity 8 |
| DECPRICE9 | NUMBER(8,2) | DEC PQ Pair Price 9 |
| DECQUANTITY9 | NUMBER(8,3) | DEC PQ Pair Quantity 9 |
| DECPRICE10 | NUMBER(8,2) | DEC PQ Pair Price 10 |
| DECQUANTITY10 | NUMBER(8,3) | DEC PQ Pair Quantity 10 |
| STARTUP COST HOT | NUMBER(8,2) | Cost to start-up when in hot warmth state. Only applicable for complex (Bid Type “C”) |
| STARTUP COST WARM | NUMBER(8,2) | Cost to start-up when in warm warmth state. Only applicable for complex (Bid Type “C”) |
| STARTUP COST COLD | NUMBER(8,2) | Cost to start-up when in cold warmth state. Only applicable for complex (Bid Type “C”) |
| SHUTDOWN COST | NUMBER(8,2) | Cost to shut-down for Demand Side Unit. Only applicable for complex (Bid Type “C”) |
| NO LOAD COST | NUMBER(8,2) | The element of Operation Cost. Only applicable for complex (Bid Type “C”) |
| PUMP STORAGE CYC EFY | NUMBER(3) | For Pumped Storage Units. Only applicable for complex (Bid Type “C”) |
| OPERATIONAL MINIMUM STORAGE | NUMBER(8,3) | For Battery Storage Units. Only applicable for complex (Bid Type “C”) |
| OPERATIONAL MAXIMUM STORAGE | NUMBER(8,3) | For Battery Storage Units. Only applicable for complex (Bid Type “C”) |

Table 46: Commercial Offer Data Report – General Public

### BM-036: Daily Demand Control Data Transaction Report

This report contains Load Shedding (demand control) volume by Imbalance Settlement Period for the previous Trading Day.

| I-SEM Report Reference: | BM-036 |
| --- | --- |
| Data Source | Market Participant |
| Periodicity: | Daily |
| Report Name: | PUB_DailyDemandControlData |
| File Names: | PUB_DailyDemandControlData.xml |
| Report Title: | Daily Demand Control Data Transaction Report |
| Audience: | General Public |
| Resolution: | Imbalance Settlement Period |
| Time Span: | Previous Trading Day (TD-1) |
| Frequency: | Daily |
| Report Format: | XML |

| Field Name | Format | Description |
| --- | --- | --- |
| TRADE DATE | DATE (YYYY-MM-DD) | Trading Day |
| START TIME | TIME(YYYY-MM-DDTH24:MI) | Imbalance Settlement Period Start time |
| END TIME | TIME(YYYY-MM-DDTH24:MI) | Imbalance Settlement Period End time |
| ESTIMATE DEMAND REDUCTION MW | NUMBER(11,3) | Estimate of any reduction in demand as a consequence of Demand Control i.e. Load Shedding |

Table 47: Daily Demand Control Data Transaction Report

### BM-037: Daily Dispatch Instructions (D+1) Report

This report contains the Dispatch Instructions for the previous Calendar Day, for all applicable Units.

| I-SEM Report Reference: | BM-037 |
| --- | --- |
| Data Source | TSO |
| Periodicity: | Daily |
| Report Name: | PUB_DailyDispatchInstrD1 |
| File Names: | PUB_DailyDispatchInstrD1.xml |
| Report Title: | Daily Dispatch Instructions (D+1) Report |
| Audience: | General Public |
| Resolution: | Per Dispatch Instruction |
| Time Span: | Calendar Day – 1 |
| Frequency: | Daily |
| Report Format: | XML |

| Field Name | Format | Description |
| --- | --- | --- |
| EFF TIME | TIME(YYYY-MM-DDTH24:MI) | Instruction Time Stamp |
| EFF UNTILTIME | TIME(YYYY-MM-DDTH24:MI) | End time of the instruction |
| PARTICIPANT NAME | VARCHAR2(12) | The PT Identifier, which represents the name of Market Participant, as registered in the Balancing Market. |
| RESOURCE NAME | VARCHAR2(32) | The name of the resource (e.g. the name of the Generating Unit, Supplier Unit, Demand Side Unit, Interconnector for which data is being reported). |
| DISPATCH INSTRUCTION MW | NUMBER(8,3) | Dispatch Instruction(MW) |
| INSTRUCTION CODE | VARCHAR2(4) | Instruction Code |
| INSTRUCTION COMBINATION CODE | VARCHAR2(4) | Instruction Combination Code |
| INSTR ISSUE TIME | TIME(YYYY-MM-DDTH24:MI) | Instruction Issue Time |
| RAMP UP RATE | NUMBER(8,3) | Ramp Up Rate |
| RAMP DOWN RATE | NUMBER(8,3) | Ramp Down Rate |

Table 48: Daily Dispatch Instructions (D+1) Report

### BM-038: Daily Dispatch Instructions (D+4) Report

This report contains Dispatch Instructions for all applicable Units for Calendar Day-4.

| I-SEM Report Reference: | BM-038 |
| --- | --- |
| Data Source | TSO |
| Periodicity: | Daily |
| Report Name: | PUB_DailyDispatchInstrD4 |
| File Names: | PUB_DailyDispatchInstrD4.xml |
| Report Title: | Daily Dispatch Instructions (D+4) Report |
| Audience: | General Public |
| Resolution: | Per Dispatch Instructions |
| Time Span: | Calendar Day - 4 |
| Frequency: | Four days after Calendar Day |
| Report Format: | XML |

| Field Name | Format | Description |
| --- | --- | --- |
| EFF TIME | TIME(YYYY-MM-DDTH24:MI) | Instruction Time Stamp |
| EFF UNTILTIME | TIME(YYYY-MM-DDTH24:MI) | End time of the instruction |
| PARTICIPANT NAME | VARCHAR2(32) | The PT Identifier, which represents the name of Market Participant, as registered in the Balancing Market. |
| RESOURCE NAME | VARCHAR2(32) | The name of the resource (e.g. the name of the Generating Unit, Supplier Unit, Demand Side Unit, Interconnector for which data is being reported). |
| DISPATCH INSTRUCTION MW | NUMBER(8,3) | Dispatch Instruction (MW) |
| INSTRUCTION CODE | VARCHAR2(4) | Instruction Code |
| INSTRUCTION COMBINATION CODE | VARCHAR2(4) | Instruction Combination Code |
| INSTRISSUE TIME | TIME(YYYY-MM-DDTH24:MI) | Instruction Issue Time |
| RAMP UP RATE | NUMBER(8,3) | Ramp Up Rate |
| RAMP DOWN RATE | NUMBER(8,3) | Ramp Down Rate |

Table 49: Daily Dispatch Instructions (D+4) Report

### BM-039: Tracked Difference Payment Shortfall Amount

This report contains the Tracked Difference Payment Shortfall Amount, which is the sum of Difference Payment Shortfall Amounts and Difference Payment Reimbursement Amounts arising to date for a Supplier Unit in a Settlement Day for use in determining reimbursements for previous shortfalls amounts relating to Difference Payments.

| I-SEM Report Reference: | BM-039 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Periodically as required |
| Report Format: | Word/PDF |

### BM-040: Carbon Price

This report contains the Carbon Price in a Month reflecting the reference carbon price used to include the cost of carbon in the Strike Price for each of the fuels according to the carbon intensity of those fuels.

| I-SEM Report Reference: | BM-040 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Periodically as required |
| Report Format: | Word/PDF |

### BM-041: Natural Gas Fuel Price

This report contains the Natural Gas Fuel Price in a Month reflecting the reference natural gas price, inclusive of fuel transport adders, used in the calculation of the Strike Price.

| I-SEM Report Reference: | BM-041 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Periodically as required |
| Report Format: | Word/PDF |

### BM-042: Oil Fuel Price

This report contains the Oil Fuel Price in a Month reflecting the reference oil price, inclusive of fuel transport adders, used in the calculation of the Strike Price.

| I-SEM Report Reference: | BM-042 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Periodically as required |
| Report Format: | Word/PDF |

### BM-043: Peaking Unit THeoretical Efficiency

This report contains The Peaking Unit Theoretical Efficiency in a Capacity Year representing the higher heating value (HHV) efficiency of a theoretical peaking unit for use in calculating the Strike Price.

| I-SEM Report Reference: | BM-043 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Periodically as required |
| Report Format: | Word/PDF |

### BM-044: Natural Gas Carbon Intensity Factor

This report contains the Natural Gas Carbon Intensity Factor in a Capacity Year used to include the cost for carbon emissions when considering the reference price for natural gas.

| I-SEM Report Reference: | BM-044 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Periodically as required |
| Report Format: | Word/PDF |

### BM-045: Oil Carbon Intensity Factor

This report contains the The Oil Carbon Intensity Factor in a Capacity Year used to include the cost for carbon emissions when considering the reference price for oil.

| I-SEM Report Reference: | BM-045 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Periodically as required |
| Report Format: | Word/PDF |

### BM-046: Aggregated Settlement Period

This report details the Aggregated Settlement Period as determined by the Regulatory Authorities.

| I-SEM Report Reference: | BM-046 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Periodically as required |
| Report Format: | Word/PDF |

### BM-047: Annual Stop-Loss Limit Factor

This report contains the Annual Stop-Loss Limit Factor, which is the multiplier (%) used to establish the annual stop-loss limit for Non-Performance Difference Charges from a Capacity Market Unit approved by the Regulatory Authorities.

| I-SEM Report Reference: | BM-047 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Periodically as required |
| Report Format: | Word/PDF |

### BM-048: Billing Period Stop-Loss Limit Factor

This report contains the Billing Period Stop-Loss Limit Factor, which is the multiplier (%) used to establish the billing period stop-loss limit for Non-Performance Difference Charges from a Capacity Market Unit determined by the Regulatory Authorities.

| I-SEM Report Reference: | BM-048 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Periodically as required |
| Report Format: | Word/PDF |

### BM-049: Total Capacity Awarded

This report details the Total Capacity Awarded after every Capacity Auction.

| I-SEM Report Reference: | BM-049 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Periodically as required |
| Report Format: | Word/PDF |

### BM-050: Response Period Duration

This report contains the Response Period Duration, which is when the Market Operator provides a Participant with a Required Credit Cover Report containing a Credit Cover Increase Notice and the previous Required Credit Cover Report provided to the Participant did not contain a Credit Cover Increase Notice.

| I-SEM Report Reference: | BM-050 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Periodically as required |
| Report Format: | Word/PDF |

### BM-051: Annual Capacity charge Exchange Rate

This Report contains the exchange rate between pounds sterling and euro to be applied for a Capacity Year in respect of calculations relating to Capacity Charges.

| I-SEM Report Reference: | BM-051 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Annually and as required |
| Report Format: | Word/PDF |

### BM-052: Capacity Duration Exchange Rate

This report contains the exchange rate between pounds sterling and euro to be applied in respect of calculations relating to amounts calculated based on a Contract Register Entry, n, in accordance with the Capacity Market Code.

| I-SEM Report Reference: | BM-052 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Annually and as required |
| Report Format: | Word/PDF |

### BM-053: Capacity Charge Metered Quantity Factor

This report contains the Capacity Charge Metered Quantity Factor in an Imbalance Settlement Period denoting which Imbalance Settlement Periods the Regulatory Authorities have determined should be used as the basis for Capacity Charge in respect of Supplier Units. It shall have a value of one for periods to be considered, and a value of zero for the periods not to be considered.

| I-SEM Report Reference: | BM-053 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Annually and as required |
| Report Format: | Word/PDF |

### BM-054: Supplier Capacity Charge price

This report contains the Supplier Capacity Charge Price in a Capacity Year, which is used in the calculation of Capacity Charges.

| I-SEM Report Reference: | BM-054 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Annually and as required |
| Report Format: | Word/PDF |

### BM-055: Difference Payment Socialisation Mulitplier

This report contains the Difference Payment Socialisation Multiplier for a Capacity Year, which is used as a multiplier of the Capacity Charge to establish the Difference Payment Socialisation Charge.

| I-SEM Report Reference: | BM-055 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Annually and as required |
| Report Format: | Word/PDF |

### BM-056: Market Price Cap

This report contains the Market Price Cap, reflecting the maximum permitted value for the Imbalance Price for any Imbalance Pricing Period.

| I-SEM Report Reference: | BM-056 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Annually and as required |
| Report Format: | Word/PDF |

### BM-057: Market Price Floor

This report contains the Market Price Floor, reflecting the minimum permitted value for the Imbalance Price for any Imbalance Pricing Period.

| I-SEM Report Reference: | BM-057 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Annually and as required |
| Report Format: | Word/PDF |

### BM-058: Residual meter Volume IntervaL Proportion

This report contains the proportion of the Residual Error Volume Charge to be applied to Supplier Units in Currency Zone in respect of their Interval Metering.

| I-SEM Report Reference: | BM-058 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Annually and as required |
| Report Format: | Word/PDF |

### BM-059: Residual Error Volume Price

This report contains the Residual Error Volume Price in a Year, which is used in the calculation of Residual Error Volume Charges.

| I-SEM Report Reference: | BM-059 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Annually and as required |
| Report Format: | Word/PDF |

### BM-060: Fixed Market Operator Charge (supplier Unit)

This report contains the charge proposed annually by the Market Operator to be applied in respect of each Supplier Unit.

| I-SEM Report Reference: | BM-060 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Annually and as required |
| Report Format: | Word/PDF |

### BM-061: Fixed Market Operator Charge (Generator Unit)

This report contains the charge proposed annually by the Market Operator to be applied in respect of each Generator Unit.

| I-SEM Report Reference: | BM-061 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Annually and as required |
| Report Format: | Word/PDF |

### BM-062: Variable Market Operator Price

This report contains the unit price at which the Variable Market Operator Charge is levied on Participants.

| I-SEM Report Reference: | BM-062 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Annually and as required |
| Report Format: | Word/PDF |

### BM-063: Engineering Tolerance

This report contains the Engineering Tolerance, which is the percentage tolerance between the Dispatch Quantity under a Dispatch Instruction and Actual Output of a Generator Unit, without accounting for frequency deviations, within which the Generator Unit is deemed to be operating in accordance with its Dispatch Instruction, and which is used in the calculation of Uninstructed Imbalances.

| I-SEM Report Reference: | BM-063 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Annually and as required |
| Report Format: | Word/PDF |

### BM-064: MW Tolerance

This report contains the tolerance value in MW within which a Generator Unit is deemed to be complying with its Dispatch Instruction, before consideration of frequency response, which is used in the calculation of Uninstructed Imbalances.

| I-SEM Report Reference: | BM-064 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Annually and as required |
| Report Format: | Word/PDF |

### BM-065: System Per Unit Regulation Factor

This report contains the System Per Unit Regulation Factor, which is the parameter reflecting the automatic response of a generating unit to variations in the system frequency which is used in the calculation of the Tolerance for Over Generation and the Tolerance for Under Generation used in the determination of Uninstructed Imbalance Charges.

| I-SEM Report Reference: | BM-065 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Annually and as required |
| Report Format: | Word/PDF |

### BM-066: DOG/PUG/TOLOG/TOLUG for Over/Under Generation

Refer to section 8.3.19 BM-103 Energy Market Financial Publication – Indicative

| I-SEM Report Reference: | BM-066 |
| --- | --- |
| Data Source | CSB Settlement System |
| Periodicity: | Daily |
| File Names: | GP_GP_ALL_<SettlementDay>_<PublicationDay>_<Market>_<RunType>_<Timestamp> |
| Report Title: | DOG/PUG/TOLOG/TOLUG for Over/Under Generation |
| Audience: | General Public |
| Resolution: | Imbalance Settlement Period |
| Time Span: | Market Volume and Settlement by Settlement Period |
| Frequency: | Settlement Day |
| Report Format: | XML |

| Field Name | Format | Description |
| --- | --- | --- |
| REPORT_TYPE | VARCHAR2(2) | Identifier for report (GP) |
| MARKET_NAME | VARCHAR2(6) | Market name (BALIMB) |
| SETTLEMENT_DATE | DATE (YYYY-MM-DD) | Calendar day of settlement |
| PUBLICATION_DATE | DATE (YYYY-MM-DD) | Calendar day for publication |
| PUBLICATION_TIMESTAMP | TIME(YYYY-MM-DDTH24:MI) | Timestamp of publication |
| RUN_TYPE | VARCHAR2(5) | Type of run (INDIC, INIT, M4, M13, AH) |

Table 50: Under/Over Generation Report Header Field Description

| Field Name | Format | Description |
| --- | --- | --- |

| RESOURCE NAME | VARCHAR2(32) | Name of the resource. Only populated if applicable. |
| --- | --- | --- |
| DETERMINANT NAME | VARCHAR2(16) | Name of the determinant |
| DETERMINANT UNIT | VARCHAR2(8) | Units for determinant (MW, MWh, etc.) |
| VALUE: DATETIME | TIME(YYYY-MM-DDTH24:MI) | Date and time of the ISP for this determinant and resource |
| VALUE AMOUNT | number(28,8) | Value of the determinant for this resource in this ISP. |

Table 51: Under/Over Generation Report Field Description

### BM-067: Transmission Loss Adjustment Factor

This report contains the factor for each Unit in each Imbalance Settlement Period to adjust the Output or Demand of that Unit for the effect of Transmission Losses and as otherwise provided for in the Trading and Settlement Code.

| I-SEM Report Reference: | BM-067 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Annually and as required |
| Report Format: | Word/PDF |

### BM-068: Distribution Loss Adjustment Factor

This report contains the factor for each Unit in each Imbalance Settlement Period to adjust the Output or Demand of that Unit for the effect of Distribution Losses and as otherwise provided for in the Trading and Settlement Code.

| I-SEM Report Reference: | BM-068 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Annually and as required |
| Report Format: | Word/PDF |

### BM-069: Combined Loss Adjustment Factor

This report contains the factor for each Unit in each Imbalance Settlement Period, as calculated by the System Operators, to adjust the Output or Demand of that Unit for the effect of Transmission Losses and Distribution Losses and as otherwise provided for in the Trading and Settlement Code.

| I-SEM Report Reference: | BM-069 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Annually and as required |
| Report Format: | Word/PDF |

### BM-070: Imperfections Price

This report contains the Imperfections Charge for a Year used in the calculation of Imperfections Charges.

| I-SEM Report Reference: | BM-070 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Annually and as required |
| Report Format: | Word/PDF |

### BM-071: Imperfections charge Factor

This report contains the Imperfections Charge Factor for a Year.

| I-SEM Report Reference: | BM-071 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Annually and as required |
| Report Format: | Word/PDF |

### BM-072: Testing Tariff

This report contains the testing tariffs applicable to Generator Units Under Test.

| I-SEM Report Reference: | BM-072 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Annually and as required |
| Report Format: | Word/PDF |

### BM-073: Schedule of Testing Tariffs

This report will contain the schedule of Testing Tariffs for a Year.

| I-SEM Report Reference: | BM-073 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Annually and as required |
| Report Format: | Word/PDF |

### BM-074: Fixed Credit Requirement, in respect of Generator and/or Supplier Units

This report contains the minimum Credit Cover requirement for any Participant in respect of each of its Generator Units and/or in respect of each of its Supplier Units, as approved by the Regulatory Authorities.

| I-SEM Report Reference: | BM-074 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Annually and as required |
| Report Format: | Word/PDF |

### BM-075: Days in Historical Assessment Period

This report contains the number of days (Historical Assessment Period) prior to the day of the issue of the latest relevant Settlement Document over which a statistical analysis of a Participant’s incurred liabilities, separately in respect of its Generator Units and Supplier Units, shall be undertaken in order to support the forecasting of undefined liabilities for that Participant.

| I-SEM Report Reference: | BM-075 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Annually and as required |
| Report Format: | Word/PDF |

### BM-076: Analysis Percentile Parameter

This report contains the percentage degree of statistical confidence that Actual Exposures, once determined for each Participant, will fall below the estimate of Undefined Potential Exposure, as approved by the Regulatory Authorities.

| I-SEM Report Reference: | BM-076 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Annually and as required |
| Report Format: | Word/PDF |

### BM-077: Credit Cover Adjustment Trigger

This report contains the trigger value for determining when a Participant should report to the Market Operator expected future changes in the total metered quantities of its Supplier Units or settlement quantities of its Generator Units such that it should be designated an Adjusted Participant, as approved by the Regulatory Authorities.

| I-SEM Report Reference: | BM-077 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Annually and as required |
| Report Format: | Word/PDF |

### BM-078: Currency Cost Price

This report contains the Currency Cost Price in a Year for use in the calculation of Currency Adjustment Charges.

| I-SEM Report Reference: | BM-078 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Annually and as required |
| Report Format: | Word/PDF |

### BM-079: Currency Cost Change Factor

This report contains the Currency Cost Charge Factor in a Year, as approved by the Regulatory Authorities.

| I-SEM Report Reference: | BM-079 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Annually and as required |
| Report Format: | Word/PDF |

### BM-080: Annual Maintenance Schedule – Transmission Line Outage

This report contains the Annual Maintenance Schedule for Transmission Line Outage.

| I-SEM Report Reference: | BM-080 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Annually, in April of each year |
| Report Format: | XLS |

### BM-081: Two Year Maintenaince Schedule – Generator outages Schedule

This report contains the Two Year Maintenance Schedule for Generator Outages.

| I-SEM Report Reference: | BM-081 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Annually and as required |
| Report Format: | XLS |

### BM-082: Monthly Maintenance Schedule – Generator Unit Outages

This report contains the Monthly Maintenance Schedule for Generator Unit Outages.

| I-SEM Report Reference: | BM-082 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Monthly |
| Report Format: | XLS |

### BM-083: Monthly Maintenance Schedule – Transmission System Line Outages

This report contains the Monthly Maintenance Schedule for Transmission System Line Outages.

| I-SEM Report Reference: | BM-083 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Monthly |
| Report Format: | XLS |

### BM-084: Daily Trading Exchange Rate Report

This report contains the Daily Trading Exchange Rate, retrieved two days prior to the Trading Day.

| I-SEM Report Reference: | BM-084 |
| --- | --- |
| Data Source | Market Operator |
| Periodicity: | Daily |
| Report Name: | PUB_DailyExchangeRate |
| File Names: | PUB_DailyExchangeRate.xml |
| Report Title: | Daily Trading Exchange Rate Report |
| Audience: | General Public |
| Resolution: | Trading Day |
| Time Span: | Trading Day + 2 |
| Frequency: | Daily |
| Report Format: | XML |

| Field Name | Format | Description |
| --- | --- | --- |
| TRADE DATE | DATE (YYYY-MM-DD) | Trading Day |
| FROM CURRENCY | CHAR(1) | From Currency - Euro (E) or Sterling Pound (P). |
| TO CURRENCY | CHAR(1) | To Currency - Euro (E) or Sterling Pound (P). |
| EXCHANGE RATE | NUMBER(4,2) | Exchange Rate |

Table 52: Daily Trading Day Exchange Rate Report

### BM-162: LTS Operational Schedule Report

This report contains the LTS Operational Schedules all Generator Units and Demand Side Units.

| I-SEM Report Reference: | BM-162 |
| --- | --- |
| Data Source | TSO |
| Periodicity: | Daily |
| Report Name: | PUB_LTSDOperationalSchedule |
| File Names: | PUB_LTSDOperationalSchedule_YYYYMMDDHHMM.xml |
| Report Title: | LTS Operational Schedule Report |
| Audience: | General Public |
| Resolution: | Half Hourly |
| Time Span: | Optimisation Time Horizon of all LTS runs |
| Frequency: | Automatically, after approval of each LTS Operational Schedule Run |
| Format: | XML |

| Field Name | Format | Description |
| --- | --- | --- |
| PUBLISH TIME | TIME(YYYY-MM-DDTH24:MI) | Timestamp corresponding to the approved LTS run |
| PARTICIPANT NAME | VARCHAR2(12) | Participant Name |
| RESOURCE NAME | VARCHAR2(32) | Name of Generating Unit or Demand Side Unit |
| RESOURCE TYPE | VARCHAR2(4) | Type of Resource |
| TRADE DATE | DATE (YYYY-MM-DD) | Trading Day |
| DELIVERY DATE | DATE (YYYY-MM-DD) | Calendar Day (referred to as “Day” in the Code). |
| START TIME | TIME(YYYY-MM-DDTH24:MI) | Start Date/Time |
| END TIME | TIME(YYYY-MM-DDTH24:MI) | End Date/Time |
| SCHEDULED QUANTITY | NUMBER(8,3) | Operational Schedule Quantity (MW) |
| RUN TYPE | CHAR( 4) | LTSD |

Table 53: LTS Operational Schedule Report

Figure 2: BM-162: LTS Operational Schedule Report – Sample

### BM-163: RTIC Operational Schedule Report

The report contains the RTIC Operational Schedules for all Generator Units and Demand Side Units.

| I-SEM Report Reference: | BM-163 |
| --- | --- |
| Data Source | TSO |
| Periodicity: | Daily |
| Report Name: | PUB_RTCOperationalSchedule |
| File Names: | PUB_RTCOperationalSchedule_YYYYMMDDHHMM.xml |
| Report Title: | RTIC Operational Schedule Report |
| Audience: | General Public |
| Resolution: | 15 mins |
| Time Span: | Optimisation Time Horizon for all RTC Operational Schedule Runs |
| Frequency: | Automatically, after approval of each RTC Operational Schedule |
| Format: | XML |

| Field Name | Format | Description |
| --- | --- | --- |
| PUBLISH TIME | TIME(YYYY-MM-DDTH24:MI) | Timestamp corresponding to the approved RTC run |
| PARTICIPANT NAME | VARCHAR2(12) | The PT Identifier, which represents the name of Market Participant, as registered in the Balancing Market. |
| RESOURCE NAME | VARCHAR2(32) | Name of Generating Unit or Demand side Unit |
| RESOURCE TYPE | VARCHAR2(4) | Type of Resource |
| TRADE DATE | DATE (YYYY-MM-DD) | Trading Day |
| DELIVERY DATE | DATE (YYYY-MM-DD) | Calendar Day (referred to as “Day” in the Code). |
| START TIME | TIME(YYYY-MM-DDTH24:MI) | Start Date/Time |
| END TIME | TIME(YYYY-MM-DDTH24:MI) | End Date/Time |
| SCHEDULED QUANTITY | NUMBER(8,3) | Operational Schedule Quantity (MW) |
| RUN TYPE | CHAR(3) | RTC |

Table 54: BM-163: RTIC Operational Schedule Report

Figure 3: BM-163: : RTIC Operational Schedule Report – Sample

## Settlement Data

### BM-085: Settlement Calendar

This report is available to all Participants and the general public. Each entry in the settlement calendar has a Calendar Event describing an expected event such as the availability of settlement reports or payment timelines.

| I-SEM Report Reference: | BM-085 |
| --- | --- |
| Data Source | CSB Settlement System |
| Periodicity: | Annually Set, updated ad hoc as required |
| File Names: | SC_PT_ALL_<SettlementDay/OperatingDay>_<PublicationDay>_<ALL_ REPT_<Timestamp> (see note below) |
| Report Title: | Settlement Calendar |
| Audience: | General Public |
| Resolution: | Event by Calendar Day |
| Time Span: | Annually, updated ad hoc as required |
| Frequency: | Annually set, updated as necessary |
| Report Format: | XML |

| Field Name | Format | Description |
| --- | --- | --- |
| REPORT_TYPE | VARCHAR2(2) | Identifier for report (SC) |
| PUBLICATION_TIMESTAMP | TIME(YYYY-MM-DDTH24:MI) | Timestamp of publication |

Table 55: Settlement Calendar Header Field Description

| Field Name | Format | Description |
| --- | --- | --- |
| OPERATIONAL DATE | DATE (YYYY-MM-DD) | This is the date the run defined by the calendar entry occurs on, and is the default sort key for the calendar. |
| MARKET | VARCHAR2(6) | This describes the primary separation of settlements, and is expected to have the following three values: - Imbalance (BALIMB) - Capacity Market (CRM) - MO - Fixed Market Operator Charge(FMOC) |
| PUBLICATION | VARCHAR2(24) | This field contains those calendar events marked as Publish, and describes the type of document produced by the run for this calendar entry. Following the SEM settlement calendar, the Publication field is expected to have the following values: - Statements - Settlement Documents - Payments IN - Payments OUT - MDP submission timelines |
| RUN_TYPE | VARCHAR2(5) | This field describes the run identifier. Following the SEM settlement calendar, the Run field is expected to have the following values: - Indicative (INDIC) - Initial (INIT) - M4 (M4) - M13 (M13) - Ad hoc (AH) |
| PERIOD_TYPE | VARCHAR2(24) | This field defines the number of days that make up the particular run. In I-SEM, Indicative and Initial runs for Statements are anticipated to have a Period Type of Day and otherwise a Period Type of Week, while all runs applying to Settlement Documents are anticipated to have a Period Type of Week. Although it is currently anticipated to follow the same timeline as the Imbalance market, the Capacity Market may require a Month period type. |
| PERIOD_END_DATE | TIME (YYY-MM-DDTH24:MI)) | This field defines the end of the last day in the settlement period defined by the Period Type. |
| MODIFIED | DATE (YYYY-MM-DD) | This value of this field is the t last modification date of the calendar entry. Here, “modification” includes both a new entry and an updated entry. |
| COMMENTS | VARCHAR2(512) | This field is a free-form text entry. The current SEM settlement calendar uses comments such as “Payments in by 12:00”, “Published by 17:00”, and “Week 44 2015”. |

Table 56: Settlement Calendar Field Description

### BM-086: Daily Meter Data Report

This report contains metered generation (MWh) for each Generator Unit and Interconnectors per Imbalance Settlement Period, on a Settlement Day basis. This report is published D+1 and D+4.

| I-SEM Report Reference: | BM-086 |
| --- | --- |
| Data Source | Meter Data Provider |
| Periodicity: | Daily |
| Report Name: | PUB_DailyMeterDataD1 PUB_DailyMeterDataD5 |
| File Names: | PUB_DailyMeterDataD1.xml PUB_DailyMeterDataD5.xml |
| Report Title: | Daily Meter Data Report |
| Audience: | General Public |
| Resolution: | Imbalance Settlement Period |
| Time Span: | Calendar Day (TD-1WD) and (TD-5WD) |
| Frequency: | Daily |
| Report Format: | XML |

| Field Name | Format | Description |
| --- | --- | --- |
| TRADE DATE | DATE (YYYY-MM-DD) | Trading Day commences at 23:00hrs (GMT) |
| PARTICIPANT NAME | VARCHAR2(12) | The PT Identifier, which represents the name of Market Participant, as registered in the Balancing Market. |
| RESOURCE NAME | VARCHAR2(32) | The name of the resource (e.g. the name of the Generating Unit, Interconnector for which data is being reported). |
| RESOURCE TYPE | VARCHAR2(4) | Indicates the type of resource for which data is being submitted - permitted values include: GU, Interconnector. |
| START TIME | TIME(YYYY-MM-DDTH24:MI) | Imbalance Settlement Period Start Time |
| END TIME | TIME(YYYY-MM-DDTH24:MI) | Imbalance Settlement Period End Time |
| JURISDICTION | VARCHAR2(32) | ROI (Republic of Ireland) or NI (Northern Ireland) |
| METERED_MW | NUMBER(8,3) | Represents the total generation (MWh) |

Table 57: Daily Meter Data Report

### BM-087: Initial Interconnector Flows and Residual Capacity Report

This report contains the initial metered Interconnector flow and the difference between metered and scheduled interconnector flow per Imbalance Settlement Period for the settlement day.

| I-SEM Report Reference: | BM-087 |
| --- | --- |
| Data Source | Settlement |
| Periodicity: | Daily |
| Report Name: | PUB_DailyIntconFlowAndResCap |
| File Names: | PUB_DailyIntconFlowAndResCap.xml |
| Report Title: | Initial Interconnector Flows and Residual Capacity Report |
| Audience: | General Public |
| Resolution: | Imbalance Settlement Period |
| Time Span: | Settlement Day-4 |
| Frequency: | Daily (every Working Day as Meter Data is received per Working Day) |
| Report Format: | XML |

| Field Name | Format | Description |
| --- | --- | --- |
| TRADE DATE | DATE (YYYY-MM-DD) | Trading Day |
| RESOURCE NAME | VARCHAR2(32) | The name of the resource (e.g. the name of the Generating Unit, Interconnector for which data is being reported). |
| START TIME | TIME(YYYY-MM-DDTH24:MI) | Imbalance Settlement Period Start Time |
| END TIME | TIME(YYYY-MM-DDTH24:MI) | Imbalance Settlement Period End Time |
| METERED FLOW | NUMBER(8,3) | Initial Flow MW |
| FLOW VARIANCE | NUMBER(8,3) | Difference between Indicative and Initial or scheduled flow |
| RESIDUAL CAPACITY | NUMBER(8,3) | Residual Capacity (MW) |

Table 58: Initial Interconnector Flows and Residual Capacity Report

### BM-088: Daily Transmission Outage Schedule Report

This report contains transmission outage schedules for all Units, where applicable, for the next Trading Day.

| I-SEM Report Reference: | BM-088 |
| --- | --- |
| Data Source | TSO |
| Periodicity: | Daily |
| Report Name: | PUB_DailyTransOutageSchedule |
| File Names: | PUB_DailyTransOutageSchedule.xml |
| Report Title: | Daily Transmission Outage Schedule Report |
| Audience: | General Public |
| Resolution: | Per outage durations |
| Time Span: | Next Trading Day |
| Frequency: | Daily |
| Report Format: | XML |

| Field Name | Format | Description |
| --- | --- | --- |
| EQUIPMENT TYPE | VARCHAR2(6) | Equipment Type |
| EQUIPMENT ID | VARCHAR2(36) | Equipment identifier |
| EQUIPMENT NAME | VARCHAR2(32) | Equipment Name |
| OUTAGE REASON FLAG | CHAR(1) | Outage Reason Flag: P – Planned F- Forced C - Consequential |
| TRADE DATE | DATE (YYYY-MM-DD) | Trading Day |
| DELIVERY DATE | DATE (YYYY-MM-DD) | Calendar Day (referred to as “Day” in the Code). |
| START TIME | TIME(YYYY-MM-DDTH24:MI) | Outage Start Date/Time |
| END TIME | TIME(YYYY-MM-DDTH24:MI) | Outage End Date/Time |

Table 59: Daily Transmission Outage Schedule Report

### BM-089: Average System Frequency Report

This report contains the nominal and calculated average System Frequency by Imbalance Settlement Period for the relevant Trading Day.

| I-SEM Report Reference: | BM-089 |
| --- | --- |
| Periodicity: | Daily |
| Report Name: | PUB_AvgSystemFrequency |
| File Names: | PUB_AvgSystemFrequency.xml |
| Report Title: | Average System Frequency Report |
| Audience: | General Public |
| Resolution: | Imbalance Settlement Period |
| Time Span: | Calendar Day - 1 |
| Frequency: | Daily |
| Report Format: | XML |

| Field Name | Format | Description |
| --- | --- | --- |
| TRADE DATE | DATE (YYYY-MM-DD) | Trading Date |
| DELIVERY DATE | DATE (YYYY-MM-DD) | Calendar Day (referred to as “Day” in the Code). |
| START TIME | TIME(YYYY-MM-DDTH24:MI) | Start time |
| END TIME | TIME(YYYY-MM-DDTH24:MI) | End time |
| NOMINAL FREQUENCY | NUMBER(6,3) | Nominal Frequency |
| AVERAGE FREQUENCY | NUMBER(6,3) | Average Frequency |

Table 60: Average System Frequency Report

### BM-090: SO Interconnector Trades Report

This daily report contains Interconnector Trades for the previous Calendar Day, including interconnector import/export prices and associated MW quantities.

| I-SEM Report Reference: | BM-090 |
| --- | --- |
| Periodicity: | Daily |
| Report Name: | PUB_DailyIntconTrades |
| File Names: | PUB_DailyIntconTrades.xml |
| Report Title: | SO Interconnector Trades Report |
| Audience: | General Public |
| Resolution: | Imbalance Settlement Period |
| Time Span: | Settlement Day – 1 |
| Frequency: | Daily |
| Report Format: | XML |

| Field Name | Format | Description |
| --- | --- | --- |
| SETTLEMENT DATE | DATE (YYYY-MM-DD) | Settlement Date |
| RESOURCE NAME | VARCHAR2(32) | The name of the resource (e.g. the name of the Generating Unit, Supplier Unit, Demand Side Unit, Interconnector for which data is being reported). |
| DELIVERY DATE | DATE (YYYY-MM-DD) | Calendar Day (referred to as “Day” in the Code). |
| START TIME | TIME(YYYY-MM-DDTH24:MI) | Imbalance Settlement Period Start time |
| END TIME | TIME(YYYY-MM-DDTH24:MI) | Imbalance Settlement Period End time |
| SO INTERCON IMP PRICE | Number(8,2) | Import Price for SO Trades |
| SO INTERCON IMP QUANTITY | Number(8,3) | Import Quantity for SO Trades |
| SO INTERCON EXP PRICE | Number(8,2) | Export Price for SO Trades |
| SO INTERCON EXP QUANTITY | Number(8,3) | Export Quantity for SO Trades |

Table 61: SO Interconnector Trades Report

### BM-091: Unit Data Report

This report contains Generator Unit and Demand Side Unit Fuel Type for the previous Trading Day.

| I-SEM Report Reference: | BM-091 |
| --- | --- |
| Periodicity: | Daily |
| Report Name: | PUB_DailyUnitData |
| File Names: | PUB_DailyUnitData.xml |
| Report Title: | Unit Data Report |
| Audience: | General Public |
| Resolution: | Trading Day |
| Time Span: | Trading Day – 1 |
| Frequency: | Daily |
| Report Format: | XML |

| Field Name | Format | Description |
| --- | --- | --- |
| JURISDICTION | VARCHAR2(32) | ROI (Republic of Ireland) or NI (Northern Ireland) |
| TRADE DATE | DATE (YYYY-MM-DD) | Trading Day |
| PARTICIPANT FULLNAME | VARCHAR2(60) | Full Name of the Participant |
| PARTICIPANT NAME | VARCHAR2(12) | Participant name |
| RESOURCE NAME | VARCHAR2(32) | The name of the resource (e.g. the name of the Generating Unit, Supplier Unit, Demand Side Unit, Interconnector for which data is being reported). |
| RESOURCE TYPE | VARCHAR2(4) | Type of resource |
| FUEL TYPE | VARCHAR2(22) | Fuel type. Possible values are: Oil Gas Peat Wind Biomass CHP Hydro Coal Distillate Nuclear Battery Storage Multi Fuel Pump Storage Fly Wheel Solar Compressed Air Storage Other Synchronous Condenser |

Table 62: Unit Data Report

### BM-094: Metered Volumes by Jurisdiction

This report contains total metered generation and demand volumes, per jurisdiction and Trading Period.

| I-SEM Report Reference: | BM-094 |
| --- | --- |
| Data Source | System |
| Periodicity | Daily |
| File Names: | MJ_GP_ALL_<SettlementDay>_<PublicationDay>_<Market>_<RunType> _<Timestamp>.xml |
| Report Title: | Metered Volumes By Jurisdiction |
| Audience: | General Public |
| Resolution: | Imbalance Settlement Period |
| Time Span: | Settlement Day |
| Frequency: | Daily - 1WD D-5WD (M+4 5WD), (M+13 5WD) and (Ad-hoc 5WD) |
| Report Format: | XML |

| Field Name | Format | Description |
| --- | --- | --- |
| REPORT_TYPE | VARCHAR2(6) | Identifier for report (MJ) |
| MARKET_NAME | VARCHAR2(6) | Market name (BALIMB) |
| PUBLICATION_DATE | DATE (YYYY-MM-DD) | Calendar day for publication |
| PUBLICATION_TIMESTAMP | TIME(YYYY-MM-DDTH24:MI) | Timestamp of publication |
| RUN_TYPE | VARCHAR2(5) | Type of run (INDIC, INIT, M4, M13, AH) |
| SETTLEMENT_DATE | DATE (YYYY-MM-DD) | Settlement Date |

Table 63: Metered Volumes by Jurisdiction Report Header Field Description

| Field Name | Format | Description |
| --- | --- | --- |
| VALUE: DATETIME | TIME(YYYY-MM-DDTH24:MI) | Date and time of the ISP |
| VALUE: METERED_GENERATION | NUMBER(28,8) | Metered Generation |
| VALUE: METERED_DEMAND | NUMBER(28,8) | Metered Demand |
| JURISDICTION | VARCHAR2(3) | ROI (Republic of Ireland) or NI (Northern Ireland) |
| VALUE: UNIT | VARCHAR2(8) | Units of measure (e.g. MW) |

Table 64: Metered Volumes by Jurisdiction Report

### BM-095: Balancing and Imbalance Market Cost View Report

This report contains the total Settlement Period Imbalance Volume (MWh) times the Imbalance Period Price. It is available for all the Participants and is run at half hourly intervals every day for the previous Imbalance Settlement Period.

| I-SEM Report Reference: | BM-094 |
| --- | --- |
| Data Source | Balancing Market Runs |
| Periodicity: | Daily |
| Report Name: | PUB_30MinImbalCost |
| File Names: | PUB_30MinImbalCost_YYYYMMDDHHMM.xml |
| Report Title: | Balancing and Imbalance Market Cost View Report |
| Audience: | General Public |
| Resolution: | Imbalance Settlement Period |
| Time Span: | 30 minutes |
| Frequency: | Half hourly intervals every day |
| Report Format: | XML |

| Field Name | Format | Description |
| --- | --- | --- |

| START TIME | TIME(YYYY-MM-DDTH24:MI) | Imbalance Settlement Period Start Time |
| --- | --- | --- |
| END TIME | TIME(YYYY-MM-DDTH24:MI) | Imbalance Settlement Period End Time |
| IMBALANCE VOLUME | NUMBER(8,3) | Quantity of Net Imbalance volume (QNIV) for the Imbalance Settlement Period |
| IMBALANCE PRICE | NUMBER(8,2) | Calculated Average time weighted imbalance price (PIMB) for the Imbalance Settlement Period |
| IMBALANCE COST | NUMBER(8,2) | Imbalance Cost (Euro) = Settlement Period Imbalance Volume (MWhr) times the Imbalance Period Price (Euro/MWhr) |

Table 65: Balance and Imbalance Market Cost View Field Description

### BM-096: Dispatch Quantity

This report contains the calculated Dispatch Quantity per Unit and Trading Period.

| I-SEM Report Reference: | BM-096 |
| --- | --- |
| Data Source | TSO |
| Periodicity | Daily |
| Report Name: | PUB_DlyDispatchQuantity |
| File Names: | PUB_DlyDispatchQuantity.xml |
| Report Title: | Dispatch Quantity |
| Audience: | General Public |
| Resolution: | Imbalance Settlement Period |
| Time Span: | Previous Settlement Day |
| Frequency: | Daily |
| Report Format: | XML |

| Field Name | Format | Description |
| --- | --- | --- |
| TRADE DATE | DATE (YYYY-MM-DD) | Trading Date |
| DELIVERY DATE | DATE (YYYY-MM-DD) | Calendar Day (referred to as “Day” in the Code). (YYYY-MM-DD) |
| START TIME | TIME(YYYY-MM-DDTH24:MI) | Start Time of Imbalance Settlement Period |
| END TIME | TIME(YYYY-MM-DDTH24:MI) | End Time of Imbalance Settlement Period |
| RESOURCE NAME | VARCHAR2(32) | Resource Name |
| DISPATCH QUANTITY | NUMBER(8,3) | Dispatch Quantity (MW) |

Table 66: Dispatch Quantity

### BM-098: Aggregated Contracted Quantities for Generation Report

This report contains the aggregated contracted quantities as provided by the NEMO(s) for each Imbalance Settlement Period for which the Balancing Market Gate Window is still open in the Trading Day, aggregated across all Generator Units and Demand Side Units.

| I-SEM Report Reference: | BM-098 |
| --- | --- |
| Data Source | NEMO(s) |
| Periodicity: | Daily |
| Report Name: | PUB_HrlyAggContractedQtyGen |
| File Names: | PUB_HrlyAggContractedQtyGen_YYYYMMDDHHMM.xml |
| Report Title: | Aggregated Contracted Quantities for Generation Report |
| Audience: | General Public |
| Resolution: | Imbalance Settlement Period |
| Time Span: | All Imbalance Settlement Periods for which the Balancing Market Gate Window is still open in the Trading Day |
| Frequency: | Once every Hour |
| Report Format: | XML |

| Field Name | Format | Description |
| --- | --- | --- |
| TRADE DATE | DATE (YYYY-MM-DD) | Trading Day |
| DELIVERY DATE | DATE (YYYY-MM-DD) | Calendar Day (referred to as “Day” in the Code). |
| START TIME | TIME(YYYY-MM-DDTH24:MI) | Imbalance Settlement Period Start Time |
| END TIME | TIME(YYYY-MM-DDTH24:MI) | Imbalance Settlement Period End time |
| MW | NUMBER(11,3) | Aggregated Contracted Quantities for Generation Units and Demand Side Units only |

Table 67: Aggregated Contracted Quantities for Generation Report

### BM-099: Aggregated Contracted Quantities for Demand Report

This report contains the aggregated contracted quantities submitted by the NEMO(s) for Imbalance Settlement for each Imbalance Settlement Period for which the Balancing Market Gate Window is still open in the Trading Day, aggregated across all Supplier Units.

| I-SEM Report Reference: | BM-099 |
| --- | --- |
| Data Source | NEMO(s) |
| Periodicity: | Daily |
| Report Name: | PUB_AggContractedQtyDemand |
| File Names: | PUB_AggContractedQtyDemand_YYYYMMDDHHMM.xml |
| Report Title: | Aggregated Contracted Quantities for Demand Report |
| Audience: | General Public |
| Resolution: | Imbalance Settlement Period |
| Time Span: | All Imbalance Settlement Periods for which the Balancing Market Gate Window is still open in the Trading Day |
| Frequency: | Every Hour |
| Report Format: | XML |

| Field Name | Format | Description |
| --- | --- | --- |
| TRADE DATE | DATE (YYYY-MM-DD) | Trading Day |
| DELIVERY DATE | DATE (YYYY-MM-DD) | Calendar Day (referred to as “Day” in the Code). |
| START TIME | TIME(YYYY-MM-DDTH24:MI) | Imbalance Settlement Period Start Time |
| END TIME | TIME(YYYY-MM-DDTH24:MI) | Imbalance Settlement Period End time |
| MW | NUMBER(11,3) | Aggregated Contracted Quantities for Supplier Units |

Table 68: Aggregated Contracted Quantities for Demand Report

### BM-100: Aggregated Contracted Quantities for Wind Report

This report contains the aggregated contracted quantities submitted by the NEMO(s) for Imbalance Settlement for each Imbalance Settlement Period for which the Balancing Market Gate Window is still open in the Trading Day, aggregated across all Wind Units.

| I-SEM Report Reference: | BM-100 |
| --- | --- |
| Data Source | NEMO(s) |
| Periodicity: | Daily |
| Report Name: | PUB_HrlyAggWindFcst |
| File Names: | PUB_HrlyAggWindFcst_YYYYMMDDHHMM.xml |
| Report Title: | Aggregated Contracted Quantities for Wind Report |
| Audience: | General Public |
| Resolution: | Imbalance Settlement Period |
| Time Span: | All Imbalance Settlement Periods for which the Balancing Market Gate Window is still open in the Trading Day |
| Frequency: | Hourly |
| Report Format: | XML |

| Field Name | Format | Description |
| --- | --- | --- |
| TRADE DATE | DATE (YYYY-MM-DD) | Trading Day |
| DELIVERY DATE | DATE (YYYY-MM-DD) | Calendar Day (referred to as “Day” in the Code). |
| START TIME | TIME(YYYY-MM-DDTH24:MI) | Imbalance Settlement Period Start Time |
| END TIME | TIME(YYYY-MM-DDTH24:MI) | Imbalance Settlement Period End time |
| MW | NUMBER(11,3) | Aggregated Contracted Quantities for Wind Units |

Table 69: Aggregated Contracted Quantities for Wind Report

### BM-101: Average Outturn Availability

This report contains the calculated average Outturn Availability per Unit and Trading Period.

| I-SEM Report Reference: | BM-101 |
| --- | --- |
| Data Source | TSO |
| Periodicity | Daily |
| Report Name: | PUB_AvgOutturnAvail |
| File Names: | PUB_AvgOutturnAvail_YYYYMMDDHHMM.xml |
| Report Title: | Average Outturn Availability |
| Audience: | General Public |
| Resolution: | Imbalance Settlement Period |
| Time Span: | Calendar Day – 1 |
| Frequency: | Daily |
| Report Format: | XML |

| Field Name | Format | Description |
| --- | --- | --- |
| TRADE DATE | DATE (YYYY-MM-DD) | Trading Day |
| DELIVERY DATE | DATE (YYYY-MM-DD) | Calendar Day (referred to as “Day” in the Code). (YYYY-MM-DD) |
| START TIME | TIME(YYYY-MM-DDTH24:MI) | Start Time of Imbalance Settlement Period |
| END TIME | TIME(YYYY-MM-DDTH24:MI) | End Time of Imbalance Settlement Period |
| PARTICIPANT_NAME | VARCHAR2(12) | Participant name |
| RESOURCE_NAME | VARCHAR2(32) | The name of the resource (e.g. the name of the Generating Unit, Supplier Unit, Demand Side Unit, Interconnector for which data is being reported). |
| AVG_OUTTURN_AVAIL | NUMBER(8,3) | Outturn Availability |

Table 70: Average Outturn Availability Report

### BM-159: Outturn Availability

This report contains XXX.

| I-SEM Report Reference: | BM-159 |
| --- | --- |
| Data Source | TSO |
| Periodicity | Hourly |
| Report Name: | PUB_D_OutturnAvail |
| File Names: | PUB_D_OutturnAvail_YYYYMMDDHHMM.xml |
| Report Title: | Outturn Availability |
| Audience: | General Public |
| Resolution: | Point in time |
| Time Span: | Whole hour preceding the time at which the report is produced |
| Frequency: | Hourly |
| Report Format: | XML |

| Field Name | Format | Description |
| --- | --- | --- |
| TRADE DATE | DATE (YYYY-MM-DD) | Trade Date |
| DELIVERY DATE | DATE (YYYY-MM-DD) | Delivery Date |
| ISSUE TIME | TIME(YYYY-MM-DDTH24:MI) | Issue time for the availability declaration |
| PARTICIPANT NAME | VARCHAR2(12) | Participant Name |
| RESOURCE NAME | VARCHAR2(32) | Resource Name |
| outturn AVAILability | NUMBER(8,3) | Maximum Availability (MW) |
| outturn minimum stable gen | NUMBER(8,3) | Minimum Stable Generation (MW) |
| outturn Minimum Output | NUMBER(8,3) | Minimum Output (MW) |

Table 71: Outturn Availability Report

### BM-160: Hourly Dispatch Instructions Report

This report is published on an hourly basis, containing all Accepted Dispatch Instructions (including Wind Units) where the Issue Time is in the previous hour.

| I-SEM Report Reference: | BM_160 |
| --- | --- |
| Data Source | TSO |
| Periodicity: | Hourly |
| Report Name: | PUB_D_HourlyDIRpt |
| File Names: | PUB_D_ HourlyDIRpt_YYYYMMDDHHMM.xml |
| Report Title: | Hourly Dispatch Instructions Report |
| Audience: | General Public |
| Resolution: | Point in time |
| Time Span: | Whole hour preceding the time at which the report is produced |
| Frequency: | Hourly |
| Report Format: | XML |

| Field Name | Format | Description |
| --- | --- | --- |
| INSTRUCTION TIME STAMP | TIME(YYYY-MM-DDTH24:MI) | Instruction Time Stamp |
| INSTRUCTION END TIME | TIME(YYYY-MM-DDTH24:MI) | End time of the instruction |
| PARTICIPANT NAME | VARCHAR2(12) | Participant Name |
| RESOURCE NAME | VARCHAR2(32) | Resource Name |
| DISPATCH INSTRUCTION | NUMBER(8,3) | Dispatch Instruction |
| INSTRUCTION CODE | VARCHAR2(4) | Instruction Code |
| INSTRUCTION COMBINATION CODE | VARCHAR2(4) | Instruction Combination Code |
| INSTRUCTION ISSUE TIME | TIME(YYYY-MM-DDTH24:MI) | Instruction Issue Time |

Table 72: Hourly Dispatch Instructions Report

### BM-161: Hourly SO Interconnector Trades Report

This report is available to all the Market Participants and general public. It is produced each hour for the previous hour, containing all SO Interconnector Trades for which the start time is less than or equal to the end of the hour for which the report is being published, or where the end time is greater than or equal to the start of the hour for which the report is being produced. The report includes interconnector import/export prices and trade MW quantities. The report will not be published when there are no applicable SO Interconnector Trades for the previous hour.

| I-SEM Report Reference: | BM_161 |
| --- | --- |
| Data Source | TSO |
| Periodicity: | Hourly |
| Report Name: | PUB_D_HourlySOTrades |
| File Names: | PUB_D_ HourlySOTrades_YYYYMMDDHHMM.xml |
| Report Title: | Hourly SO Interconnector Trades Report |
| Audience: | General Public |
| Resolution: | Individual SO Interconnector Trades |
| Time Span: | Whole hour preceding the time at which the report is produced |
| Frequency: | Hourly |
| Report Format: | XML |

| Field Name | Format | Description |
| --- | --- | --- |
| TRADE DATE | DATE(YYYY-MM-DD) | Trading Date |
| RESOURCE NAME | VARCHAR2(32) | Resource Name |
| DELIVERY DATE | DATE(YYYY-MM-DD) | Delivery Date |
| START TIME | TIME(YYYY-MM-DDTH24:MI) | Imbalance Settlement Period Start time |
| END TIME | TIME(YYYY-MM-DDTH24:MI) | Imbalance Settlement Period End time |
| SO INTERCON IMP PRICE | NUMBER(8,2) | Import Price for SO Trades |
| SO INTERCON IMP QUANTITY | NUMBER(8,3) | Import Quantity for SO Trades |
| SO INTERCON EXP PRICE | NUMBER(8,2) | Export Price for SO Trades |
| SO INTERCON EXP QUANTITY | NUMBER(8,3) | Export Quantity for SO Trades |

Table 73: Hourly SO Interconnector Trades Report

### BM-102: Unit Under Test

This report contains details of the Units Under Test for the following Trading Day.

| I-SEM Report Reference: | BM-102 |
| --- | --- |
| Data Source | System |
| Report Name: | PUB_DailyUnitUnderTest |
| File Names: | PUB_DailyUnitUnderTest.xml |
| Report Title: | Unit Under Test Report |
| Audience: | General Public |
| Resolution: | Per Physical Notification segment that is Under Test |
| Time Span: | Trading Day + 1 |
| Frequency: | Daily |
| Report Format: | XML |

| Field Name | Format | Description |
| --- | --- | --- |
| PARTICIPANT NAME | VARCHAR2(12) | The PT Identifier, which represents the name of Market Participant, as registered in the Balancing Market |
| RESOURCE NAME | VARCHAR2(32) | Generation Unit Identification |
| TRADE DATE | DATE (YYYY-MM-DD) | Trading Day |
| START TIME | TIME(YYYY-MM-DDTH24:MI) | Start time of the Under Test PN segment |
| END TIME | TIME(YYYY-MM-DDTH24:MI) | End time of the Under Test PN segment |

Table 74: Unit Under Test Report

### BM-103: Energy Market Financial Publication – Indicative

The Energy Market Financial Publication, Energy Market Information Publication, Capacity Market Financial Publication, Capacity Market Information Publication, Metered Generation Information Publication, DOG/PUG/TOLOG/TOLUG for Over/Under Generation, and Make Whole Payment will share the common set of fields given here. These reports are collectively referred to as “General Public Settlement Reports”.

| I-SEM Report Reference: | BM-103 |
| --- | --- |
| Data Source | CSB Settlement System |
| Periodicity: | Daily |
| File Names: | EF_PT_ALL_<SettlementDay>_<PublicationDay>_<Market>_<RunType>_<Timestamp> |
| Report Title: | Energy Market Financial Publication - Indicative |
| Audience: | General Public |
| Resolution: | Imbalance Settlement Period |
| Time Span: | Market Volume and Settlement by Settlement Period |
| Frequency: | Settlement Day |
| Report Format: | XML |

| Field Name | Format | Description |
| --- | --- | --- |

| REPORT_TYPE | VARCHAR(2) | Identifier for Report (EF) |
| --- | --- | --- |
| MARKET_NAME | VARCHAR2(6) | Market name (BALIMB) |
| SETTLEMENT_DATE | DATE (YYYY-MM-DD) | Calendar day of settlement |
| PUBLICATION_DATE | DATE (YYYY-MM-DD) | Calendar day for publication |
| PUBLICATION_TIMESTAMP | TIME(YYYY-MM-DDTH24:MI) | Timestamp of publication |
| RUN_TYPE | VARCHAR2(5) | Type of run (INDIC, INIT, M4, M13, AH) |

Table 75: General Public Settlement Header Field Description

| Field Name | Format | Description |
| --- | --- | --- |

| VALUE: DATETIME | TIME(YYYY-MM-DDTH24:MI) | Date and time of the ISP for this determinant and resource |
| --- | --- | --- |
| DETERMINANT:NAME | VARCHAR2(16) | Name of the determinant |
| RESOURCE: NAME | VARCHAR2(32) | Name of the resource (if applicable) |
| DETERMINANT: UNIT | VARCHAR2(8) | Units for the determinant (e.g. MW) |
| VALUE: AMOUNT | NUMBER(28,8) | Amount of the determinant in the ISP |

Table 76: General Public Settlement Detail Field Description

### BM-104: Energy Market Financial Publication – Initial

Refer to section 8.3.19 BM-103 Energy Market Financial Publication – Indicative

| I-SEM Report Reference: | BM-104 |
| --- | --- |
| Data Source | CSB Settlement System |
| Periodicity: | Daily |
| File Names: | EF_PT_ALL_<SettlementDay>_<PublicationDay>_<Market>_<RunType>_<Timestamp> |
| Report Title: | Energy Market Financial Publication - Initial |
| Audience: | General Public |
| Resolution: | Imbalance Settlement Period |
| Time Span: | Market Volume and Settlement by Settlement Period |
| Frequency: | Settlement Day |
| Report Format: | XML |

### BM-165: Energy Market Financial Publication – m+4

Refer to section 8.3.19 BM-103 Energy Market Financial Publication – Indicative

| I-SEM Report Reference: | BM-165 |
| --- | --- |
| Data Source | CSB Settlement System |
| Periodicity: | Daily |
| File Names: | EF_PT_ALL_<SettlementDay>_<PublicationDay>_<Market>_<RunType>_<Timestamp> |
| Report Title: | Energy Market Financial Publication - Indicative |
| Audience: | General Public |
| Resolution: | Imbalance Settlement Period |
| Time Span: | Market Volume and Settlement by Settlement Period |
| Frequency: | Settlement Day |
| Report Format: | XML |

### BM-166: Energy Market Financial Publication – m+13

Refer to section 8.3.19 BM-103 Energy Market Financial Publication – Indicative

| I-SEM Report Reference: | BM-166 |
| --- | --- |
| Data Source | CSB Settlement System |
| Periodicity: | Daily |
| File Names: | EF_PT_ALL_<SettlementDay>_<PublicationDay>_<Market>_<RunType>_<Timestamp> |
| Report Title: | Energy Market Financial Publication - Indicative |
| Audience: | General Public |
| Resolution: | Imbalance Settlement Period |
| Time Span: | Market Volume and Settlement by Settlement Period |
| Frequency: | Settlement Day |
| Report Format: | XML |

### BM-167: Energy Market Financial Publication – Adhoc

Refer to section 8.3.19 BM-103 Energy Market Financial Publication – Indicative

| I-SEM Report Reference: | BM-167 |
| --- | --- |
| Data Source | CSB Settlement System |
| Periodicity: | Daily |
| File Names: | EF_PT_ALL_<SettlementDay>_<PublicationDay>_<Market>_<RunType>_<Timestamp> |
| Report Title: | Energy Market Financial Publication - Indicative |
| Audience: | General Public |
| Resolution: | Imbalance Settlement Period |
| Time Span: | Market Volume and Settlement by Settlement Period |
| Frequency: | Settlement Day |
| Report Format: | XML |

### BM-105: Capacity Payments by Market

The Capacity Payments by Market Report will report aggregated market level Capacity Payment amounts per Imbalance Settlement Period.

| I-SEM Report Reference: | BM-105 |
| --- | --- |
| Data Source | CSB Settlement System |
| Periodicity: | Daily |
| File Names: | CJ_GP_ALL_<SettlementDay>_<PublicationDay> _<Market> _<RunType>_<Timestamp> |
| Report Title: | Capacity Payments by Market |
| Audience: | General Public |
| Resolution: | Imbalance Settlement Period |
| Time Span: | Daily |
| Frequency: | Settlement Day |
| Report Format: | XML |

| Field Name | Format | Description |
| --- | --- | --- |

| REPORT_TYPE | VARCHAR2(2) | Identifier for Report (CJ) |
| --- | --- | --- |
| MARKET_NAME | VARCHAR2(6) | Market name (CRM) |
| SETTLEMENT_DATE | DATE (YYYY-MM-DD) | Calendar day of settlement |
| PUBLICATION_DATE | DATE (YYYY-MM-DD) | Calendar day for publication |
| PUBLICATION_TIMESTAMP | TIME(YYYY-MM-DDTH24:MI) | Timestamp of publication |
| RUN_TYPE | VARCHAR2(5) | Type of run (INDIC, INIT) |

Table 77: Capacity Payments by Market Report Header Field Description

| Field Name | Format | Description |
| --- | --- | --- |

| CHARGE: DATETIME | TIME(YYYY-MM-DDTH24:MI) | Date and time of the ISP for this determinant and resource |
| --- | --- | --- |
| CHARGE: NAME | VARCHAR2(16) | Name of the determinant |
| JURISDICTION: UNIT | VARCHAR2(8) | Unit of Measure identifier |
| JURISDICTION: NAME | VARCHAR2(3) | ROI (Republic of Ireland) or NI (Northern Ireland) The field is left blank if Jurisdiction is not applicable for determinant. |
| CHARGE:AMOUNT | NUMBER(28,8) | Amount of the determinant in the ISP |

Table 78: Capacity Payments by Market Report Detail Field Description

### BM-106: Capacity Payments by Unit

The Capacity Payments by Unit Report will report on a unit level Capacity Payment amounts per Imbalance Settlement Period.

| I-SEM Report Reference: | BM-106 |
| --- | --- |
| Data Source | CSB Settlement System |
| Periodicity: | Daily |
| File Names: | CU_GP_ALL_<SettlementDay>_<PublicationDay>_<Market>_ <RunType> _<Timestamp> |
| Report Title: | Capacity Payments by Unit |
| Audience: | General Public |
| Resolution: | Imbalance Settlement Period |
| Time Span: | Daily |
| Frequency: | Trading Period |
| Report Format: | XML |

| Field Name | Format | Description |
| --- | --- | --- |

| REPORT_TYPE | VARCHAR2(2) | Identifier for Report (CU) |
| --- | --- | --- |
| MARKET_NAME | VARCHAR2(6) | Market name (CRM) |
| SETTLEMENT_DATE | DATE (YYYY-MM-DD) | Calendar day of settlement |
| PUBLICATION_DATE | DATE (YYYY-MM-DD) | Calendar day for publication |
| PUBLICATION_TIMESTAMP | TIME(YYYY-MM-DDTH24:MI) | Timestamp of publication |
| RUN_TYPE | VARCHAR2(5) | Type of run (INDIC, INIT) |

Table 79: Capacity Payments by Unit Report - Header Field Description

| Field Name | Format | Description |
| --- | --- | --- |

| RESOURCE NAME | VARCHAR2(32) | Name of the resource. Only populated if applicable. |
| --- | --- | --- |
| CHARGE: DATETIME | TIME(YYYY-MM-DDTH24:MI) | Date and time of the ISP for this determinant and resource |
| CHARGE NAME | VARCHAR2(16) | Name of the determinant |
| CHARGE AMOUNT | NUMBER(28,8) | Amount of the determinant in the ISP |

Table 80: Capacity Payments by Unit Report - Detail Field Description

### BM-107: Daily Generator Unit Technical Characteristic Data Transaction Report

This report contains Spot Daily Outturn Availability, Minimum Output and Minimum Stable Generation for Generator Units and Demand Side Units, for the previous Trading Day.

| I-SEM Report Reference: | BM-107 |
| --- | --- |
| Data Source | Participant |
| Periodicity: | Daily |
| Report Name: | PUB_DlyGenUnitTechCharDataTrans |
| File Names: | PUB_DlyGenUnitTechCharDataTrans.xml |
| Report Title: | Daily Generator Unit Technical Characteristic Data Transaction Report |
| Audience: | General Public |
| Resolution: | Spot values per Generator Unit |
| Time Span: | Previous Trading Day (TD-1) |
| Frequency: | Daily |
| Report Format: | XML |

| Field Name | Format | Description |
| --- | --- | --- |
| TRADE DATE | DATE (YYYY-MM-DD) | Trading Day |
| PARTICIPANT_ NAME | VARCHAR2(12) | The PT Identifier, which represents the name of Market Participant, as registered in the Balancing Market. |
| RESOURCE NAME | VARCHAR2(32) | The name of the resource (e.g. the name of the Generating Unit, Supplier Unit, Demand Side Unit, Interconnector for which data is being reported). |
| RESOURCE TYPE | VARCHAR2(4) | Indicates the type of resource for which data is being submitted - permitted values include: GU, DSU, TU, AU, EU, SU, IU, IRCU, CAU, IEU. |
| EFFECTIVE TIME | TIME(YYYY-MM-DDTH24:MI) | Effective time stamp |
| ISSUE TIME | TIME(YYYY-MM-DDTH24:MI) | Issue time stamp |
| OUTTURN AVAILABILITY | NUMBER(8,3) | Outturn Availability, spot values, by Unit ID |
| OUTTURN MINIMUM STABLE GEN | NUMBER(8,3) | Outturn Minimum Stable Generation, spot values, by Unit ID |
| OUTTURN MINIMUM OUTPUT | NUMBER(8,3) | Outturn Minimum Output, spot values, by Unit Id |
| FUEL USE FLAG | VARCHAR2(1) | Indicates the fuel use type selected. The valid values are: P and S |

Table 81: Daily Generator Unit Technical Characteristics Data Transaction Report

### BM-108: Credit Assessment Price

This report contains the Credit Assessment Price for the Undefined Exposure Period used in the calculation of Required Credit Cover for a Participant.

| I-SEM Report Reference: | BM-108 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Daily |
| Report Format: | Word/PDF |

### BM-109: Billing Period Undefined Potential Exposure Quantity

This report contains the Billing Period Undefined Potential Exposure Quantity for a Participant in respect of its Supplier Units, in the Historical Assessment to be applied the Undefined Exposure Period.

| I-SEM Report Reference: | BM-109 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Daily, post Trading Day or Settlement Day |
| Report Format: | Word/PDF |

### BM-110: Trading Payments and Charges

This report outlines the Trading Payments and Charges accrued by a Participant within a Trading Day.

| I-SEM Report Reference: | BM-110 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Daily, post Trading Day or Settlement Day |
| Report Format: | Word/PDF |

### BM-111: Capacity Quantity Scaling factor

This report contains the Capacity Quantity Scaling Factor in an Imbalance Settlement Period, which is used to adjust the Net Capacity Quantity to account for variations in Metered Demand for the calculation of a Capacity Market Unit’s Obligated Capacity Quantity.

| I-SEM Report Reference: | BM-111 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Daily, post Trading Day or Settlement Day |
| Report Format: | Word/PDF |

### BM-112: Required Capacity Quantity

This report contains the quantity for the Capacity Requirement most recently determined for a Capacity Year in accordance with the Capacity Market Code.

| I-SEM Report Reference: | BM-112 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Daily, post Trading Day or Settlement Day |
| Report Format: | Word/PDF |

### BM-113: Reserve Adjustment Capacity

This report contains the Reserve Adjustment Capacity, the quantity of any adjustments for reserve considered in the determination of the Capacity Requirement most recently determined for a Capacity Year in accordance with the Capacity Market Code.

| I-SEM Report Reference: | BM-113 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Daily, post Trading Day or Settlement Day |
| Report Format: | Word/PDF |

### BM-145: Energy Market Information Publication – Indicative

This report contains determinant values by Trading Period with respect to Balancing Market energy settlement.

| I-SEM Report Reference: | BM-145 |
| --- | --- |
| Data Source | CSB Settlement System |
| Periodicity | Daily |
| File Names: | EI_PT_ALL_<SettlementDay>_<PublicationDay>_<Market>_<RunType>_<Timestamp> |
| Report Title: | Energy Market Information Publication - Indicative |
| Audience: | General Public |
| Resolution: | Imbalance Settlement Period |
| Time Span: | Market Volume and Settlement by Settlement Period |
| Frequency: | Settlement Day |
| Report Format: | XML |

| Field Name | Format | Description |
| --- | --- | --- |
| REPORT_TYPE | VARCHAR(2) | Identifier for Report (EI) |
| MARKET_NAME | VARCHAR2(6) | Market name (BALIMB) |
| SETTLEMENT_DATE | DATE (YYYY-MM-DD) | Calendar day of settlement |
| PUBLICATION_DATE | DATE (YYYY-MM-DD) | Calendar day for publication |
| PUBLICATION_TIMESTAMP | TIME(YYYY-MM-DDTH24:MI) | Timestamp of publication |
| RUN_TYPE | VARCHAR2(5) | Type of run (INDIC, INIT, M4, M13, AH) |

Table 82: Energy Market Information Publication Header Field Description

| Field Name | Format | Description |
| --- | --- | --- |
| VALUE: DATETIME | TIME(YYYY-MM-DDTH24:MI) | Date and time of the ISP for this determinant and resource |
| VALUE: DATE | DATE (YYYY-MM-DD) | Date. Only present if applicable. |
| DETERMINANT:NAME | VARCHAR2(16) | Name of the determinant |
| RESOURCE: NAME | VARCHAR2(32) | Name of the resource (if applicable) |
| DETERMINANT: UNIT | VARCHAR2(8) | Units for the determinant (e.g. MW) |
| VALUE: AMOUNT | NUMBER(28,8) | Amount of the determinant in the ISP |
| VALUE: ORDER | STRING(25) | Number of the order (may be negative). Only populated for determinants which are order-specific. |
| VALUE: BAND | NUMBER(2) | Number of the Price Quantity Pair for an Imbalance Settlement Period. Only present if applicable. |

Table 83: Energy Market Information Publication Detail Field Description

### BM-146: Energy Market Information Publication – Initial

Refer to section  8.3.33 BM-145: Energy Market Information Publication – Indicative

| I-SEM Report Reference: | BM-146 |
| --- | --- |
| Data Source | CSB Settlement System |
| Periodicity: | Daily |
| File Names: | EI_PT_ALL_<SettlementDay>_<PublicationDay>_<Market>_<RunType>_<Timestamp> |
| Report Title: | Energy Market Information Publication - Initial |
| Audience: | General Public |
| Resolution: | Imbalance Settlement Period |
| Time Span: | Market Volume and Settlement by Settlement Period |
| Frequency: | Settlement Day |
| Report Format: | XML |

### BM-168: Energy Market Information Publication – M+4

Refer to section  8.3.33  BM-145: Energy Market Information Publication – Indicative

| I-SEM Report Reference: | BM-168 |
| --- | --- |
| Data Source | CSB Settlement System |
| Periodicity | Daily |
| File Names: | EI_PT_ALL_<SettlementDay>_<PublicationDay>_<Market>_<RunType>_<Timestamp> |
| Report Title: | Energy Market Information Publication - Indicative |
| Audience: | General Public |
| Resolution: | Imbalance Settlement Period |
| Time Span: | Market Volume and Settlement by Settlement Period |
| Frequency: | Settlement Day |
| Report Format: | XML |

### BM-169: Energy Market Information Publication – M+13

Refer to section 8.3.33 BM-145: Energy Market Information Publication – Indicative

| I-SEM Report Reference: | BM-169 |
| --- | --- |
| Data Source | CSB Settlement System |
| Periodicity | Daily |
| File Names: | EI_PT_ALL_<SettlementDay>_<PublicationDay>_<Market>_<RunType>_<Timestamp> |
| Report Title: | Energy Market Information Publication - Indicative |
| Audience: | General Public |
| Resolution: | Imbalance Settlement Period |
| Time Span: | Market Volume and Settlement by Settlement Period |
| Frequency: | Settlement Day |
| Report Format: | XML |

### BM-170: Energy Market Information Publication – Adhoc

Refer to section 8.3.33 BM-145: Energy Market Information Publication – Indicative

| I-SEM Report Reference: | BM-170 |
| --- | --- |
| Data Source | CSB Settlement System |
| Periodicity | Daily |
| File Names: | EI_PT_ALL_<SettlementDay>_<PublicationDay>_<Market>_<RunType>_<Timestamp> |
| Report Title: | Energy Market Information Publication - Indicative |
| Audience: | General Public |
| Resolution: | Imbalance Settlement Period |
| Time Span: | Market Volume and Settlement by Settlement Period |
| Frequency: | Settlement Day |
| Report Format: | XML |

### BM-147: Capacity Market Information Publication – Indicative

This report contains determinant values by Trading Period with respect to Capacity Market settlement.

| I-SEM Report Reference: | BM-147 |
| --- | --- |
| Data Source | CSB Settlement System |
| Periodicity: | Daily |
| File Names: | CI_PT_ALL_<SettlementDay>_<PublicationDay>_<Market>_<RunType>_<Timestamp> |
| Report Title: | Capacity Market Information Publication - Indicative |
| Audience: | General Public |
| Resolution: | Imbalance Settlement Period |
| Time Span: | Market Volume and Settlement by Settlement Period |
| Frequency: | Settlement Day |
| Report Format: | XML |

| Field Name | Format | Description |
| --- | --- | --- |

| REPORT_TYPE | VARCHAR(2) | Identifier for Report (CI) |
| --- | --- | --- |
| MARKET_NAME | VARCHAR2(6) | Market name (CRM) |
| SETTLEMENT_DATE | DATE (YYYY-MM-DD) | Calendar day of settlement |
| PUBLICATION_DATE | DATE (YYYY-MM-DD) | Calendar day for publication |
| PUBLICATION_TIMESTAMP | TIME(YYYY-MM-DDTH24:MI) | Timestamp of publication |
| RUN_TYPE | VARCHAR2(5) | Type of run (INDIC, INIT, M4, M13, AH) |

Table 84: Capacity Market Information Publication Header Field Description

| Field Name | Format | Description |
| --- | --- | --- |

| VALUE: DATE | DATE (YYYY-MM-DD) | Date for this determinant and resource |
| --- | --- | --- |
| DETERMINANT:NAME | VARCHAR2(16) | Name of the determinant |
| RESOURCE: NAME | VARCHAR2(32) | Name of the resource (if applicable) |
| DETERMINANT: UNIT | VARCHAR2(8) | Units for the determinant (e.g. MW) |
| VALUE: AMOUNT | NUMBER(28,8) | Amount of the determinant in the ISP |
| VALUE: RO_TRADE | NUMBER(20) | RO trade. Only present if applicable. |

Table 85: Capacity Market Information Publication Detail Field Description

### BM-148: Capacity Market Information Publication – Initial

Refer to section  8.3.38 BM-147: Capacity Market Information Publication - Indicative

| I-SEM Report Reference: | BM-148 |
| --- | --- |
| Data Source | CSB Settlement System |
| Periodicity: | Daily |
| File Names: | CI_PT_ALL_<SettlementDay>_<PublicationDay>_<Market>_<RunType>_<Timestamp> |
| Report Title: | Capacity Market Information Publication - Initial |
| Audience: | General Public |
| Resolution: | Imbalance Settlement Period |
| Time Span: | Market Volume and Settlement by Settlement Period |
| Frequency: | Settlement Day |
| Report Format: | XML |

### BM-171: Capacity Market Information Publication – M+4

Refer to section  8.3.38 BM-147: Capacity Market Information Publication - Indicative

| I-SEM Report Reference: | BM-171 |
| --- | --- |
| Data Source | CSB Settlement System |
| Periodicity: | Daily |
| File Names: | CI_PT_ALL_<SettlementDay>_<PublicationDay>_<Market>_<RunType>_<Timestamp> |
| Report Title: | Capacity Market Information Publication - Initial |
| Audience: | General Public |
| Resolution: | Imbalance Settlement Period |
| Time Span: | Market Volume and Settlement by Settlement Period |
| Frequency: | Settlement Day |
| Report Format: | XML |

### BM-172: Capacity Market Information Publication – M+13

Refer to section  8.3.38 BM-147: Capacity Market Information Publication - Indicative

| I-SEM Report Reference: | BM-172 |
| --- | --- |
| Data Source | CSB Settlement System |
| Periodicity: | Daily |
| File Names: | CI_PT_ALL_<SettlementDay>_<PublicationDay>_<Market>_<RunType>_<Timestamp> |
| Report Title: | Capacity Market Information Publication - Initial |
| Audience: | General Public |
| Resolution: | Imbalance Settlement Period |
| Time Span: | Market Volume and Settlement by Settlement Period |
| Frequency: | Settlement Day |
| Report Format: | XML |

### BM-173: Capacity Market Information Publication – Adhoc

Refer to section  8.3.38 BM-147: Capacity Market Information Publication - Indicative

| I-SEM Report Reference: | BM-173 |
| --- | --- |
| Data Source | CSB Settlement System |
| Periodicity: | Daily |
| File Names: | CI_PT_ALL_<SettlementDay>_<PublicationDay>_<Market>_<RunType>_<Timestamp> |
| Report Title: | Capacity Market Information Publication - Initial |
| Audience: | General Public |
| Resolution: | Imbalance Settlement Period |
| Time Span: | Market Volume and Settlement by Settlement Period |
| Frequency: | Settlement Day |
| Report Format: | XML |

### BM-149: Capacity Market Financial Publication – Indicative

This report contains determinant values by Trading Period with respect to Capacity Market settlement.

| I-SEM Report Reference: | BM-149 |
| --- | --- |
| Data Source | CSB Settlement System |
| Periodicity: | Daily |
| File Names: | CF_PT_ALL_<SettlementDay>_<PublicationDay>_<Market>_<RunType>_<Timestamp> |
| Report Title: | Capacity Market Financial Publication - Indicative |
| Audience: | General Public |
| Resolution: | Imbalance Settlement Period |
| Time Span: | Market Volume and Settlement by Settlement Period |
| Frequency: | Settlement Day |
| Report Format: | XML |

| Field Name | Format | Description |
| --- | --- | --- |
| REPORT_TYPE | VARCHAR(2) | Identifier for Report (CF) |
| MARKET_NAME | VARCHAR2(6) | Market name (CRM) |
| SETTLEMENT_DATE | DATE (YYYY-MM-DD) | Calendar day of settlement |
| PUBLICATION_DATE | DATE (YYYY-MM-DD) | Calendar day for publication |
| PUBLICATION_TIMESTAMP | TIME(YYYY-MM-DDTH24:MI) | Timestamp of publication |
| RUN_TYPE | VARCHAR2(5) | Type of run (INDIC, INIT, M4, M13, AH) |

Table 86: Capacity Market Financial Publication Header Field Description

| Field Name | Format | Description |
| --- | --- | --- |

| VALUE: DATETIME | TIME(YYYY-MM-DDTH24:MI) | Date and time of the ISP for this determinant and resource |
| --- | --- | --- |
| DETERMINANT:NAME | VARCHAR2(16) | Name of the determinant |
| RESOURCE: NAME | VARCHAR2(32) | Name of the resource (if applicable) |
| DETERMINANT: UNIT | VARCHAR2(8) | Units for the determinant (e.g. MW) |
| VALUE: AMOUNT | NUMBER(28,8) | Amount of the determinant in the ISP |

Table 87: Capacity Market Financial Publication Detail Field Description

### BM-150: Capacity Market Financial Publication – Initial

Refer to section .8.3.43 BM-149: Capacity Market Financial Publication – Indicative

| I-SEM Report Reference: | BM-150 |
| --- | --- |
| Data Source | CSB Settlement System |
| Periodicity: | Daily |
| File Names: | CF_PT_ALL_<SettlementDay>_<PublicationDay>_<Market>_<RunType>_<Timestamp> |
| Report Title: | Capacity Market Financial Publication - Initial |
| Audience: | General Public |
| Resolution: | Imbalance Settlement Period |
| Time Span: | Market Volume and Settlement by Settlement Period |
| Frequency: | Settlement Day |
| Report Format: | XML |

### BM-174: Capacity Market Financial Publication – M+4

Refer to section 8.3.43 BM-149: Capacity Market Financial Publication – Indicative

| I-SEM Report Reference: | BM-174 |
| --- | --- |
| Data Source | CSB Settlement System |
| Periodicity: | Daily |
| File Names: | CF_PT_ALL_<SettlementDay>_<PublicationDay>_<Market>_<RunType>_<Timestamp> |
| Report Title: | Capacity Market Financial Publication - Indicative |
| Audience: | General Public |
| Resolution: | Imbalance Settlement Period |
| Time Span: | Market Volume and Settlement by Settlement Period |
| Frequency: | Settlement Day |
| Report Format: | XML |

### BM-175: Capacity Market Financial Publication – M+13

Refer to section 8.3.43 BM-149: Capacity Market Financial Publication – Indicative

| I-SEM Report Reference: | BM-175 |
| --- | --- |
| Data Source | CSB Settlement System |
| Periodicity: | Daily |
| File Names: | CF_PT_ALL_<SettlementDay>_<PublicationDay>_<Market>_<RunType>_<Timestamp> |
| Report Title: | Capacity Market Financial Publication - Indicative |
| Audience: | General Public |
| Resolution: | Imbalance Settlement Period |
| Time Span: | Market Volume and Settlement by Settlement Period |
| Frequency: | Settlement Day |
| Report Format: | XML |

### BM-176: Capacity Market Financial Publication – Adhoc

Refer to section .8.3.43 BM-149: Capacity Market Financial Publication – Indicative

| I-SEM Report Reference: | BM-176 |
| --- | --- |
| Data Source | CSB Settlement System |
| Periodicity: | Daily |
| File Names: | CF_PT_ALL_<SettlementDay>_<PublicationDay>_<Market>_<RunType>_<Timestamp> |
| Report Title: | Capacity Market Financial Publication - Indicative |
| Audience: | General Public |
| Resolution: | Imbalance Settlement Period |
| Time Span: | Market Volume and Settlement by Settlement Period |
| Frequency: | Settlement Day |
| Report Format: | XML |

### BM-151: Metered Generation Information Publication - Indicative

This report contains determinant values by Trading Period with respect to settlement-related metering.

| I-SEM Report Reference: | BM-151 |
| --- | --- |
| Data Source | CSB Settlement System |
| Periodicity: | Daily |
| File Names: | MI_PT_ALL_<SettlementDay>_<PublicationDay>_<Market>_<RunType>_<Timestamp> |
| Report Title: | Metered Generation Information Publication - Indicative |
| Audience: | General Public |
| Resolution: | Imbalance Settlement Period |
| Time Span: | Market Volume and Settlement by Settlement Period |
| Frequency: | Settlement Day |
| Report Format: | XML |

| Field Name | Format | Description |
| --- | --- | --- |
| REPORT_TYPE | VARCHAR(2) | Identifier for Report (MI) |
| MARKET_NAME | VARCHAR2(6) | Market name (BALIMB) |
| SETTLEMENT_DATE | DATE (YYYY-MM-DD) | Calendar day of settlement |
| PUBLICATION_DATE | DATE (YYYY-MM-DD) | Calendar day for publication |
| PUBLICATION_TIMESTAMP | TIME(YYYY-MM-DDTH24:MI) | Timestamp of publication |
| RUN_TYPE | VARCHAR2(5) | Type of run (INDIC, INIT, M4, M13, AH) |

Table 88: Metered Generation Information Publication -Header Field Description

| Field Name | Format | Description |
| --- | --- | --- |
| VALUE: DATETIME | TIME(YYYY-MM-DDTH24:MI) | Date and time of the ISP for this determinant and resource |
| DETERMINANT:NAME | VARCHAR2(16) | Name of the determinant |
| RESOURCE: NAME | VARCHAR2(32) | Name of the resource (if applicable) |
| DETERMINANT: UNIT | VARCHAR2(8) | Units for the determinant (e.g. MW) |
| VALUE: QUANTITY | NUMBER(28,8) | Quantity of the determinant in the ISP |

Table 89: Metered Generation Information Publication -Detail Field Description

### BM-152: Metered Generation Information Publication – Initial

Refer to section 8.3.48 BM-151: Metered Generation Information Publication - Indicative

| I-SEM Report Reference: | BM-152 |
| --- | --- |
| Data Source | CSB Settlement System |
| Periodicity: | Daily |
| File Names: | MI_PT_ALL_<SettlementDay>_<PublicationDay>_<Market>_<RunType>_<Timestamp> |
| Report Title: | Metered Generation Information Publication - Initial |
| Audience: | General Public |
| Resolution: | Imbalance Settlement Period |
| Time Span: | Market Volume and Settlement by Settlement Period |
| Frequency: | Settlement Day |
| Report Format: | XML |

### BM-177: Metered Generation Information Publication – M+4

Refer to section 8.3.48 BM-151: Metered Generation Information Publication - Indicative

| I-SEM Report Reference: | BM-177 |
| --- | --- |
| Data Source | CSB Settlement System |
| Periodicity: | Daily |
| File Names: | MI_PT_ALL_<SettlementDay>_<PublicationDay>_<Market>_<RunType>_<Timestamp> |
| Report Title: | Metered Generation Information Publication - Initial |
| Audience: | General Public |
| Resolution: | Imbalance Settlement Period |
| Time Span: | Market Volume and Settlement by Settlement Period |
| Frequency: | Settlement Day |
| Report Format: | XML |

### BM-178: Metered Generation Information Publication – M+13

Refer to section 8.3.48 BM-151: Metered Generation Information Publication - Indicative

| I-SEM Report Reference: | BM-178 |
| --- | --- |
| Data Source | CSB Settlement System |
| Periodicity: | Daily |
| File Names: | MI_PT_ALL_<SettlementDay>_<PublicationDay>_<Market>_<RunType>_<Timestamp> |
| Report Title: | Metered Generation Information Publication - Initial |
| Audience: | General Public |
| Resolution: | Imbalance Settlement Period |
| Time Span: | Market Volume and Settlement by Settlement Period |
| Frequency: | Settlement Day |
| Report Format: | XML |

### BM-179: Metered Generation Information Publication – Adhoc

Refer to section 8.3.48 BM-151: Metered Generation Information Publication - Indicative

| I-SEM Report Reference: | BM-179 |
| --- | --- |
| Data Source | CSB Settlement System |
| Periodicity: | Daily |
| File Names: | MI_PT_ALL_<SettlementDay>_<PublicationDay>_<Market>_<RunType>_<Timestamp> |
| Report Title: | Metered Generation Information Publication - Initial |
| Audience: | General Public |
| Resolution: | Imbalance Settlement Period |
| Time Span: | Market Volume and Settlement by Settlement Period |
| Frequency: | Settlement Day |
| Report Format: | XML |

### BM-153: Make Whole Payments

This report contains determinant values by Trading Period with respect to Make Whole Payment elements of settlement.

| I-SEM Report Reference: | BM-153 |
| --- | --- |
| Data Source | CSB Settlement System |
| Periodicity: | Weekly |
| File Names: | MW_GP_ALL_<SettlementDay>_<PublicationDay>_ <Market>_<RunType> _<Timestamp> |
| Report Title: | Make Whole Payment |
| Audience: | General Public |
| Resolution: | Imbalance Settlement Period |
| Time Span: | Daily |
| Frequency: | Settlement Day |
| Report Format: | XML |

| Field Name | Format | Description |
| --- | --- | --- |
| REPORT_TYPE | VARCHAR2(2) | Identifier for Report (MW) |
| MARKET_NAME | VARCHAR2(6 | Market name (CRM) |
| SETTLEMENT_DATE | DATE (YYYY-MM-DD) | Calendar day of settlement |
| PUBLICATION_DATE | DATE (YYYY-MM-DD) | Calendar day for publication |
| PUBLICATION_TIMESTAMP | TIME(YYYY-MM-DDTH24:MI) | Timestamp of publication |
| RUN_TYPE | VARCHAR2(5) | Type of run (INDIC, INIT) |

Table 90: Make Whole Payments Report Header Field Description

| Field Name | Format | Description |
| --- | --- | --- |
| RESOURCE NAME | VARCHAR2(32) | Name of the resource. Only populated if applicable. |
| DETERMINANT: NAME | VARCHAR2(16) | Name of the determinant |
| JURISDICTION NAME | VARCHAR2(3) | ROI (Republic of Ireland) or NI (Northern Ireland) The field is left blank if Jurisdiction is not applicable for determinant. |
| DETERMINANT: UNIT | VARCHAR2(8) | Units for the determinant (e.g. MW) |
| VALUE: AMOUNT | NUMBER(28,8) | Amount of the determinant in the ISP |
| VALUE: DATETIME | TIME(YYYY-MM-DDTH24:MI) | Date and time of the ISP for this determinant and resource |

Table 91: Make Whole Payments Report Detail Field Description

### BM-154: Cross Border Monthly Capacity Data

The content of the report is summarised as follows:

- Run Type (e.g. INIT, M4, M13)

- Total Capacity Sales (MPCSTOT)

- Total Capacity Purchases (MPCPTOT)

- For each Place of Establishment (e.g. NI, ROI, France):

  - Capacity Purchases for the Place of Establishment (MPCP)

  - Capacity Sales for the Place of Establishment (MPCS)

  - VAT Capacity sales proportion for the Place of Establishment (VATCS)

  - VAT Capacity purchases proportion for the Place of Establishment (VATCP)

| I-SEM Report Reference: | BM-154 |
| --- | --- |
| Data Source | CSB Settlement System |
| Periodicity: | Daily |
| File Names: | XM_GP_ALL_<PublicationDay>_ <PublicationDay>_ <Market>_REPT _<Timestamp> |
| Report Title: | Cross Border Capacity Data |
| Audience: | General Public |
| Resolution: | Billing Period |
| Time Span: | Month |
| Frequency: | Monthly |
| Report Format: | XML |

| Field Name | Format | Description |
| --- | --- | --- |
| REPORT_TYPE | VARCHAR2(2) | Identifier for report (XM) |
| MARKET_NAME | VARCHAR2(6) | Market name (CRM) |
| PUBLICATION_DATE | DATE (YYYY-MM-DD) | Calendar day for publication |
| PUBLICATION_TIMESTAMP | TIME(YYYY-MM-DDTH24:MI) | Timestamp of publication |

Table 92: Cross Border Monthly Capacity Report Header Field Description

| Field Name | Format | Description |
| --- | --- | --- |
| BILL_PERIOD: RUN_TYPE | VARCHAR2(5) | Type of run (INDIC, INIT, M4, M13, AH) |
| BILL_PERIOD:BP_START_DATE | DATE (YYYY-MM-DD) | Billing Period Start Date |
| BILL_PERIOD: BP_END_DATE | DATE (YYYY-MM-DD)MI) | Billing Period End Date |
| DETERMINANT: MPCPTOT | number(28,8) | Value of total Capacity Purchases |
| DETERMINANT: MPCSTOT | number(28,8) | Value of total Capacity Sales |

Table 93: Cross Border Monthly Capacity Report Summary Field Description

| Field Name | Format | Description |
| --- | --- | --- |
| BILL_PERIOD: RUN_TYPE | VARCHAR2(5) | Type of run (INDIC, INIT, M4, M13, AH) |
| BILL_PERIOD:BP_START_DATE | DATE (YYYY-MM-DD) | Billing Period Start Date |
| BILL_PERIOD: BP_END_DATE | DATE (YYYY-MM-DD)MI) | Billing Period End Date |
| PLACE_OF_ESTABLISHMENT NAME | VARCHAR2(16) | Type of run (INDIC, INIT, M4, M13, AH) |
| DETERMINANT: NAME | VARCHAR2(16) | Name of Determinant (MPCS, MPCP, VATCS, VATCP) |
| DETERMINANT: AMOUNT | number(28,8) | Value of Determinant |

Table 94: Cross Border Monthly Capacity Report Field Description

### BM-155: Cross Border Weekly Energy Data

The content of the report is summarised as follows:

- Run Type (e.g. INIT, M4, M13)

- Total Energy Sales (MPESTOT)

- Total Energy Purchases (MPEPTOT)

- For each Place of Establishment (e.g. NI, ROI, France):

  - Energy Purchases for the Place of Establishment (MPEP)

  - Energy Sales for the Place of Establishment (MPES)

  - VAT Energy sales proportion for the Place of Establishment (VATES)

  - VAT Energy purchases proportion for the Place of Establishment (VATEP)

| I-SEM Report Reference: | BM-155 |
| --- | --- |
| Data Source | CSB Settlement System |
| Periodicity: | Daily |
| File Names: | XW_GP_ALL_<PublicationDay>_<PublicationDay> _<Market>_REPT _<Timestamp> |
| Report Title: | Cross Border Energy Data |
| Audience: | General Public |
| Resolution: | Billing Period |
| Time Span: | Billing Period |
| Frequency: | Billing Period |
| Report Format: | XML |

| Field Name | Format | Description |
| --- | --- | --- |
| REPORT_TYPE | VARCHAR2(2) | Identifier for report (XW) |
| MARKET_NAME | VARCHAR2(6) | Market name (BALIMB) |
| PUBLICATION_DATE | DATE (YYYY-MM-DD) | Calendar day for publication |
| PUBLICATION_TIMESTAMP | TIME(YYYY-MM-DDTH24:MI) | Timestamp of publication |

Table 95: Cross Border Energy Data Report Header Field Description

| Field Name | Format | Description |
| --- | --- | --- |
| BILL_PERIOD: RUN_TYPE | VARCHAR2(5) | Type of run (INDIC, INIT, M4, M13, AH) |
| BILL_PERIOD:BP_START_DATE | DATE (YYYY-MM-DD) | Billing Period Start Date |
| BILL_PERIOD: BP_END_DATE | DATE (YYYY-MM-DD)MI) | Billing Period End Date |
| DETERMINANT: MPEPTOT | number(28,8) | Value of total Energy Purchases |
| DETERMINANT: MPESTOT | number(28,8) | Value of total Energy Sales |

Table 96: Cross Border Energy Data Report Summary Field Description

| Field Name | Format | Description |
| --- | --- | --- |
| BILL_PERIOD: RUN_TYPE | VARCHAR2(5) | Type of run (INDIC, INIT, M4, M13, AH)) |
| BILL_PERIOD:BP_START_DATE | DATE (YYYY-MM-DD) | Billing Period Start Date |
| BILL_PERIOD: BP_END_DATE | DATE (YYYY-MM-DD)MI) | Billing Period End Date |
| PLACE_OF_ESTABLISHMENT NAME | VARCHAR2(16) | Type of run (INDIC, INIT, M4, M13, AH) |
| DETERMINANT: NAME | VARCHAR2(16) | Name of Determinant (MPES, MPEP, VATES, VATEP) |
| DETERMINANT: AMOUNT | number(28,8) | Value of Determinant |

Table 97: Cross Border Energy Data Report Field Description

### BM-115: Proposed Market Operator Isolated Market System Testing Schedule

This report contains the proposed Market Operator Isolated Market System Testing Schedule.

| I-SEM Report Reference: | BM-115 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Periodically as required |
| Report Format: | Word/PDF |

### BM-116: Calculations and Methodology used by the Market Operator during administered Settlement

This report contains the calculations and methodology used by the Market Operator during Administered Settlement.

| I-SEM Report Reference: | BM-116 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Periodically as required |
| Report Format: | Word/PDF |

### BM-117: Market Operator Performance Report

This Report contains the performance report prepared by the Market Operator and provided to the Regulatory Authorities.

| I-SEM Report Reference: | BM-117 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Monthly |
| Report Format: | Word/PDF |

### BM-130: Quarterly Renewable Dispatch Down (Constraint & Curtailment)

This Report contains the Quarterly Renewable Dispatch Down (Constraint & Curtailment) data.

| I-SEM Report Reference: | BM-130 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Quarterly |
| Report Format: | Word/PDF |

### BM-131: Annual Renewable Dispatch Down (Constraint & Curtailment)

This Report contains the Annual Renewable Dispatch Down (Constraint & Curtailment) data.

| I-SEM Report Reference: | BM-131 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Yearly |
| Report Format: | Word/PDF |

### BM-132: Quarterly Constraint Cost Outturn

This Report contains the Quarterly Constraint Cost Outrun data.

| I-SEM Report Reference: | BM-132 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Quarterly |
| Report Format: | Word/PDF |

### BM-137: Monthly Constraint Update

This Report contains the Monthly Constraint data.

| I-SEM Report Reference: | BM-137 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Monthly |
| Report Format: | Word/PDF |

### BM-138: Weekly Constraint Update

This Report contains the Weekly Constraint Update data.

| I-SEM Report Reference: | BM-138 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Weekly |
| Report Format: | Word/PDF |

### BM-139: Ad Hoc Constraint Update

This Report contains the Ad Hoc Constraint Update data.

| I-SEM Report Reference: | BM-139 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Ad Hoc |
| Report Format: | Word/PDF |

## Balancing Market Methodologies and Processes

### BM-129: Ad Hoc Constraint Update

This report contains the interim methodology used to calculate the cross zonal capacity.

| I-SEM Report Reference: | BM-129 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Ad Hoc |
| Report Format: | Word/PDF |

### BM-136: Balancing market Principles Statement

This is the Balancing Market Principles Statement.

| I-SEM Report Reference: | BM-136 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Ad Hoc |
| Report Format: | Word/PDF |

### BM-146: Determination of Scheduling and Dispatch Policy Parameters

This is the Determination of Scheduling and Dispatch Policy Parameters report.

| I-SEM Report Reference: | BM-146 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Ad Hoc |
| Report Format: | Word/PDF |

### BM-147: System Operator and Non-Marginal Flagging

This is the System Operator and Non-Marginal Flagging report.

| I-SEM Report Reference: | BM-147 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Ad Hoc |
| Report Format: | Word/PDF |

### BM-140: Demand Forecasting

This report describes the operational process for Demand Forecasting.

| I-SEM Report Reference: | BM-140 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Ad Hoc |
| Report Format: | Word/PDF |

### BM-141: Scheduling

This report describes the operational process for Scheduling.

| I-SEM Report Reference: | BM-141 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Ad Hoc |
| Report Format: | Word/PDF |

### BM-142: Dispatch

This report describes the operational process for Dispatch.

| I-SEM Report Reference: | BM-142 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Ad Hoc |
| Report Format: | Word/PDF |

### BM-143: Cross-Zonal Actons

This report describes the operational process for Cross-Zonal Actions.

| I-SEM Report Reference: | BM-143 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Ad Hoc |
| Report Format: | Word/PDF |

### BM-144: Wind Farm Controllability Categorisation

This report describes the operational process for Wind Farm Controllability Categorisation.

| I-SEM Report Reference: | BM-144 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Ad Hoc |
| Report Format: | Word/PDF |

## Balancing Market Operator Performance

### BM-133: Annual All-Island Transmission System Performance Report

This report contains information relating to the annual all-island transmission system performance

| I-SEM Report Reference: | BM-133 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Yearly |
| Report Format: | Word/PDF |

### BM-134: Scheduling and Dispatch Policy Parameter Performance Report

This report contains information relating to the performance of Scheduling and Dispatch policy parameters.

| I-SEM Report Reference: | BM-134 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Yearly |
| Report Format: | Word/PDF |

### BM-135: Annual Scheduling and Dispatch Process Audit Report

This report contains information relating to the annual Scheduling and Dispatch Audit.

| I-SEM Report Reference: | BM-135 |
| --- | --- |
| Audience: | General Public |
| Frequency: | Yearly |
| Report Format: | Word/PDF |

# Appendix C: SEMO Website API Specification

XML 1.0 Specification. World Wide Web Consortium. Retrieved 22 August 2010.

Adobe Systems Incorporated, PDF Reference, 6th Editions, version 1.23, Nov 2006, p33

File Type – can be either Operational day or Settlement day depending on how the report is generated