TSOs’ note on preferred start time of WD1 Trading Day

The concern which the TSOs have regarding the start time of the second part of the WD1 trading day is how it relates to the time of the peak demand. As shown in the demand curves below, in summer the peak typically happens at lunchtime but on occasion a peak of similar or greater magnitude happens in the evening. In the late spring and early autumn, the demand is consistently high from lunchtime till the late evening when the lighting load kicks in.

In autumn, winter and spring as the lighting and heating demands coincide, the peak will happen in the evening time.  On top of managing the magnitude of the peak demand of the day, the rise in demand up to the peak happens at a very fast rate. A huge focus of the planning of the generation engineer in the control centre is in ensuring that there is adequate generation committed and that it reaches the required output to meet the demand at the peak. Margins between total available generation and system demand are generally at their tightest around this time and although reserve is carried to cover the loss of the largest generator on the system, losing a generator coming up to the peak, when the system is stressed, would typically be the worst possible time for an event like this to happen.   If a new WD schedule with corresponding new inter-connector schedules were to take effect before the peak, there could be inadequate time to manage the change, in what is already one of the most difficult periods of the day. To manage this risk, the TSO’s would have to apply greater restrictions around the inter-connector ramp rates or limit the size of changes between trading periods.

It is understood that from a market point of view, there may be a wish to open up as much of the day as possible to intra-day trading. For the reasons detailed above, the TSOs would like to push the start time to after the peak but realise that at particular times of the year that would mean delaying it till as late as 22:00, which would limit the usefulness of intra-day trading. With later peak times, the TSOs will have more time to plan for and call on additional generation if required to deal with changed forthcoming inter-connector schedules. Balancing both of these concerns, the preference of the TSOs is for the start time of the second part of the WD1 trading day (i.e. start time from which WD1 prices apply) to be 18:00.